- Configure the PC address to 1.1.1.2  24

- Open TFTP software, such as 3CD, set the file storage location

- display version to check the version of the device.

[MSR810]dis version

H3C Comware Software, Version 7.1.064, Release 0707P24

Copyright (c) 2004-2020 New H3C Technologies Co., Ltd. All rights reserved.

H3C MSR810 uptime is 0 weeks, 0 days, 0 hours, 4 minutes

- Restart the device, and long press Ctrl+B when restarting the device to enter the bootrom.

<MSR810>reboot

Start to check configuration with next startup configuration file, please wait.........DONE!

This command will reboot the device. Continue? [Y/N]:Y

Now rebooting, please wait...

%Jan  1 00:09:47:099 2011 MSR810 DEV/5/SYSTEM_REBOOT: System is rebooting now.

@

System is starting...

Press Ctrl+D to access BASIC-BOOTWARE MENU

Booting Normal Extended BootWare

****************************************************************************

*                                                                          *

*                   H3C MSR810 BootWare, Version 1.61                      *

*                                                                          *

****************************************************************************

Copyright (c) 2004-2019 New H3C Technologies Co., Ltd.

Compiled Date       : May  5 2019

CPU ID              : 0xa

CPU L1 Cache        : 32KB

CPU L2 Cache        : 256KB

Memory Type         : DDR3 SDRAM

Memory Size         : 1024MB

Flash Size          : 256MB

PCB Version         : 2.0

BootWare Validating...

Press Ctrl+B to access EXTENDED-BOOTWARE MENU...

Password recovery capability is enabled.

Note: The current operating device is flash

Enter < Storage Device Operation > to select device.

==========================<EXTENDED-BOOTWARE MENU>==========================

|<1> Boot System                                                           |

|<2> Enter Serial SubMenu                                                  |

|<3> Enter Ethernet SubMenu                                                |

|<4> File Control                                                          |

|<5> Restore to Factory Default Configuration                              |

|<6> Skip Current System Configuration                                     |

|<7> BootWare Operation Menu                                               |

|<8> Skip Authentication for Console Login                                 |

|<9> Storage Device Operation                                              |

|<0> Reboot                                                                |

============================================================================

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+C: Display Copyright

Ctrl+F: Format File System

Enter your choice(0-9): 3

==============<Enter Ethernet SubMenu>==========================

|Note:the operating device is flash                                        |

|<1> Download Image Program To SDRAM And Run                               |

|<2> Update Main Image File                                                |

|<3> Update Backup Image File                                              |

|<4> Download Files(*.*)                                                   |

|<5> Modify Ethernet Parameter                                             |

|<0> Exit To Main Menu                                                     |

|<Ensure The Parameter Be Modified Before Downloading!>                    |

============================================================================

Enter your choice(0-5): 5

==========================<ETHERNET PARAMETER SET>==========================

|Note:       '.' = Clear field.                                            |

|            '-' = Go to previous field.                                   |

|         Ctrl+D = Quit.                                                   |

============================================================================

Protocol (FTP or TFTP) :tftp

Load File Name         :MSR810-CMW710-R0809P25.ipe                      :

Target File Name       :MSR810-CMW710-R0809P25.ipe                       :

Server IP Address      :1.1.1.2

Local IP Address       :1.1.1.1

Subnet Mask           :255.255.255.0

Gateway IP Address    :1.1.1.2

=================<Enter Ethernet SubMenu>==========================

|Note:the operating device is flash                                        |

|<1> Download Image Program To SDRAM And Run                               |

|<2> Update Main Image File                                                |

|<3> Update Backup Image File                                              |

|<4> Download Files(*.*)                                                   |

|<5> Modify Ethernet Parameter                                             |

|<0> Exit To Main Menu                                                     |

|<Ensure The Parameter Be Modified Before Downloading!>                    |

============================================================================

Enter your choice(0-5): 2

Loading.............................................................................................................................

..............................................................................................................................................

..............................................Done.

79714304 bytes downloaded!

Image file msr810-cmw710-boot-r0809p25.bin is self-decompressing...

Saving file flash:/msr810-cmw710-boot-r0809p25.bin ...................Done.

Image file msr810-cmw710-system-r0809p25.bin is self-decompressing...

Saving file flash:/msr810-cmw710-system-r0809p25.bin

...................................................................................................Done.

==================<Enter Ethernet SubMenu>==========================

|Note:the operating device is flash                                        |

|<1> Download Image Program To SDRAM And Run                               |

|<2> Update Main Image File                                                |

|<3> Update Backup Image File                                              |

|<4> Download Files(*.*)                                                   |

|<5> Modify Ethernet Parameter                                             |

|<0> Exit To Main Menu                                                     |

|<Ensur Be Modified Before Downloading!>                    |

============================================================================

Enter your choice(0-5): 0

==========================<EXTENDED-BOOTWARE MENU>==========================

|<1> Boot System                                                           |

|<2> Enter Serial SubMenu                                                  |

|<3> Enter Ethernet SubMenu                                                |

|<4> File Control                                                          |

|<5> Restore to Factory Default Configuration                              |

|<6> Skip Current System Configuration                                     |

|<7> BootWare Operation Menu                                               |

|<8> Skip Authentication for Console Login                                 |

|<9> Storage Device Operation                                              |

|<0> Reboot                                                                |

============================================================================

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+C: Display Copyright

Ctrl+F: Format File System

Enter your choice(0-9): 1

Loading the main image files...

Loading file flash:/msr810-cmw710-system-r0809p25.bin.......................

.............................................Done.

Loading file flash:/msr810-cmw710-security-r0809p25.bin....Done.

Loading file flash:/msr810-cmw710-voice-r0809p25.bin...Done.

Loading file flash:/msr810-cmw710-data-r0809p25.bin......Done.

Loading file flash:/msr810-cmw710-wifidog-r0809p25.bin...Done.

Loading file flash:/msr810-cmw710-wwd-r0809p25.bin...Done.

Loading file flash:/msr810-cmw710-boot-r0809p25.bin..........Done.

Image file flash:/msr810-cmw710-boot-r0809p25.bin is self-decompressing.....

............................................................................

............................................................................

............................................................................

............................................................................

............................................................................

............................................................................

............................................................................

.......................................................................

Done.

System image is starting...

Cryptographic algorithms tests passed.

Line con0 is available.

Press ENTER to get started.

<MSR810>%Jan  1 00:03:24:378 2011 MSR810 SHELL/5/SHELL_LOGIN: Console logged in from con0.

<MSR810>display version

H3C Comware Software, Version 7.1.064, Release 0809P25

Copyright (c) 2004-2020 New H3C Technologies Co., Ltd. All rights reserved.

H3C MSR810 uptime is 0 weeks, 0 days, 0 hours, 3 minutes