H3C MSR810 Product

Test Procedures

Copyright © 2021 New H3C Technologies Co., Ltd. All rights reserved.

No part of this manual may be reproduced or transmitted in any form or by any

means without prior written consent of New H3C Technologies Co., Ltd.

The information in this document is subject to change without notice.

## Statement

This Test procedure is only used to describe possible security issues. Without the permission of H3C Security, the content of this test procedure may not be modified or added or deleted, and it may not be used for commercial purposes in any way. Any direct or indirect consequences and losses caused by the dissemination and use of the information provided in this security bulletin shall be the responsibility of the user himself. The New H3C Security and the author of the security bulletin shall not bear any responsibility for this. The New H3C Security has the right to modify and interpret this test procedure. If you want to reprint or disseminate this test procedure, you must ensure the integrity of this test procedure, including all content such as the copyright statement.

Revision Record

| Date | Revision Version | Sec No. | Change Description | Author |
| --- | --- | --- | --- | --- |

Content

Statement	2

1.	WAN Select Mode 3G	5

2.	VRRP	6

3.	Traffic Shaping	10

4.	STP	14

5.	NAT Pool	18

6.	OSPFv6 Function	19

7.	VLAN (Access/Trunk/Native/Interface VLAN) and MTU	23

8.	DHCP(Client / Server)	36

9.	DHCP(Relay)	42

10.	OSPF function	48

11.	Traffic policing	51

12.	SIM authentication and IP assignment	63

13.	Maximum number of rules supported by ACL policy under single interface		66

14.	LLDP	79

15.	NAT Static	82

16.	Basic BGP Function	87

17.	Congestion Management/Avoidance	89

18.	BFD	96

19.	Configure and Management	106

20.	Maximum Number of Neighbors Per OSPF Process	109

21.	WAN Select Mode 4G	118

22.	IP Routing	122

22.1	Static IPv4	122

22.2	Routing Policy	122

23.	RADIUS authentication	125

24.	Syslog	132

25.	Gre over ipsec	133

26.	SNMP Traps	138

27.	IPV6	141

28.	Link aggregation	142

29.	Basic BGPv6	147

30.	BGP neighbor number	149

31.	L2TP	156

32.	ACL	159

33.	FORWARDING CAPACITY	163

34.	NTP	169

35.	Network Bridge	170

36.	Static IPv6	172

37.	Priority	174

38.	IPsec	181

39.	GRE	184

- LAN

  - LLDP

| Test Item Name: LLDP |  |
| --- | --- |
| Work Hour: 10 min |  |
| Test Topology: |  |
| Test Process: - Enable LLDP globally on DUT 1 and DUT 2. - Display the global local LLDP information on DUT 1. Result 1 is expected. - Display the local LLDP information about Port 1 on DUT 1. Result 2 is expected. - Display the LLDP information that Port 1 of DUT 1 has received from Port 1 of DUT 2. Result 3 is expected. |  |
| Expect Results: - The system displays the global local LLDP information on DUT 1, including the MAC address, device name, description, supported and enabled features of the system, hardware version, and firmware version. - The system displays the local LLDP information about Port 1 on DUT 1, including the port ID, management address, and port auto-negotiation configuration. - The system displays the LLDP information about Port 2 on DUT 2, including the port ID, management address, and port auto-negotiation configuration. |  |
| Actual Result： - Enable LLDP. - Display the global local LLDP information on DUT 1. Result 1 is expected. 3. Display the local LLDP information about Port 1 on DUT 1. Result 2 is expected 4. Display the LLDP information that Port 1 of DUT 1 has received from Port 1 of DUT 2. Result 3 is expected. |  |
| ☐ Passed | ☐ Failed |

  - STP

Test Item: Verify the basic functions of STP.

Work Hour: 60 min

Test Topology: Connect the devices as shown in figure

Test Process: 1. Set the spanning tree device priorities of DUT 1, DUT 2, and DUT 3 in descending order. 2. Send traffic from tester continuously. 3. Remove the cable between DUT 1 and DUT 2. Check the network topology and status of the continuous ping operation. Results 1 and 2 are expected. 4. Reconnect DUT 1 and DUT 2, and then remove the cable between DUT 1 and DUT 3. Check the network topology and status of the continuous ping operation. Results 1 and 2 are expected. 5. Reconnect DUT 1 and DUT 3, and then remove the cable between DUT 2 and DUT 3. Check the network topology and status of the continuous ping operation. Results 1 and 2 are expected.

Expected results： 1. Transient packets loss occurs after the cable is removed. 2. DUT 1 is always the root bridge, and the DUT with the highest root path cost is the end of the spanning tree.

Actual Result： - Enable STP globally [DUT] stp global enable [DUT] stp mode stp - Configure DUT1 as the root bridge [DUT1] stp root primary - Check the STP status on DUT - Send broadcast traffic on Port A - No broadcast storm occurs. - Send bi-direction unicast traffic, traffic is forwarding normally, and no traffic loss occurs - Remove the link between DUT 1 and DUT 2, transient packets loss occurs after the cable is removed, and then traffic recover. - Reconnect DUT 1 to DUT 2 and traffic can recover

| ☐ Passed | ☐ Failed |
| --- | --- |

  - VLAN (Access/Trunk/Native/Interface VLAN) and MTU

| Test Item Name: VLAN (Access/Trunk/Native/Interface VLAN) and MTU |  |
| --- | --- |
| Work Hour: 30 min |  |
| Test Topology: |  |
| Test Process: - Create VLAN 10 on DUT1. Assign Port1 to VLAN 10. - Create VLAN 11 on DUT1. Assign Port3 to VLAN 11. - Send Layer 2 broadcast traffic out of PortA of Tester, and receive the traffic on Port C of Tester. Result 1 is expected. - Create VLAN 10 on DUT1, and assign Port1 and Port2 to VLAN 10. Create VLAN 10 on DUT2, and assign Port1 and Port2 to VLAN 10. Cascade DUT1 and DUT2. - Send Layer 2 broadcast packets out of PortA of Tester. Result 2 is expected. - Create VLAN 10 on DUT1. Assign Port1 to VLAN 10. - Create VLAN 10 on DUT2. Assign Port1 to VLAN 10. - Set up a trunk link with the encapsulation format as 802.1Q between DUT1 and DUT2. Configure Port2 on DUT1 as a trunk port, and assign Port2 to VLAN 10. Configure Port2 on DUT2 as a trunk port, and assign Port2 to VLAN 10. Result 2 is expected. - Set up a trunk link with the encapsulation format as 802.1Q between DUT1 and DUT2. Configure Port2 on DUT1 as a trunk port, and assign Port2 to VLAN 1. Configure Port2 on DUT2 as a trunk port, and assign Port2 to VLAN 1. - Send common Layer 2 packets out of Port A of Tester. Result 3 is expected. - Create interface vlan10 on DUT1, assign Port1 to VLAN 10 and configure IP address 10.10.10.1/24. - Create interface vlan20 on DUT1, assign Port2 to VLAN 20 and configure IP address 10.20.20.1/24. Result 4 is expected. - Adjust the MTU value of all ports to the maximum. |  |
| Expect Results: - Port C of Tester cannot receive Layer 2 broadcast traffic sent out of Port A of Tester. Different VLANs are isolated at Layer 2. - Packets sent out Port A of Tester are received on Port B of Tester. - Packets sent out Port A of Tester cannot be received on Port B of Tester. - Port A of Tester can ping the Port2 on DUT1. - Validate the device support the function of VLAN |  |
| Actual Result： - Create VLAN 10 on DUT1. Assign Port1 to VLAN 10. - Create VLAN 11 on DUT1. Assign Port3 to VLAN 11. - Send Layer 2 broadcast traffic out of PortA of Tester, and receive the traffic on Port C of Tester. Result 1 is expected. Add raw stream Delete the IPv4 header. Start the traffic Port C of Tester cannot receive Layer 2 broadcast traffic sent out of Port A of Tester - Create VLAN 10 on DUT1, and assign Port1 and Port2 to VLAN 10. Create VLAN 10 on DUT2, and assign Port1 and Port2 to VLAN 10. Cascade DUT1 and DUT2. - Send Layer 2 broadcast packets out of PortA of Tester. Result 2 is expected. Add raw stream Delete the IPv4 header. Start the traffic Packets sent out Port A of Tester are received on Port B of Tester. - Create VLAN 10 on DUT1. Assign Port1 to VLAN 10. - Create VLAN 10 on DUT2. Assign Port1 to VLAN 10. - Set up a trunk link with the encapsulation format as 802.1Q between DUT1 and DUT2. Configure Port2 on DUT1 as a trunk port, and assign Port2 to VLAN 10. Configure Port2 on DUT2 as a trunk port, and assign Port2 to VLAN 10. Result 2 is expected. Packets sent out Port A of Tester are received on Port B of Tester. - Set up a trunk link with the encapsulation format as 802.1Q between DUT1 and DUT2. Configure Port2 on DUT1 as a trunk port, and assign Port2 to VLAN 1. Configure Port2 on DUT2 as a trunk port, and assign Port2 to VLAN 1. - Send common Layer 2 packets out of Port A of Tester. Result 3 is expected. Packets sent out Port A of Tester cannot be received on Port B of Tester. - Create interface vlan10 on DUT1, assign Port1 to VLAN 10 and configure IP address 10.10.10.1/24. - Create interface vlan20 on DUT1, assign Port2 to VLAN 20 and configure IP address 10.20.20.1/24. Result 4 is expected. Add a device to port A of tester Configure IP address and gateway Learn the ARP Ping Port 2 on DUT1 from Port A on tester. - Adjust the MTU value of all ports to the maximum. |  |
| ☐ Passed | ☐ Failed |

- WAN Select Mode 3G

| Test Item Name: WAN Select mode 3G |  |
| --- | --- |
| Work Hour: 30 min |  |
| Test Process: - Pre-condition is that all device work normally to validate the device support the function of WAN Select mode 3G. - Insert the SIM / UIM card (without PIN code protection enabled) into the 3G-MODEM. Insert USB-3G-MODEM into USB0 port, or insert SIC-3G-MODEM into slot0; - Configure dialing function on DUT. |  |
| Excepted Result 1. The work mode is only WCDMA. |  |
| Actual results - Configure on DUT - Check the configuration on DUT |  |
| ☐ Passed | ☐ Failed |

- VRRP

Test Item Name: Verify the VRRP load balancing function.

Work Hour: 60 min

Test Topology: Connect the devices as shown in the following figure

Test Process: - Each router is configured to ensure the connectivity of the network. Two routers DUT1 and DUT2 backup each other's gateways. - Configure VRRP on DUT 1 and DUT 2, DUT 1 as the master and DUT 2 as the backup. Display the state of the VRRP on DUT 1 and DUT 2. Result 1 is expected. - Configure track and BFD, DUT 1 monitors the uplink, DUT 2 monitors DUT 1. - Configure the virtual IP address of the VRRP as the default gateway. Send traffic from port A to Port B on Tester. Result 2 is expected. - Shut down Port 1 on DUT 1, and send traffic from port A to Port B on Tester. Result 3 is expected.

Expected results: - DUT 1 is the master in the VRRP, and DUT 2 is the backup. - DUT 1 is the master in the VRRP group, traffic can pass normally. Traffic from Tester is forwarded through DUT 1. - DUT 2 is the master in the VRRP, traffic can pass normally, and traffic fromTester is forwarded through DUT 2.

Actual results: - Configuration: DUT1 DUT2 CE 1 - DUT 1 is the master in the VRRP group, and DUT 2 is the backup in the VRRP group. - Construct traffic and trafic can pass normally. Traffic from CE 2 can be forwarded to CE 1. - Shut down Port 1 on DUT 1, DUT 2 is the master in the VRRP group, traffic can be forwarded after some packets loss, and traffic from A to B on tester is forwarded through DUT 2.

| ☐ Passed | ☐ Failed |
| --- | --- |

- Traffic Shaping

Test Item: Traffic shaping

Work Hour: 30 min

Test Topology: Validate the device support the function of Traffic shaping

Test Process: - Configure an ACL to match IP on DUTA; - Configure the interface GTS matching the above ACL on the output interface of DUTA and DUTB, the speed limit is 5Mbps; - The test instrument sends a packet stream that conforms to the ACL matching rules, from Port 1 connected to DUTA to Port 2 of the test instrument, and the flow size is 10Mbps - Observe the message reception status of the Port2 port of the tester, and there should be the expected result 1; - View the GTS statistics of the interface and get the expected result 2.

Excepted Result: 1. The rate of receiving messages on the tester Port2 is the cir value rate of GTS 5Mbps; 2. The interface GTS has statistics of packets in the queue and statistics of packet loss.

Actual results: - Configuration: DUT_1: # acl advanced 3000 rule 0 permit ip source 1.1.1.0 0.0.0.255 # traffic classifier 1 operator and if-match acl 3000 # traffic behavior 1 gts cir 5000 # qos policy 1 classifier 1 behavior 1 # interface Ethernet0/0 port link-mode route description connect with port1 ip address 1.1.1.1 255.255.255.0 # interface Ethernet0/1 port link-mode route description connect with DUT2 ip address 3.1.1.1 255.255.255.0 qos apply policy 1 outbound # DUT_2: # interface Ethernet0/1 port link-mode route description connect with DUT1 ip address 3.1.1.2 255.255.255.0 # interface Ethernet0/2 port link-mode route description connect with port1 ip address 2.1.1.1 255.255.255.0 # - Configure traffic flow: Port 1 Port 2 9868376/(1500*8)=822

| ☐ Passed | ☐ Failed |
| --- | --- |

- NAT Pool

Test Item Name: To test the NAT function when hosts on the private network access servers on the public network.

Work Hour: 30 min

Test Topology:

Test Process: - Configure an address group, and add an address range to the group. - Configure an ACL to identify packets. - Enable outbound dynamic NAT on interface.

Expected results 1. PC1 can communicate with PC2. If you check the session table, you can see that the source address and source port are translated.

Actual results - Configuration: - Display the nat session

| ☐ Passed | ☐ Failed |
| --- | --- |

- OSPFv6 Function

Test Item: Basic OSPFv6 function

Work Hour: 30 min

Test Topology:

Test Setup 1. Assign IPv6 address on DUT as the diagram above 2. Configure ospfv3 on each DUT1 and DUT2.

Expected Result: - OSPF v3 can peer and show on routing table

Actual Results: - Configuration: DUT 1 DUT 2 - Display result:

| ☐ Passed | ☐ Failed |
| --- | --- |

- DHCP(Client / Server)

| Test Item Name: DHCP(Client / Server) |  |
| --- | --- |
| Work Hour: 20 min |  |
| Test Topology: |  |
| Test Process: - Assign IP addresses for the Port 2 and Port 3. - Enable DHCP. - Exclude the IP addresses of the gateway from dynamic assignment. - Configure parameters in DHCP address pool 0, including the subnet for address assignment, domain name suffix, DNS server address, and gateway address. - Apply DHCP address pool 0 to Port 2 on Server. - Configure the PC to request network configuration parameters from the DHCP server. Result 1 is expected. - Configure parameters in DHCP address pool 1, including the subnet for address assignment, domain name suffix, DNS server address, and gateway address. - Apply DHCP address pool 1 to Port 3 on Server. - Configure Port 1 on Client to use DHCP to obtain an IP address. - Display DHCP client information about Port 1 on Client. Result 2 is expected. |  |
| Expect Results: - The PC obtains an IP address and other configuration parameters from address pool 0. The obtained gateway address and the DNS server address are the same as the gateway address and the DNS server address for address pool 0. - The Client obtains an IP address and other configuration parameters from address pool 1. The obtained gateway address and the DNS server address are the same as the gateway address and the DNS server address for address pool 1. |  |
| Actual Result： Configuration on DHCP Server： - Assign IP addresses for the Port 2 and Port 3. - Enable DHCP - Exclude the IP addresses of the gateway from dynamic assignment. - Configure parameters in DHCP address pool 0, including the subnet for address assignment, domain name suffix, DNS server address, and gateway address. - Apply DHCP address pool 0 to Port 2 on Server. - Configure the PC to request network configuration parameters from the DHCP server. Result 1 is expected. Select computer network card Set the network card to obtain IP address automatically The PC can get IP address from DHCP server. - Configure parameters in DHCP address pool 1, including the subnet for address assignment, domain name suffix, DNS server address, and gateway address. - Apply DHCP address pool 1 to Port 3 on Server. - Configure Port 1 on Client to use DHCP to obtain an IP address. On Client: - Display DHCP client information about Port 1 on Client. Result 3 is expected. |  |
| ☐ Passed | ☐ Failed |

- DHCP(Relay)

| Test Item Name: DHCP(Relay) |  |
| --- | --- |
| Work Hour: 20 min |  |
| Test Topology: |  |
| Test Process: - Assign IP addresses for the Port 1. - Enable DHCP on DHCP Server. - Exclude the IP addresses of the gateway from dynamic assignment. - Configure parameters in DHCP address pool 0, including the subnet for address assignment, domain name suffix, DNS server address, and gateway address. - Add the static route to the 10.1.1.1 network segment on the server, the next hop is Port 2. - Enable DHCP on DHCP Relay. - Enable the DHCP relay agent on Port 3. - Configure PC to request an IP address from the DHCP server. Result 1 is expected. |  |
| Expect Results: - PC obtains an IP address on subnet 10.1.1.0/24. |  |
| Actual Result： Configuration on DHCP Server: - Assign IP addresses for the Port 1. - Enable DHCP on DHCP Server. - Exclude the IP addresses of the gateway from dynamic assignment. - Configure parameters in DHCP address pool 0, including the subnet for address assignment, domain name suffix, DNS server address, and gateway address. - Add the static route to the 10.1.1.1 network segment on the server, the next hop is Port 2. Configuration on DHCP Relay: - Assign IP addresses for the Port 2 and Port 3. - Enable DHCP on DHCP Relay. - Enable the DHCP relay agent on Port 3. - Configure PC to request an IP address from the DHCP server. Result 1 is expected. Select computer network card Set the network card to obtain IP address automatically The PC can get IP address from DHCP server. |  |
| ☐ Passed | ☐ Failed |

- OSPF function

| Test Item Name: OSPF function |  |
| --- | --- |
| Work Hour：15 min |  |
| Test Purpose: Verify that the device supports DHCP |  |
| Test Topology: |  |
| Test Process: - Configure IP addresses for Port1 and Port2 - Configure IP addresses for Loopback interfaces on DUT1 and DUT2 - Configure OSPF on DUT1 and DUT2. Result 1 is expected. - Check the routing-table on DUT1 and DUT2. Result 2 is expected. - Perform ping between the two loopback interfaces. Result 3 is expected. |  |
| Expect Results: - DUT1 and DUT2 can build OSPF neighbors - The loopback routing can be seen in the routing-table, and its type is OSPF - DUT1 and DUT2 can successfully ping each other. |  |
| Actual Result： Configuration on DHCP Server： - Configure IP addresses for Port1 and Port2 - Configure IP addresses for Loopback interfaces on DUT1 and DUT2 - Configure OSPF on DUT1 and DUT2. Result 1 is expected. Check the OSPF neighbor relationship on two devices: DUT1 and DUT2 can build OSPF neighbors - Check the routing-table on DUT1 and DUT2. Result 2 is expected. Display the IP routing table The loopback routing can be seen in the routing-table, and its type is OSPF View the OSPF routing in IP routing table - Perform ping between the two loopback interfaces. Result 3 is expected. DUT1 and DUT2 can successfully ping each other. |  |
| ☐ Passed | ☐ Failed |

- Traffic policing

| Test Item Name: Traffic policing |  |
| --- | --- |
| Work Hour: 30 min |  |
| Test Topology: |  |
| Test Process: - Configure IP addresses for Port1 and Port2 - Configure ACL 2001 and ACL 2002 to permit the packets with the source IP address 10.0.0.10 and 10.0.0.11, respectively. - Create a traffic class named c1, and use ACL 2001 as the match criterion - Create a traffic class named c2, and use ACL 2002 as the match criterion. - Create a traffic behavior named b1, and configure a traffic policing action (CIR 5 M). - Create a traffic behavior named b2, and configure a traffic policing action (CIR 10 M). - Create a QoS policy named car, and associate traffic classes c1 and c2 with traffic behaviors b1 and b2 in QoS policy car, respectively. - Apply QoS policy car to the inbound direction of Port 1. - Send two flows with source IP address 10.0.0.10 and source IP address 10.0.0.11 to DUT from Port A, both at 15 Mbps. - Observe the received flows on the receiver. Result 1 is expected. |  |
| Expect Results: - Port A and Port B of the tester send and receive traffic correctly. |  |
| Actual Result: - Configure IP addresses for Port1 and Port2 - Configure ACL 2001 and ACL 2002 to permit the packets with the source IP address 10.0.0.10 and 10.0.0.11, respectively. - Create a traffic class named c1, and use ACL 2001 as the match criterion - Create a traffic class named c2, and use ACL 2002 as the match criterion. - Create a traffic behavior named b1, and configure a traffic policing action (CIR 5 M). - Create a traffic behavior named b2, and configure a traffic policing action (CIR 10 M). - Create a QoS policy named car, and associate traffic classes c1 and c2 with traffic behaviors b1 and b2 in QoS policy car, respectively. - Apply QoS policy car to the inbound direction of Port 1. - Send two flows with source IP address 10.0.0.10 and source IP address 10.0.0.11 to DUT from Port A, both at 15 Mbps. Add device on Port A and Port B, configure the IP address Add two raw streams: Stream 1: Configure the IP address Stream 2: Configure the IP address Change the scheduling mode to Load per Stream Block Change the speed of each stream to 15Mbps and the frame size to 1450 Start the traffic: Start to send the stream 1 which source IP address is 10.0.0.10: Stop the stream 1 and start to send the stream 2 which source IP address is 10.0.0.11: - Observe the received flows on the receiver. Result 1 is expected. |  |
| ☐ Passed | ☐ Failed |

- SIM authentication and IP assignment

| Test Item Name: SIM Authentication and IP Assignment |  |
| --- | --- |
| Work Hour: 20 min |  |
| Test Topology: |  |
| Test Process: - Insert a SIM or UIM card (not enabled with PIN verification) into a 4G modem. Then install the 4G modem on the DUT. - Configure dialer group 1 and configure DDR to place calls for IP packets. - Enter Cellular 1/0 interface view. Channelize the cellular interface into an Eth-channel interface. - Enter Eth-channel 1/0:0 interface view. And enable traditional DDR on Eth-channel 1/0:0. - Associate Eth-channel 2/4/0:0 with dialer group 1 - Configure the timeout period for establishing a call to 5 seconds - Set the interval for DDR to make the next call attempt to 5 seconds. - Configure the dial string for placing calls as *99. - Configure an IP address for Eth-channel 1/0:0. - Set the TCP MSS to 1280 bytes. - Configure NAT outbound - Ping an IP address on the DUT to trigger 4G dialup for Internet connection. Then ping http://www.baidu.com/. Result 1 is expected. |  |
| Expect Results: - The DUT can ping http://www.baidu.com/ |  |
| Actual Result： - Insert a SIM or UIM card (not enabled with PIN verification) into a 4G modem. Then install the 4G modem on the DUT. You need to insert sim card after power off the device. Check whether the device recognizes the SIM card: - Configure dialer group 1 and configure DDR to place calls for IP packets. - Enter Cellular 1/0 interface view. Channelize the cellular interface into an Eth-channel interface. - Enter Eth-channel 1/0:0 interface view. And enable traditional DDR on Eth-channel 1/0:0. - Associate Eth-channel 2/4/0:0 with dialer group 1 - Configure the timeout period for establishing a call to 5 seconds - Set the interval for DDR to make the next call attempt to 5 seconds. - Configure the dial string for placing calls as *99#. Different ISP have different dialer numbers - Configure an IP address for Eth-channel 1/0:0. - Set the TCP MSS to 1280 bytes. - Configure NAT outbound - Ping an IP address on the DUT to trigger 4G dialup for Internet connection. Then ping http://www.baidu.com/. Result 1 is expected. After configuration, the interface E-Ch1/0:0 will get the IP address Display IP address on interface E-Ch1/0:0 received from Cellular: Ping http://www.baidu.com |  |
| ☐ Passed | ☐ Failed |

- Maximum number of rules supported by ACL policy under single interface

| Test Item Name: Maximum number of rules supported by ACL policy under single interface |  |
| --- | --- |
| Work Hour: 30 min |  |
| Test Topology: |  |
| Test Process: - Configure IP address for Port 1 and Port 2 on DUT. - Add the default static route, the next hop is 1.1.1.2. - Use the Tcl script to create 3000 rules - Create a traffic class named c1, and use ACL 3000 as the match criterion - Create a traffic behavior named b1, and deny the traffic - Create a QoS policy named q1, and associate traffic classes c1 with traffic behaviors b1 in QoS policy q1. - Create a Router on the Port 2 port of the tester to simulate the DUT default gateway: - Create a stream on Port 1 and send 3000 packets. The source address of the packet is the same as the address of the 3000 rule configured in the ACL: - Send the traffic on Port A in bursts mode. The packets matching ACL 3000 will be denied. Therefore, all the packets will be denied. Result 1 is expected. |  |
| Expect Results: - Port B can't receive any packets. The number of valid rules in the ACL in the DUT is more than 3000 |  |
| Actual Result： - Clear interface packet statistics on DUT 1. - Add the default static route, the next hop is 1.1.1.2. - Use the Tcl script to create 3001 rules set m 0 set n 1 for {set i 0} {$i<3000} {incr i} { acl advanced 3000 rule $i permit ip source 10.0.$m.$n 0 incr n if {$n % 256 == 0} { incr m set n 0 } } It will take more than an hour to create these rules. - Create a traffic class named c1, and use ACL 3001 as the match criterion - Create a traffic behavior named b1, and deny the traffic - Create a QoS policy named rule, and associate traffic classes c1 with traffic behaviors b1 in QoS policy rule. - Create a Router on the Port 2 port of the tester to simulate the DUT default gateway and edit the IP and MAC address. Add the Router: Edit the Router: Learn the ARP: - Create a stream on Port 1 and send 3000 packets. The source address of the packet is the same as the address of the 3000 rule configured in the ACL: Enter the Frame tab and change the Source IP to 10.0.0.1 and the Gateway to 10.0.255.254 Right-click on the Source field and click Insert IPv4 Modifier Click on the Last Page and you can see that the Source IP of the last Flow is 10.0.11.184. - Send the traffic on Port A in bursts mode. The packets hitting the ACL are intercepted. Therefore, the number of valid rule entries in the ACL is 3000 - Port B has received the number of packets. Result 1 is expected. Change the Duration mode of the data stream to Bursts, and change the Burst(s) value to 3000 Perform ARP/ND analysis on the Stream Block Click Port load and change the stream rate to 0.1% Ping 1.1.1.2 on the DUT to enable the DUT to learn the ARP address of the default route Clear previous results Send traffic and observe the results Port B can't receive any packets. So the number of valid rules in the ACL in the DUT is more than 3000 |  |
| ☐ Passed | ☐ Failed |

- NAT Static

| Test Item Name: Nat static |  |
| --- | --- |
| Work Hour: 30 min |  |
| Test Topology: |  |
| Test Process: - Configure IP address on DUT 1, DUT 2 and Tester. - Configure one-to-one static address translation mapping between intranet IP address and external IP address, The configured static address translation takes effect on the Port 1 of DUT1. - Ping from Tester to DUT2, result 1 is expected - Display nat static and display nat session verbose, result 2 is expected. |  |
| Expect Results: - Tester can ping DUT2. - Through the following display command, you can verify that the above configuration is successful and NAT session information is generated when the accessing an external network. |  |
| Actual Result： - Configure IP address on DUT1, DUT2 and Tester. 2. Configure nat static and enable this function on Port 1 of DUT1. 3. Ping from Tester to DUT2, result 1 is expected. 4. Input relative commands, result 2 is expected |  |
| ☐ Passed | ☐ Failed |

- Basic BGP Function

| Test Item Name: Basic BGP Function |  |
| --- | --- |
| Work Hour: 20 min |  |
| Test Topology: |  |
| Test Process: - Configure IP address on Loopback interface and Port 1 of DUT 1 and DUT 2. - Enable EBGP peer. - Ping from Loopback interface of DUT1 to Loopback interface of DUT2, result 1 is expected. - Display the BGP peer information on DUT 1. Result 2 is expected. |  |
| Expect Results: - Can ping each other. - On DUT1, the state of the neighbor relationship for DUT2 is established. |  |
| Actual Result： - Configure IP address. 2. Configure BGP 3. Ping from Loopback interface of DUT1 to Loopback interface of DUT2. Result 1 is expected. 4. Display the BGP peer information on DUT 1. Result 2 is expected |  |
| ☐ Passed | ☐ Failed |

- Congestion Management/Avoidance

| Test Item Name: Congestion Management/Avoidance |  |
| --- | --- |
| Work Hour: 80 min |  |
| Test Topology: |  |
| Test Process: - Configure IP address on DUT and Tester. - Configure one sp queue and seven queue with different priority and weight by using qos policy. - Port 1 of Tester constructs eight streams with different TOS value and the same loading 2,000,000 bps, and send these streams at the same time. - Enable speed limitation to 1,000,000 on Port 2 of DUT. Result 1 is expected. - Enable speed limitation to 5,300,000 on Port 2 of DUT. Result 2 is expected. |  |
| Expect Results: - Without congestion, the bandwidth utilization of all traffic is the same. - The utilization is consistent with the previous QoS policy when congestion occurs. |  |
| Actual Result： - Configure IP address on DUT and Tester. - Configure 3 queue with different DSCP priority and bandwidth percent by using qos policy. - Port 1 of Tester constructs 3 streams with different DSCP value and one stream with no DSCP tag, and send these streams at the same time. Result 1 is expected. Just take af11 DSCP stream as an instance. - Repeat step 3 to create the rest streams. Click apply and click start all traffic, adjust the Tester view. Result 1 is expected. - Enable speed limitation to 100Kbps on Port 2 of DUT. Result 2 is expected. |  |
| ☐ Passed | ☐ Failed |

- BFD

| Test Item Name: BFD |  |
| --- | --- |
| Work Hour: 100 min |  |
| Test Topology: |  |
| Test Process: - Configure IP address on DUT 1, DUT 2 and Tester. - Enable Ospf function. Result 1 is expected. - Tester sends layer 3 traffic from Port 1 of Tester to Port 2 of Tester, and modifies OSPF cost value of Port 2 interface of DUT 1. Result 2 is expected. - Shutdown the link between L2 switches, result 3 is expected. - Undo shutdown the link. Configure the BFD protocol under the corresponding interfaces of DUT 1 and DUT 2. Result 4 is expected. - Repeated step 4, result 5 is expected. |  |
| Expect Results: - Ospf neighbor can be established. Port 1 of tester has 2 paths reaching to Port 2 of tester. - The traffic is forwarded through L2 switches. - The traffic is forwarded through Port 1 of DUT 1. Convergence time is more than 300ms. - BFD session is established. - The traffic is forwarded through Port 1 of DUT 1. Convergence time is less than 300ms. |  |
| Actual Result： - Configure IP address on DUT 1, DUT 2 and Tester. - Enable Ospf function. - Check Ospf state. Result 1 is expected. - Tester sends layer 3 traffic from Port 1 of Tester to Port 2 of Tester - Modifies OSPF cost value of Port 1 interface of DUT 1. Adjust the view on Tester, there is no packet drop and Result 2 is expected. - Unplug the cable between L2 switches, The traffic is forwarded through Port 1 of DUT 1. Without bfd configuration, the packet drop is 300,000 frames This means the convergence time is 35s (300,000/8446).Result 3 is expected. - Repair network cable between L2 switches. Shutdown and open Port 1 of DUT1 to make sure the traffic run between switches. Configure the BFD protocol under the corresponding I interfaces of DUT 1 and DUT 2. Result 4 is expected. - BFD session is established. - Unplug the cable between L2 switches, The traffic is forwarded through Port 1 of DUT 1. The drop packet is 1226 frames, so the convergence time is 145ms (1226/8445). Result 5 is expected. |  |
| ☐ Passed | ☐ Failed |

- Configure and Management

| Test Item Name: Configure and Management |  |
| --- | --- |
| Work Hour: 10 min |  |
| Test Topology: |  |
| Test Process: - Configure IP address. Result 1 is expected. - Enable the SNMP agent. - Enable all SNMP versions. - Create an SNMP read-only community named public and read-write community named private. - On the MIB browser of the PC, use SNMPv2c to access the DUT and request the value of MIB object sysObjectID. Result 2 is expected. |  |
| Expect Results: - Can ping each other. - The MIB browser can obtain the value of sysObjectID through SNMPv2c successfully. |  |
| Actual Result： - Configure IP address and PC ping DUT. Result 1 is expected - Configure SNMP - MIB browser of the PC can use SNMPv2c to access the DUT and request the value of MIB object sysObjectID. |  |
| ☐ Passed | ☐ Failed |

- Maximum Number of Neighbors Per OSPF Process

| Test Item Name: Maximum Number of Neighbors Per OSPF Process |  |
| --- | --- |
| Work Hour: 100 min |  |
| Test Topology: |  |
| Test Process: - Create 512 vlan and enable OSPF. - In tclsh view, using scripts to create 512 sub-interface with vlan-tag 1 to 512 and enable all the ports in area 0 of OSPF. Result 1 is expected - Emulation 512 devices to establish neighbors in the tester. Result 2 is expected. |  |
| Expect Results: - Create 512 sub-interface successfully. - Neighbor relationship of OSPF is estabished. |  |
| Actual Result： - Create 512 vlan and enable OSPF. - In tclsh view, using scripts to create 512 sub-interface with vlan-tag 1 to 512 and enable all the ports in area 0 of OSPF. Result 1 is expected. - Emulation 512 devices to establish neighbors in the tester. - Start arp solution. - Choose OSPFv2, start all device and check on DUT. Result 2 is expected. NOTE: 1. MSR810 can only create 512 vlan, so the maximum number of neighbors per OSPF process can not beyond 512. Just learn this method. 2.Scripts: for {set i 1} {$i <= 512} {incr i} { interface g0/3.$i; ip address 10.[expr $i/256].[expr $i%256].2 24 vlan-type dot1q vid [expr $i] ospf 1 area 0 } |  |
| ☐ Passed | ☐ Failed |

- WAN Select Mode 4G

| Test Item Name: WAN Select Mode 4G |  |
| --- | --- |
| Work Hour: 20 min |  |
| Test Topology: |  |
| Test Process: - Insert a SIM or UIM card (not enabled with PIN verification) into a 4G modem. Result 1 is expected - Configure the 4G modem and dialup settings on the DUT. Result 2 is expected - Ping an google IP address on the DUT. Result 3 is expected. |  |
| Expect Results: - The SIM Status is OK. - Mobile network name match to the SIM card and the mode is lte. - Can ping google network. |  |
| Actual Result： - Insert a CHN-CT SIM card into a 4G modem.Then plug the DUT and check, result 1 is expected. - Configure the 4G modem and check the status. Result 2 is expected. - Ping google IP address. Result 3 is expected. |  |
| ☐ Passed | ☐ Failed |

- IP Routing

  - Static IPv4

| Test Item | Validate the device support the function of Static Ipv4 |
| --- | --- |
| Work Hour | 30min |
| Test Setup | Network diagram: Refers to testing diagram 1. Assign IP address on DUT as the diagram above. 2. Configure static routes and default routes on each DUT1 and DUT2. |
| Excepted Result | DUT1 can ping to IP loopback of DUT2. |
| Actual results | Configuration: DUT_1 DUT_2 Result: |

  - Routing Policy

| Test Item: Routing policy |  |
| --- | --- |
| Work Hour: 60min |  |
| Test Topology: |  |
| Test Process: 1. Configure BGP on each DUT1 and DUT2 2. Creating a routing policy. 3. Configuring if-match clauses. 4.Configuring apply clauses. 5.Configuring the continue clause. |  |
| Excepted Result: Can tag local AS number 109 |  |
| Actual Results: Configuration: DUT_1 DUT2: Result: DUT_1 |  |
| ☐ Passed | ☐ Failed |

- RADIUS authentication

| Test Item Name: RADIUS authentication |  |
| --- | --- |
| Work Hour: 30 min |  |
| Test Topology: |  |
| Test Process: Configure the RADIUS server correctly, add two accounts for the telnet users and specify the passwords for the accounts. - Telnet to the switch from Host A Enable the telnet server. - Enable scheme authentication for VTY line 0 to 63. - Enable the default user role feature. - Create a RADIUS scheme named test. - Specify the primary authentication and accounting servers. - Set the shared keys to expert in plaintext form for secure RADIUS communication. - Exclude domain names from the usernames sent to the RADIUS server. - Create an ISP domain named test. - Specify the ISP domain as the default ISP domain. - Configure the ISP domain to use RADIUS scheme test as the default authentication, authorization, and accounting methods. - Configure the RADIUS server correctly, add two accounts for the telnet users, and specify the passwords for the accounts. - Telnet to the device from Host A and Host B, and enter the correct usernames and passwords. Result 1 is expected. |  |
| Expect Results: - The users can telnet to the switch from Host A and Host B. |  |
| Actual Result： - Configuration on DUT: # telnet server enable # line vty 0 63 authentication-mode scheme user-role network-admin user-role network-operator # role default-role enable # radius scheme test primary authentication 192.168.207.57 primary accounting 192.168.207.57 key authentication simple expert key accounting simple expert user-name-format without-domain nas-ip 192.168.207.144 # domain test authentication login radius-scheme test authorization login radius-scheme test accounting login radius-scheme test # domain default enable test # - Configuring the RADIUS server: - Add the router to the IMC platform as an access device: Log in to IMC, click the User tab, and select User Access Policy > Access Device Management > Access Device from the navigation tree. Then, click Add to configure an access device as follows: a. Set the shared key for secure RADIUS communication to expert. b. Set the ports for authentication and accounting to 1812 and 1813, respectively. c. Select an access device from the device list or manually add an access device. In this example, the device IP address is 192.168.207.144 (equals to nas-ip address 192.168.207.144) d. Use the default values for other parameters and click OK. The IP address of the access device specified here must be the same as the source IP address of the RADIUS packets sent from the router. The source IP address is chosen in the following order on the router: IP address specified by using the nas-ip command. IP address specified by using the radius nas-ip command. IP address of the outbound interface (the default). - Add an access user for device management: Select Access User > All Access Users from the navigation tree. Then, click Add to configure a device management account as follows: a. Enter account name zhh and specify the password as admin form the Access Information. b. Specify 10 from the Max Concurrent Logins list. d. Click OK. - Telnet to the device, and enter the correct usernames and passwords. Result 1 is expected. |  |
| ☐ Passed | ☐ Failed |

- Syslog

| Test Item：Syslog. |  |
| --- | --- |
| Work Hour：30min |  |
| Test Topology： Connect the devices as shown in figure |  |
| Test Process： 1. Configure SW1 syslog service 2. NSM server receive syslog information |  |
| Expected results： The syslog information generated by the router can be sent to the syslog server in time |  |
| Actual Result： - Open the 3CD on computer and choose syslog Server - Configure syslog server on DUT <Device>system-view [Device]info-center enable [Device]info-center loghost 192.168.207.46-------this is the computer’s ip (this computer serves as syslog server) - Shutdown other Port on DUT and check whether the server can receive the log |  |
| ☐ Passed | ☐ Failed |

- Gre over ipsec

| Test Item: VPN GRE+IPSEC |  |
| --- | --- |
| Work Hour: 120min |  |
| Test Topology: |  |
| Test Process: 1. Create GRE tunnel. 2. Configure IPsec to protect the traffic. |  |
| Excepted Result:Tunnel up and can ping. |  |
| Actual results: Configuration: DUT1: # interface LoopBack0 ip address 1.1.1.1 255.255.255.255 ospf 1 area 0.0.0.0 # # interface GigabitEthernet0/1 port link-mode route combo enable copper ip address 12.1.1.2 255.255.255.0 # interface GigabitEthernet0/2 port link-mode route combo enable copper ip address 10.1.1.1 255.255.255.0 ospf 1 area 0.0.0.0 ipsec apply policy h3c # interface Tunnel1 mode gre ip address 16.1.1.1 255.255.255.0 source 10.1.1.1 destination 10.2.2.2 # ip route-static 13.1.1.0 24 10.1.1.2 # acl advanced 3100 rule 15 permit ip source 12.1.1.0 0.0.0.255 destination 13.1.1.0 0.0.0.255 # ipsec transform-set h3c esp encryption-algorithm aes-cbc-128 esp authentication-algorithm sha384 pfs dh-group14 # ipsec policy h3c 1 isakmp transform-set h3c security acl 3100 local-address 10.1.1.1 remote-address 10.2.2.2 ike-profile h3c # ike profile h3c keychain h3c local-identity address 10.1.1.1 match remote identity address 10.2.2.2 255.255.255.0 proposal 1 # ike proposal 1 encryption-algorithm aes-cbc-128 dh group14 authentication-algorithm sha384 sa duration 7200 # ike keychain h3c pre-shared-key address 10.2.2.2 255.255.255.0 key cipher $c$3$Nn6vgeU7yIwfkcFFC+DXqF8TaSI9+A== # return DUT2; # ospf 1 area 0.0.0.0 # interface LoopBack0 ip address 2.2.2.2 255.255.255.255 # interface GigabitEthernet0/0 port link-mode route combo enable copper ip address 10.1.1.2 255.255.255.0 ospf 1 area 0.0.0.0 # interface GigabitEthernet0/1 port link-mode route combo enable copper ip address 10.2.2.1 255.255.255.0 ospf 1 area 0.0.0.0 # Result : |  |
| ☐ Passed | ☐ Failed |

- SNMP Traps

| Test Item Name: SNMP Traps |  |
| --- | --- |
| Work Hour: 10 mins |  |
| Test Topology: |  |
| Test Process: - Enable SNMP Trap. - Configure DUT to support SNMPv2 access. - Create an SNMP read-only community named public and read-write community named private. - Configure the SNMP 2 Trap receiver. - Simulate to generate Trap event, result 1 is expected. |  |
| Expect Results: - The trap information could be seen on MIB Brower. |  |
| Actual Result： - Enable SNMP Trap. - Configure DUT to support SNMPv1 access. - Create an SNMP read-only community named public and read-write community named private. - Configure the SNMP Trap receiver. - Simulate to generate Trap event, result 1 is expected. - Shutdown a port , result 1 is expected. Undo shutdown |  |
| ☐ Passed | ☐ Failed |

- IPV6

| Test Item Name: IPv6 |  |
| --- | --- |
| Work Hour: 10 mins |  |
| Test Topology: |  |
| Test Process: 1. Enable the IPv6 forwarding function on Router A and Router B. 2. Configure IPv6 global unicast addresses for the interfaces. |  |
| Expect Results: - Router A and Router B can ping ipv6 address each other. |  |
| Actual Result： |  |
| ☐ Passed | ☐ Failed |

- Link aggregation

| Test Item Name: link aggregation |  |
| --- | --- |
| Work Hour: 30 mins |  |
| Test Topology: |  |
| Test Process: - Create the three-layer static aggregation group 1 and add the ports port1 and port2 to the static aggregation group 1 and configure the IP of the aggregation group to be in the same network segment. Create the vlan2 on the device dut1 and set the port port3 to allow the vlan2 packet to pass. And create a virtual interface vlan2, configure the IP address. - Tester port A sends the destination IP address to the IP address of the DUT2 aggregation group. The source IP address and the DUT1 virtual interface VLAN 2 are forwarded on the same network segment. The expected result is 1. - Disconnect one link between device DUT1 and device DUT2 (link with traffic); observe the reception of packet traffic on Tester port B. Expected result 2. - Restore the link and observe the received traffic of Tester port B. The expected result is 2. |  |
| Expect Results: - Port B receives the broadcast traffic sent by Port A. - Transient traffic loss occurs, and then Port B receives the broadcast traffic sent by Port A. |  |
| Actual Result： - Create the three-layer static aggregation group 1 and add the ports port1 and port2 to the static aggregation group 1 and configure the IP of the aggregation group to be in the same network segment. Create the vlan2 on the device dut1 and set the port port3 to allow the vlan2 packet to pass. And create a virtual interface vlan2, configure the IP address. - Tester port A sends the destination IP address to the IP address of the DUT2 aggregation group. The source IP address and the DUT1 virtual interface vlan 2 are forwarded on the same network segment. The expected result is 1. - Disconnect one link between device DUT1 and device DUT2 (link with traffic); observe the reception of packet traffic on Tester port B. Expected result 2. - Restore the link and observe the received traffic of Tester port B. The expected result is 2. |  |
| ☐ Passed | ☐ Failed |

- Basic BGPv6

| Test Item Name: basic BGPv6 |  |
| --- | --- |
| Work Hour: 10 mins |  |
| Test Topology: |  |
| Test Process: 1. Enable the IPv6 forwarding function on Router A and Router B. 2. Configure IPv6 global unicast addresses for the interfaces. |  |
| Expect Results: - Router A and Router B can ping ipv6 address each other. |  |
| Actual Result： - Enable the IPv6 forwarding function on Router A and Router B. Router A and Router B can ping ipv6 address each other. |  |
| ☐ Passed | ☐ Failed |

- BGP neighbor number

| Test Item Name: Maximum number of neighbors per BGP process. |  |
| --- | --- |
| Work Hour: 40 mins |  |
| Test Topology: |  |
| Test Process: 1. Verify DUT declared bgp neighbor number. 2. Establish EBGP neighbor relationship between DUT and TC1 using sub-interface. The sub-interface number is the same as declared neighbor amount. 3. Every BGP neighbor advertised 10 routes and check bgp routing on DUT. |  |
| Expect Results: - DUT can establish all declared BGP neighbor relationship. - DUT can learn all advertised BGP routing. |  |
| Actual Result： Assign sub-interface with ip address The sub-interface capacity is 1024 The vlan number on DUT is 512 To tester side，create at least 1024 sets device Check bgp routing table |  |
| ☐ Passed | ☐ Failed |

- L2TP

| Test Item Name: L2TP |  |
| --- | --- |
| Work Hour: 25 mins |  |
| Test Topology: |  |
| Test Process: 1. Create L2TP group in LAC mode. 2. Configure the local tunnel name , and specify the IP address of the tunnel peer (LNS) 3. Enable tunnel authentication, and configure the authentication key. 4. Create Virtual-PPP 2. 5. Trigger the LAC to establish an L2TP tunnel with the LNS. 6. Configure local user on LNS and set the authentication domain. 7. Configure ip pool and virtual template interface to allocate ip address for ppp user 8.config l2tp tunnel on LNS |  |
| Expect Results: 1. The L2TP session is established successfully. 2. The L2TP tunnel is established successfully. |  |
| Actual Result： LAC Configure L2tp group and set local tunnel name ,and peer ip adress Use l2tp auto client function to trigger lac to start request to establish l2tp tunnel. Let virtual-ppp interface can get ip address(192.168.0.10) from lns Configure private network routing to make any request to visit corporate can be forwarded by virtual ppp interface. display l2tp session on LAC display l2tp session on LNS Check virtual-ppp interface LNS Configure local-user and set user password Using local authentication to verify ppp user Using ip pool aaa to assign ip address for ppp user and virtual ppp interface on LAC. Config private route used to let L2TP tunnel to forward traffic visiting ppp user. [LNS] ip route-static 10.2.0.0 16 192.168.0.2 Configure tunnel local side is named lns and the opposite end is LAC and let lns and lac shared the same tunnel key. |  |
| ☐ Passed | ☐ Failed |

- ACL

| Test Item Name: ACL layer3 |  |
| --- | --- |
| Work Hour: 15 mins |  |
| Test Topology: |  |
| Test Process: 1. Make the DUT work normally refer to the topology. 2. Configure the advanced ACL and applied this ACL to one port. 3. Telnet to the DUT from 810-1 to check ACL can block the telnet service. 4. Remove ACL configuration on port gi0/4 and telnet to switch through 810-2. 5. Add rule 5 in acl 3000 to deny http service. |  |
| Expect Results: - When step 3 is done，the telnet service will be failed. - When step 4 is done, the telnet service can succeed. - When step 5 is done, the http service is failed, too. Also, when this rule is get rid of, user can visit http service. |  |
| Actual Result： Basically，810-1 can ping switch used to simulate pc/host through 810-2 (DUT) Here static route is configured on 810-1 and switch to make network accessibility. When the acl is applied to deny telnet TCP connection. Cause packet filter work normally，so Telnet is failed. When acl 3000 (packet filter) is removed from gi0/4 port which is connected to DUT 810-1,810-1 can Telnet to PC/host simulated by switch Using acl to deny http packet Set pc network card with ip add 192.168.1.2 Add route to make PC can access to switch through DUT and then visit switch web GUI by browser. When the acl is removed from port connected to PC The web GUI for switch can be accessed. |  |
| ☐ Passed | ☐ Failed |

- FORWARDING CAPACITY

| Test Item Name: Validate basic forwarding capacity of device |  |
| --- | --- |
| Work Hour: 35 mins |  |
| Test Topology: |  |
| Test Process: 1. Configure IP address on Port1 and Port2 of DUT 2. Send streams from port1/3 to port1/9 of tester, expected result 3. Set stream packet size to 1500 byte 4. Change packet forwarding speed step by step to find maximum value without packet loss. |  |
| Expect Results: 1. port1/9 of tester can receive completed streams with maximum speed stream |  |
| Actual Result： Config IP address on DUT，and later this ip address is used to create traffic flow. Set up traffic flow with packet size 1500 bytes, source ip address varied from 172.168.20.2 to 172.168.23.233 ，the ip address number is totally 1000. The result preview Creating traffic flow on port 2 to ，the traffic flow destination ip address is 172.168.20.2-172.168.23.233. so the 1000 flow can be established. Start the traffic and display CPU（50Mbps/bidirectional/variable source） Display memory（50Mbps/bidirectional/variable source） Start the traffic and display CPU（100Mbps/bidirectional/variable source） Display memory（100Mbps/bidirectional/variable source） Start the traffic and display CPU（500Mbps/bidirectional/variable source），packet loss occurred. set traffic forwarding speed to 470Mbps and no packet loss And check cpu-usage and memory So the DUT forwarding capacity is 470Mbps with bidirectional/variable source traffic. |  |
| ☐ Passed | ☐ Failed |

- NTP

| Test Item Name: NTP |  |
| --- | --- |
| Work Hour：20 min |  |
| Test Topology: |  |
| Test Process: - Enable and configure NTP feature on the DUT, and gets the expect result 1. |  |
| Expect Results: - The DUT must synchronize the date/time with 2 NTP server |  |
| Actual Result： - Enable and configure NTP feature on the DUT NTP Client: - The DUT1 synchronize the date/time with DUT2 |  |
| ☐ Passed | ☐ Failed |

- Network Bridge

| Test Item Name: NTP |  |
| --- | --- |
| Work Hour：20 min |  |
| Test Topology: |  |
| Test Process: - Configure Port 1&2 as bridge mode - Create host or use PC on Port A&B - Perform PING command with expected result 1 |  |
| Expect Results: - Both 2 host can be PING successfully |  |
| Actual Result： - Configuration on Port 1&2 # interface GigabitEthernet0/1 port link-mode bridge # interface GigabitEthernet0/2 port link-mode bridge # - Create host on Port A&B - Start ARP learning and the state will change to Resolved and enable respond to PING - Both 2 host can be PING successfully |  |
| ☐ Passed | ☐ Failed |

- Static IPv6

| Test Item Name: Static IPv6 |  |
| --- | --- |
| Work Hour：20 min |  |
| Test Topology: |  |
| Test Process: - Configure IPv6 address for all DUT - PING DUT3 from DUT1 with expected result 1 - Configure static route DUT1 - PING DUT3 from DUT1 with expected result 2 |  |
| Expect Results: - DUT3 can’t be ping from DUT1 - DUT3 can be ping from DUT1 |  |
| Actual Result： - Configuration for Ports DUT1: # interface GigabitEthernet0/3 port link-mode route ipv6 address 100::1/64 # DUT2: # interface GigabitEthernet0/1 port link-mode route ipv6 address 100::2/64 # interface GigabitEthernet0/2 port link-mode route ipv6 address 200::1/64 # DUT3: # interface GigabitEthernet0/3 port link-mode route ipv6 address 200::2/64 # - Ping DUT3 from DUT1 - Add static routes on DUT [810-1]ipv6 route-static 200:: 64 100::2 [810-2]ipv6 route-static 100:: 64 200::1 - Ping DUT3 from DUT1 |  |
| ☐ Passed | ☐ Failed |

- Priority

| Test Item Name: Priority |  |
| --- | --- |
| Work Hour：40 min |  |
| Test Topology: |  |
| Test Process: - Configure CBQ on DUT to meet the following requirements: - Packets with source IP address 10.0.0.10 are assigned to an SP queue. - Packets with source IP address 10.0.0.11 are assigned to an EF queue, with a maximum bandwidth of 10 Mbps. - Packets with source IP address 10.0.0.12 are assigned to an AF queue, with a minimum guaranteed bandwidth of 10 Mbps. - Packets with source IP address 10.0.0.13 are assigned to a BE queue, with no guaranteed bandwidth. - View the packet loss ratio of each flow on the receiver. - Configure a rate limit of 25 Mbps on the output interface on Router 1, result 1 is expected. - Configure a rate limit of 100 Mbps on the output interface on Router 1, result 2 is expected. - Send the following four flows to DUT from the tester connected to it: Flow ID; Source IP; CBQ queue; Reserved bandwidth; Traffic rate. 1; 10.0.0.10; SP; 10 Mbps; 10 Mbps. 2; 10.0.0.11; EF; 10 Mbps; 10 Mbps. 3; 10.0.0.12; AF; 10 Mbps; 10 Mbps. 4; 10.0.0.13; BE; None; 60 Mbps. - View the result of each flow on the receiver with expected result 1 |
| Expect Results: - The packet loss ratios of flows 1 through 2 are all 0%, and the packet loss ratio of flow 3 is nearly 50%,and the packet loss ratio of flow 4 is nearly 100% - The packet loss ratios of flows 1 through 4 are all 0%. |  |
| Actual Result： - Configuration: # acl advanced 3000 rule 0 permit ip source 10.0.0.10 0 # acl advanced 3001 rule 0 permit ip source 10.0.0.11 0 # acl advanced 3002 rule 0 permit ip source 10.0.0.12 0 # traffic classifier 1 operator and if-match acl 3000 # traffic classifier 2 operator and if-match acl 3001 # traffic classifier 3 operator and if-match acl 3002 # traffic behavior 1 queue sp # traffic behavior 2 queue ef bandwidth 10000 # traffic behavior 3 queue af bandwidth 10000 # qos policy 1 classifier 1 behavior 1 classifier 2 behavior 2 classifier 3 behavior 3 # interface Ethernet0/1 port link-mode route description connect with portA ip address 10.0.0.1 255.255.255.0 # interface Ethernet0/2 port link-mode route description connect with portB ip address 20.0.0.1 255.255.255.0 qos lr outbound cir 25000 qos apply policy 1 outbound # - Create traffic flows on Tester Port A Set packet size to 1518 Edit the traffic flow: Stream 1- Stream 2- Stream 3- Stream 4- Change the traffic speed: - Start all the traffic and get result 1 - Delete the speed limit on Port 2 [H3C-GigabitEthernet0/2]undo qos lr outbound - Start all the traffic and get result 2 |  |
| ☐ Passed | ☐ Failed |

- IPsec

| Test Item Name: IPsec |  |
| --- | --- |
| Work Hour：20 min |  |
| Test Topology: |  |
| Test Process: - Establish an IPsec tunnel between Router A and Router B to protect data flows between subnet 10.1.1.0/24 and subnet 10.1.2.0/24. Configure the IPsec tunnel as follows: - Specify the encapsulation mode as tunnel, the security protocol as ESP, the encryption algorithm as 128-bit AES, and the authentication algorithm as SHA1. - Set up SAs through IKE negotiation. |  |
| Expect Results: - Initiate a connection from subnet 10.1.1.0/24 to subnet 10.1.2.0/24 to trigger IKE negotiation. After IPsec SAs are successfully negotiated by IKE, the traffic between the two subnets is IPsec-protected |  |
| Actual Result： - Configuration DUT1: # acl advanced 3101 rule 0 permit ip source 10.1.1.0 0.0.0.255 destination 10.1.2.0 0.0.0.255 # ipsec transform-set tran1 encapsulation-mode tunnel protocol esp esp encryption-algorithm aes-cbc-128 esp authentication-algorithm sha1 # ipsec policy map1 10 manual transform-set tran1 security acl 3101 remote-address 2.2.3.1 sa spi inbound esp 54321 sa string-key inbound esp simple gfedcba sa spi outbound esp 12345 sa string-key outbound esp simple abcdefg # interface GigabitEthernet0/1 port link-mode route ip address 10.1.1.1 255.255.255.0 # interface GigabitEthernet0/3 port link-mode route ip address 2.2.2.1 255.255.255.0 ipsec apply policy map1 # ip route-static 10.1.2.0 24 GigabitEthernet0/3 2.2.2.3 # DUT2: # acl advanced 3101 rule 0 permit ip source 10.1.2.0 0.0.0.255 destination 10.1.1.0 0.0.0.255 # ipsec transform-set tran1 encapsulation-mode tunnel protocol esp esp encryption-algorithm aes-cbc-128 esp authentication-algorithm sha1 # ipsec policy use1 10 manual transform-set tran1 security acl 3101 remote-address 2.2.2.1 sa spi inbound esp 12345 sa string-key inbound esp simple abcdefg sa spi outbound esp 54321 sa string-key outbound esp simple gfedcba # interface GigabitEthernet0/1 port link-mode route ip address 10.1.2.1 255.255.255.0 # interface GigabitEthernet0/3 port link-mode route ip address 2.2.3.1 255.255.255.0 ipsec apply policy use1 # ip route-static 10.1.1.0 24 GigabitEthernet0/3 2.2.3.3 # - Check the IPsec sa on DUT - Create host on Tester and configure IP address - Send bi-direction traffic and there is no traffic loss |  |
| ☐ Passed | ☐ Failed |

- GRE

| Test Item Name: GRE |  |
| --- | --- |
| Work Hour：20 min |  |
| Test Topology: |  |
| Test Process: - Create a GRE tunnel interface, and specify the tunnel mode as GRE/IPv4. - Configure an IP address for the tunnel interface based on the passenger protocol. - Configure a source address or source interface for the tunnel interface. - Configure a destination address for the tunnel interface. - Configure a static route through the tunnel. |  |
| Expect Results: - The gre tunnel is established successfully. - The private network can ping each other. |  |
| Actual Result： - Configuration: DUT1: # ospf 1 area 0.0.0.0 network 2.2.0.0 0.0.255.255 # interface GigabitEthernet0/1 port link-mode route ip address 10.1.1.1 255.255.255.0 # interface GigabitEthernet0/3 port link-mode route ip address 2.2.2.1 255.255.255.0 # interface Tunnel0 mode gre ip address 10.1.2.1 255.255.255.0 source 2.2.2.1 destination 2.2.3.1 # ip route-static 10.1.3.0 24 Tunnel0 # DUT2: # ospf 1 area 0.0.0.0 network 2.2.0.0 0.0.255.255 # interface GigabitEthernet0/1 port link-mode route ip address 10.1.3.1 255.255.255.0 # interface GigabitEthernet0/3 port link-mode route ip address 2.2.3.1 255.255.255.0 # interface Tunnel0 mode gre ip address 10.1.2.2 255.255.255.0 source 2.2.3.1 destination 2.2.2.1 # ip route-static 10.1.1.0 24 Tunnel0 # Internet: # ospf 1 area 0.0.0.0 network 2.2.0.0 0.0.255.255 # interface GigabitEthernet1/0/11 port link-mode route ip address 2.2.2.3 255.255.255.0 # interface GigabitEthernet1/0/12 port link-mode route ip address 2.2.3.3 255.255.255.0 # - Create host on Tester and configrue IP address - Send traffic and there is no traffic loss |  |
| ☐ Passed | ☐ Failed |