H3C F1000-AI-10 Product

Test Procedures

H3C Technologies Co., Ltd.

0618 2021

Contents

1.	Test Description	4

1.1	Project information	4

1.2	Test Equipment	4

1.3	Test Support Tools and Accessories	4

2.	Checklist of Testing	5

3.	Methods of Testing Item	6

3.1	SSH secure login	6

3.2	HTTP and HTTPS access	9

3.3	SNMP	18

3.4	NTP	28

3.5	Syslog	33

3.6	DHCP Configuration (OPTION 49 and OPTION 26)	38

3.7	HTTP/HTTPS connection idle timeout	46

3.8	Access control based on source IPv4 addresses, destination IPv4 addresses, and time ranges	49

3.9	Access control based on source IPv6 addresses and destination IPv6 addresses	59

3.10	MAC Binding and Limiting	65

3.11	Access control based on MAC addresses	70

3.12	Access control based on user identity	76

3.13	NAT（PAT）	84

3.14	NAT（NO-PAT）	93

3.15	NAT server	102

3.16	NAT ALG	108

3.17	NAT hairpin	122

3.18	NAT 64（AFT）	131

3.19	NAT 66	142

3.20	Global NAT (source address translation)	148

3.21	IRF HA	155

3.22	Virtual firewall - independent management and independent services	159

3.23	IPS	165

3.24	Signature library update	175

3.25	HTTP-based anti-virus	182

3.26	URL filtering by category	192

3.27	Application auditing	204

3.28	Bandwidth limiting based on total bandwidth and shared mode	215

3.29	IPsec transport encapsulation mode with ESP	225

3.30	IPsec transport encapsulation mode with AH	237

3.31	IPsec SVTI tunnel mode	247

3.32	SYN flood attack defense	256

3.33	ARP inspection	267

3.34	RADIUS Authentication	275

3.35	TACACS Authentication	290

Appendix 1. Testing Protocol	305

- Test Description

  - Project information

| Project Name |  |
| --- | --- |
| Customer |  |
| Testing Date |  |
| Testing Site |  |
| Participants |  |

  - Test Equipment

| Manufacturer, Country |  |
| --- | --- |
| Model |  |
| Product Number |  |
| Serial Number |  |
| Software Version |  |

  - Test Support Tools and Accessories

| Item | Qty. | Provider | Description |
| --- | --- | --- | --- |

- Checklist of Testing

| S.No | Test Item | Result Yes/No |
| --- | --- | --- |
| 3.1 | SSH secure login |  |
| 3.2 | HTTP and HTTPS access |  |
| 3.3 | SNMP |  |
| 3.4 | NTP |  |
| 3.5 | Syslog |  |
| 3.6 | DHCP Configuration (OPTION 49 and OPTION 26) |  |
| 3.7 | HTTP/HTTPS connection idle timeout |  |
| 3.8 | Access control based on source IPv4 addresses, destination IPv4 addresses, and time ranges |  |
| 3.9 | Access control based on source IPv6 addresses and destination IPv6 addresses |  |
| 3.10 | MAC Binding and Limiting |  |
| 3.11 | Access control based on MAC addresses |  |
| 3.12 | Access control based on user identity |  |
| 3.13 | NAT (PAT) |  |
| 3.14 | NAT (NO-PAT) |  |
| 3.15 | NAT server |  |
| 3.16 | NAT ALG |  |
| 3.17 | NAT hairpin |  |
| 3.18 | NAT 64 (AFT) |  |
| 3.19 | NAT 66 |  |
| 3.20 | Global NAT (source address translation) |  |
| 3.21 | IRF HA |  |
| 3.22 | Virtual firewall - independent management and independent services |  |
| 3.23 | IPS |  |
| 3.24 | Signature library update |  |
| 3.25 | HTTP-based anti-virus |  |
| 3.26 | URL filtering by category |  |
| 3.27 | Application auditing |  |
| 3.28 | Bandwidth limiting based on total bandwidth and shared mode |  |
| 3.29 | IPsec transport encapsulation mode with ESP |  |
| 3.30 | IPsec transport encapsulation mode with AH |  |
| 3.31 | IPsec SVTI tunnel mode |  |
| 3.32 | SYN flood attack defense |  |
| 3.33 | ARP inspection |  |
| 3.34 | RADIUS Authentication |  |
| 3.35 | TACACS Authentication |  |

- Methods of Testing Item

  - SSH secure login

| Test Item Name: SSH secure login |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that the device supports SSH login. |  |
| Test Topology: |  |
| Test Process: - Use the PC as an SSH client and the Firewall device as an SSH server. - On the Firewall device, configure the user login authentication mode as scheme. - On the Firewall device, configure a user named user-ssh. Configure the user's login method as SSH, password as h3c.com123, and service type as SSH. Enable the SSH service. - On the PC, run the SSH client software. Use username user-ssh and password h3c.com123 to log in to the FW device. Result 1 is expected. |  |
| Expect Results: - The user user-ssh can successfully log in to the device through SSH. |  |
| Actual Result： Configuration on web: - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interface. Edit interface GE1/0/0. Configure an IP address and security zones for the interface. Configuration on CLI: - Configure the user authentication mode as scheme. - Configure user user-ssh. - Enable SSH. - On the PC, run the SSH client software. Use username user-ssh and password h3c.com123 to log in to the Firewall device. |  |
| ☐ Passed | ☐ Failed |

  - HTTP and HTTPS access

| Test Item Name: HTTP and HTTPS access |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that specified users can access the firewall through HTTP and HTTPS. |  |
| Test Topology: |  |
| Test Process: - Use the PC as the client to access the device from the Web interface. - Assign an IP address to GE 1/4/1. - Add G1/4/1 to security zone Trust. - Create security policy 1. Set security zone to Local and Trust, destination zone to Local and Trust, and action to Permit. - Configure an ACL on the device to allow access only from IP address 1.1.1.2. - Specify the ACL as the HTTP and HTTPs login control ACL. - Use IP address 1.1.1.2 to access the device through HTTP. Result 1 is expected. - Use IP address 1.1.1.2 to access the device through HTTPS. Result 1 is expected. - Use an IP address other than 1.1.1.2 to access the device through HTTP. Result 2 is expected. - Use an IP address other than 1.1.1.2 to access the device through HTTPs. Result 2 is expected. |  |
| Expect Results: - The login succeeds. - The login fails. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for GE1/4/1. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Local zone. - Configure an ACL on the device. Click Objects > ACL > IPv4 ACL. Create ACL 2001. Create a rule to permit traffic only from IP address 1.1.1.2 for ACL 2001. - Click Network > Services > HTTP/HTTPS. Specify ACL 2001 as the HTTP and HTTPS login control ACL. - Use IP address 1.1.1.2 to access the device through HTTP. Result 1 is expected. The login succeeds. - Use IP address 1.1.1.2 to access the device through HTTPS. Result 1 is expected. The login succeeds. - Use an IP address other than 1.1.1.2 to access the device through HTTP. Result 2 is expected. Change the IP address of PC: The login fails. - Use an IP address other than 1.1.1.2 to access the device through HTTPs. Result 2 is expected. The login fails. |  |
| ☐ Passed | ☐ Failed |

  - SNMP

| Test Item Name: SNMP |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that SNMP functions correctly. |  |
| Test Topology: |  |
| Test Process: - Assign IP address 1.1.1.1/24 to G1/4/1 on the device. - Add G1/4/1 to security zone Trust. - Create security policy 1. Set security zone to Local and Trust, destination zone to Local and Trust, and action to Permit. - Enable SNMPv2 on the device. Create read-only community name public and read-write community name private. - Use a MIB browser to connect the device through SNMPv2 by using read-only community name public and read-write community name private. Result 1 is expected. Use the MIB browser to view MIB objects on the device. Result 2 is expected. |  |
| Expect Results: - The MIB Browser connects to the device through SNMPv2 successfully. - The MIB objects of the device can be viewed from the MIB browser. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Local zone. - Configure SNMPv2 settings on the device. Click System > Maintenance > SNMP. Then click SNMP. In the Global Configuration area, select SNMPv2c and then click Apply. In the Configure Version area, set Read community to public and Write community to private and then click Apply. - On the MIB browser of the PC, use SNMPv2c to access the Firewall and request the value of MIB object sysObjectID. The MIB browser can obtain the value of sysObjectID through SNMPv2c successfully. Start the MIB Browser software on PC and set the ip address of remote SNMP agent to 1.1.1.1. Use ping to verify the connection. Select View - SNMP Protocol Preferences to change the preferences of the software. Select the SNMPv2c, set the read community as public and write community as private, and set port number as 161, then click OK. SNMPv2c connect successfully. Right-click the iso and select the Find. Find the sysObjectID ; Right-click the project and select Get; The MIB browser can obtain the value of sysObjectID through SNMPv2c successfully. |  |
| ☐ Passed | ☐ Failed |

  - NTP

| Test Item Name: NTP |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that NTP functions correctly. |  |
| Test Topology: |  |
| Test Process: - Set the IP address of interface G1/4/1 of firewall to 1.1.1.1. Add the interface to the Trust zone. Configure the security policy to allow the access of traffic prior to the Trust and Local zones. - Set the IP address of interface G1/0/1 of switch to 1.1.1.2. - Enable the NTP service on firewall. Set the local clock as the reference clock. Set the firewall to work on L2. - Enable the NTP service on switch. Set the firewall to obtain clock through NTP. Set firewall as the NTP server of switch. Result 1 and 2 is expected. |  |
| Expect Results: - The firewall supports NTP and NTP settings can be configured on it. The firewall can synchronize time with other devices through NTP. - The NTP session information is displayed on the device. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Local zone. - Enable the NTP service on firewall. Set the local clock as the reference clock. Set the firewall to work on L2. - Enable the NTP service on switch. Set the switch to obtain clock through NTP. Set firewall as the NTP server of switch. Set the IP address for switch. Set NTP: - Switch synchronizes time with firewall. After time synchronization, you can view the NTP status of switch. The status shows that switch has synchronized the time with firewall, and switch (L3) is one layer higher than firewall. - The query result of all IPv4 session information of switch's NTP service shows that a session has established between switch and firewall. - Click Monitor > Sessions on the webpage of firewall, you can see the NTP session on firewall |  |
| ☐ Passed | ☐ Failed |

  - Syslog

| Test Item Name: Syslog |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that syslog functions correctly. |  |
| Test Topology: |  |
| Test Process: - Assign IP address 1.1.1.1/24 to G1/4/1 on the device. - Add G1/4/1 to security zone Trust. - Create security policy 1. Set Security zone to Local and Trust, Destination zone to Local and Trust, and Action to Permit. - Configure syslog server settings on the server. - Configure and enable syslog server on the PC. Check whether the server receives log messages from the firewall. |  |
| Expect Results: - The syslog server has received log messages sent from the firewall. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Local zone. - Click System > Log Settings > Basic Settings. Configure the log host information on firewall - View syslog through 3CD. Shutdown the other interface, the relevant syslogs are generated in the list. The 3CD has received log messages sent from the firewall. |  |
| ☐ Passed | ☐ Failed |

  - DHCP Configuration (OPTION 49 and OPTION 26)

| Test Item Name: DHCP Configuration (OPTION 49 and OPTION 26) |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that the DHCP server supports the Option field. |  |
| Test Topology: |  |
| Test Process: - Add interface G1/4/1 of the firewall to the Trust zone and configure the IP address. - Create security policy 1. Set security zone to Local and Trust, destination zone to Local and Trust, and action to Permit. - Configure the DHCP function. Set the address pool to 100.1.1.0/24. Disable IP address 100.1.1.1. - Configure DHCP Option 26 in hexadecimal format with the option content set to 1020. Configure DHCP Option 49 in IP address format with the option content set to 20.1.1.2. - Enable the packet capture function on the PC. Adopt the automatic IP acquisition mode. Expected result 1 is obtained. |  |
| Expect Results: - The IP address of the address pool can be obtained on the PC. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Local zone. - Enable the DHCP function and configure the address pool. Click the Network > DHCP > DHCP Service. Select Enable for DHCP service. - Configure the DHCP address pool. Click the Network > DHCP > DHCP Address Pools. Click Create address pool. Set Address pool name to 1 and Subnet for dynamic allocation to 1.1.1.0/24. Then click OK. - Configure available Option fields. Click the Network > DHCP > DHCP Address Pools. Click the Address Pool Options tab. a) Configure DHCP Option 26 in hexadecimal format with the option content set to 1020. b) Configure DHCP Option 49 in IP address format with the option content set to 20.1.1.2. Click OK to save the configuration. - Enable the packet capture function on the PC and set the NIC to automatically obtain the IP address. The IP address of the address pool can be obtained on the PC. Based on the packets captured on the PC, it shows that the DHCP ACK packet sent by the DHCP server contains Options 26 and 49, and the option content is consistent with that set on the firewall. |  |
| ☐ Passed | ☐ Failed |

  - HTTP/HTTPS connection idle timeout

| Test Item Name: HTTP/HTTPS connection idle timeout |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that HTTP/HTTPS connection idle timeout functions correctly. |  |
| Test Topology: |  |
| Test Process: - Enable HTTP/HTTPS on the device. Configure both the username and password as admin and set the connection idle timeout to 1 minutes. - Access the firewall from the PC through HTTP/HTTPS. - Do not perform any operations for 1 minutes. Result 1 is expected. |  |
| Expect Results: - After 1 minute of no operation, HTTP / HTTPS is disconnected. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interface. Edit interface GE1/0/0. Configure an IP address and security zones for the interface. - Click Network > Services > HTTP/HTTPS. Enable HTTP/HTTPS and set the connection idle timeout to 1 minutes and then click Apply. - Use the IP address of 192.168.206.241 to log in to the firewall. Do nothing for a minute, the webpage is disconnected. |  |
| ☐ Passed | ☐ Failed |

  - Access control based on source IPv4 addresses, destination IPv4 addresses, and time ranges

| Test Item Name: Access control based on source IPv4 addresses, destination IPv4 addresses, and time ranges |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that access control can be performed based on source IPv4 addresses, destination IPv4 addresses, and time ranges. |  |
| Test Topology: |  |
| Test Process: - Create a time period named work from 09:00 to 17:00 on weekdays; - Configure the IP addresses of GE1/4/2 and GE1/4/1; - Add GE1/4/2 to the Trust zone, and GE1/4/1 to Untrust; - PC accesses network resources in the Internet, and the expected result is 1; - Create IPv4 address object group s1, and create address object 1.1.1.2 in s1; - Create IPv4 address object group d1, and create address object 10.1.1.2 in d1; - Create an IPv4 object policy. The rule allows packets whose source ip is IPv4 address object group s1 and destination address object group d1 to pass; - Configure the security policy 1 from the Trust zone to the Untrust zone; - PC ping 10.1.1.2 the expected result is 2; - Change the IPv4 object policy so that the packets whose source ip is IPv4 address object group s1 and destination address object group d1 are allowed to pass when the time period is work, and cannot access 10.1.1.2 at other times; - PC accesses the network resources in the Internet again from 9:00 to 17:00 on working days, and there is expected result 3; - PC accesses network resources on the Internet again in other time periods, and the expected result is 4. |  |
| Expect Results: - When the above configuration is completed, the expected result 1 is obtained without adding security policy. PC can't ping Internet. - After adding the security policy from trust to untrust. PC can ping Internet. - PC can ping Internet. - PC can't ping Internet. |  |
| Actual Result： - Click Objects > Object Groups > Time Ranges Create a time period named work from 09:00 to 17:00 on weekdays; - Configure the interface IP address and add the interface to the security zone. Click the Network > Interface Configuration > Interfaces. Set the IPv4 address of interface GE1/4/1 to 1.1.1.1 and add the interface to the Trust zone. Set the IPv4 address of interface GE1/4/2 to 10.1.1.1 and add the interface to the Untrust zone. - Ping 10.1.1.2 from PC, and the expected result is 1; - Create IPv4 address object group Click the Objects > Object Groups > IPv4 Address Object Groups. - Create a security policy. Click the Policies > Security Policies > Security Policies. Set the Action in the IPv4 security policy for the access from the Trust zone to the Untrust zone to Permit. And configure source IP/MAC address to s1 and destination IP to d1. - Ping 10.1.1.2 from host1, and the expected result is 2; - Change the time range of security policy. Click the Policies > Security Policies > Security Policies. Change the time range of security policy to work. - Ping 10.1.1.2 from PC from 9:00 to 17:00 on weekdays, PC can ping Internet. - Ping 10.1.1.2 from PC in other time periods , PC can't ping Internet. Change the time range |  |
| ☐ Passed | ☐ Failed |

  - Access control based on source IPv6 addresses and destination IPv6 addresses

| Test Item Name: Access control based on source IPv6 addresses and destination IPv6 addresses |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that access control can be performed based on source and destination IPv6 addresses. |  |
| Test Topology: |  |
| Test Process: - Assign IPv6 addresses to GE1/4/1 and GE1/4/2. - Assign GE1/4/1 to the Trust security zone, and assign GE1/4/2 to the Untrust security zone. - Access Internet from PC. Result 1 is expected. - Create an IPv6 address object group named src, and configure IPv6 address object 192::/64. - Create an IPv6 address object group named des, and configure IPv4 address object 218::/64. - Configure a security policy with the source security zone as src and the destination security zone as des. - Access Internet from PC. Result 2 is expected. |  |
| Expect Results: - PC fails to access Internet. - PC can access Internet. |  |
| Actual Result： - Configure the interface IP address and add the interface to the security zone. Click the Network > Interface Configuration > Interfaces. Set the IPv4 address of interface GE1/4/1 to 192::1 and add the interface to the Trust zone. Set the IPv4 address of interface GE1/4/2 to 218::1 and add the interface to the Untrust zone. - Ping 218::2 from PC, and the expected result is 1; PC fails to access Internet. - Create IPv6 address object group Click the Objects > Object Groups > IPv6 Address Object Groups. - Create a security policy. Click the Policies > Security Policies > Security Policies. Set the Action in the IPv4 security policy for the access from the Trust zone to the Untrust zone to Permit. And configure source IP/MAC address to s1 and destination IP to d1. - Ping 10.1.1.2 from host1, and the expected result is 2; PC can access Internet. |  |
| ☐ Passed | ☐ Failed |

  - MAC Binding and Limiting

| Test Item Name: MAC Binding and Limiting |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that the basic IP-MAC binding functions correctly. |  |
| Test Topology: |  |
| Test Process: - Configure the interface address, security zone, and security policy. Allow basic traffic to pass through the firewall. - Configure the IP-MAC binding for PC. Configure a correct MAC address. Set the default action to Permit. Expected result 1 is obtained. - Configure an incorrect MAC address for PC. Expected result 2 is obtained. |  |
| Expect Results: - The firewall can access PC. - The firewall cannot access PC. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Local zone. - Configure IP-MAC binding. Click the Network > ARP. Set interface GE1/4/1 to the member List. Click Create to add IP-MAC binding. - Ping 1.1.1.2 on firewall, result 1 is expected. The firewall can access PC. - Configure an incorrect MAC address. Click Edit to change IP-MAC binding information - Ping 1.1.1.2 on Firewall, result 2 is expected. The firewall cannot access PC. |  |
| ☐ Passed | ☐ Failed |

  - Access control based on MAC addresses

| Test Item Name: Access control based on MAC addresses |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that access control can be performed based on MAC addresses. |  |
| Test Topology: |  |
| Test Process: - Assign IPv4 addresses to GE1/4/1 and GE1/4/2. - Assign GE1/4/1 to the Trust security zone, and assign GE1/4/2 to the Untrust security zone. - Create a MAC address object group named 1, and configure address object 00-2B-67-56-68-0B. - Create an IPv4 security policy: The source security zone is Trust, the destination security zone is Untrust, the source address object group is 1, and the action is permit. - Ping PC 2 from PC 1. Result 1 is expected. - Modify the action of the IPv4 security policy to deny. - Ping PC 2 from PC 1. Result 2 is expected. |  |
| Expect Results: - The ping operation succeeds. - The ping operation fails. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv4 address of interface GE1/4/2 to 10.1.1.1 and add the interface to the Untrust zone. - Click Objects > Object Groups > MAC Address Object Group. Create a MAC address object group named 1, and configure address object 00-2B-67-56-68-0B. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Untrust zone. - Ping PC 2 from PC 1. Result 1 is expected. The ping operation succeeds. - Click Policies > Security Policies > Security Policies. Modify the action of the IPv4 security policy to deny. - Ping PC 2 from PC 1. Result 2 is expected. The ping operation fails. |  |
| ☐ Passed | ☐ Failed |

  - Access control based on user identity

| Test Item Name: Access control based on user identity |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that access control can be performed based on user identity. |  |
| Test Topology: |  |
| Test Process: - Create a periodic time range named work during 9:00 am and 11:00 am on working days. - Assign IPv4 addresses to GE1/4/1 and GE1/4/2. - Assign GE1/4/1 to the Trust security zone, and assign GE1/4/2 to the Untrust security zone. - Create user usera, and bind the host to the user. - Create a zone pair with the source security zone as Trust and the destination security zone as Untrust, and apply object policy ippolicy1. - Create an IPv4 security policy: The source security zone is Trust, the destination security zone is Untrust, the action is permit and apply the usera. - Access the Internet from PC during a time period other than time range work. Result 1 is expected. - Access the Internet from PC during the time range work. Result 2 is expected |  |
| Expect Results: - During a time period other than time range work, the PC cannot access the Internet. - During time range work, the PC can successfully access the Internet. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv4 address of interface GE1/4/2 to 10.1.1.1 and add the interface to the Untrust zone. - Click Objects > Object Groups > Time Ranges Create a time period named work from 09:00 to 11:00 on weekdays; - Click Objects > User > User Management > Local Users Create a new local user name usera. - Click Objects > User > User Management > Online Users Configure a static identity user usera and enable the user identification function. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Untrust zone and apply the usera. - Access the Internet from PC during a time period other than time range work. Result 1 is expected. The PC cannot access the Internet. - Access the Internet from PC during the time range work. Result 2 is expected. The PC can successfully access the Internet. |  |
| ☐ Passed | ☐ Failed |

  - NAT（PAT）

| Test Item Name: NAT（PAT） |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that NAT functions correctly. |  |
| Test Topology: |  |
| Test Process: - Add GE1/4/1, the interface that connects to PC1 on the private network, to the zone Trust, add GE1/4/2, the interface that connects to PC2 on the public network, to the zone Untrust, and configure a security policy to permit packets from Trust to Untrust. - Configure an ACL to identify the target traffic on the device. - Configure a NAT address group on the device. - Configure outbound NAT on GE1/4/2. - Ping PC 2 from PC 1. Result 1 is expected. |  |
| Expect Results: - The ping succeeds. A NAT session has been generated. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv4 address of interface GE1/4/1 to 1.1.1.1 and add the interface to the Trust zone. Set the IPv4 address of interface GE1/4/2 to 10.1.1.1 and add the interface to the Untrust zone. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Untrust zone. - Click Objects > ACL > IPv4 ACL. Click Create. Configure an ACL and the ACL rule. - Click Policies > NAT > NAT Advanced Settings > NAT Address Groups. Click Create. Configure a NAT address group. - Select Policies > NAT > Dynamic NAT. Click the Outbound Dynamic NAT (ACL-Based) tab, and then click Create. Configure an outbound NAT rule. - Ping PC 2 from PC 1. Result 1 is expected. The ping succeeds. A NAT session has been generated. |  |
| ☐ Passed | ☐ Failed |

  - NAT（NO-PAT）

| Test Item Name: NAT（NO-PAT） |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that NAT（NO-PAT） functions correctly. |  |
| Test Topology: |  |
| Test Process: - Add GE1/4/1, the interface that connects to PC1 on the private network, to the zone Trust, add GE1/4/2, the interface that connects to PC2 on the public network, to the zone Untrust, and configure a security policy to permit packets from Trust to Untrust. - Configure an ACL to identify the target traffic on the device. - Configure a NAT address group on the device. - Configure NAT with NO-PAT on GE1/4/2. - Ping PC 2 from PC 1. Result 1 is expected. |  |
| Expect Results: - The ping succeeds. A NAT session has been generated. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv4 address of interface GE1/4/1 to 1.1.1.1 and add the interface to the Untrust zone. Set the IPv4 address of interface GE1/4/2 to 10.1.1.1 and add the interface to the Untrust zone. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Untrust zone. - Click Objects > ACL > IPv4 ACL. Click Create. Configure an ACL and an ACL rule. - Click Policies > NAT > NAT Advanced Settings > NAT Address Groups. Click Create. Configure a NAT address group. - Select Policies > NAT > Dynamic NAT. Click the Outbound Dynamic NAT (ACL-Based) tab, and then click Create. Configure an outbound NAT rule. - Ping PC 2 from PC 1. Result 1 is expected. The ping succeeds. A NAT session has been generated. |  |
| ☐ Passed | ☐ Failed |

  - NAT server

| Test Item Name: NAT server |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that the NAT server functions correctly. |  |
| Test Topology: |  |
| Test Process: - Add GE1/4/1, the interface that connects to PC1 on the private network, to the zone Trust, add GE1/4/2, the interface that connects to PC2 on the public network, to the zone Untrust, and configure a security policy to permit packets from Untrust to Trust. - Configure a NAT server rule on GE1/4/2. - Ping PC1 from PC2. Result 1 is expected. |  |
| Expect Results: - The ping succeeds. A NAT session have been generated. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv4 address of interface GE1/4/2 to 10.1.1.1 and add the interface to the Untrust zone. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Untrust zone. - Click Policies > NAT > NAT Servers > Policy Configuration. Click Create. Configure a NAT server rule. Choose use primary IP of the interface(Easy IP) as public IP address of the NAT server - Use PC2 to access the IP address of GE1/4/2. The ping succeeds. A NAT session have been generated. |  |
| ☐ Passed | ☐ Failed |

  - NAT ALG

| Test Item Name: NAT ALG |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that NAT ALG functions correctly. |  |
| Test Topology: |  |
| Test Process: - Add GE1/4/1, the interface that connects to PC on the private network, to the zone Trust, add GE1/4/2, the interface that connects to public Internet, to the zone Untrust, and configure a security policy to permit packets from Trust to Untrust. - Configure an IPv4 address object group. - Configure an NAT address group. - Configure a NAT rule on GE1/4/2. - Enable NAT ALG for FTP. - Establish the ftp server on Internet. Visit the ftp server from PC. Result 1 is expected. |  |
| Expect Results: - PC can access the ftp server and the NAT session is generated. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv4 address of interface GE1/4/2 to 10.1.1.1 and add the interface to the Untrust zone. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Untrust zone. - Click Objects > Object Groups > IPv4 Address Object Groups. Click Create. Configure an IPv4 address object group. - Click Policies > NAT > NAT Advanced Settings > NAT Address Groups. Click Create. Configure a NAT address group. - Click Policies > NAT > Dynamic NAT. On the Outbound Dynamic NAT (Object Group-Based) tab, click Create. Configure an outbound NAT rule. - Click Policies > NAT > NAT Advanced Settings > NAT ALG. Select the protocols you want to enable NAT ALG for. - Establish the ftp server on Internet. Use one PC to simulate the Internet. Use the FileZilla Server to establish it. 3CD is no suitable for this, the traffic from 3CD will be recognized as general TCP instead of FTP. Start the ftp server. Add the user for ftp. Set the downloading path. - Use PC to visit the ftp server. PC can access the ftp server. Check the NAT session on the firewall: |  |
| ☐ Passed | ☐ Failed |

  - NAT hairpin

| Test Item Name: NAT hairpin |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that the firewall supports the NAT hairpin function. |  |
| Test Topology: |  |
| Test Process: - Configure the IP address of each interface according to the network diagram - Add GE1/4/1 and 1/4/3 to zone Trust, and add GE1/4/2 to zone Untrust. - Configure an IPv4 security policy to permit packets between zone Trust to zone Untrust. - Configure NAT outbound policy on G1/4/2 and apply the ACL rules. - Configure NAT internal server on Gigabit Ethernet 1/4/2 and allow the external host to access the internal FTP server with the address 10.1.1.1. - Enable the NAT hairpin function on interface GE 1/4/1 and GE 1/4/3. - Use the intranet and extranet PC to access the intranet FTP Server. Result 1 is expected. |  |
| Expect Results: - The intranet and extranet PC can access the intranet FTP Server through the external network address 10.1.1.1. When accessing FTP Server, the source address will be translated to 10.1.1.1. |  |
| Actual Result： - Assign IP addresses to interfaces and add the interfaces to security zones. On the top navigation bar, click Network. From the navigation pane, select Interface Configuration > Interfaces. Set the IPv4 address of interface GE1/4/2 to 10.1.1.1 and add the interface to the Untrust zone. Set the IPv4 address of interface GE1/4/3 to 3.1.1.1 and add the interface to the Trust zone. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Untrust zone. - Configure NAT. Click Policies > NAT > NAT Servers > Policy Configuration. Click Create, create a NAT server rule. - Click Policies > NAT > Dynamic NAT. On the Outbound Dynamic NAT (ACL-Based) tab, click Create. Create an outbound dynamic NAT rule. - Click Policies > NAT > NAT Advanced Settings > NAT Hairpin. Select GE 1/4/1 and GE1/4/3, and click Enable. GE 1/4/1 and GE 1/4/3 are enabled with NAT hairpin. - Use the intranet and extranet PC to access the intranet FTP Server. The intranet and extranet PC can access the intranet FTP Server through the external network address 10.1.1.1. When accessing FTP Server, the source address will be translated to 10.1.1.1. Use the extranet PC: Check the NAT session on the firewall: Use the intranet PC: Check the NAT session on the firewall: Click Monitor > Sessions Check the sessions in firewall webpage. |  |
| ☐ Passed | ☐ Failed |

  - NAT 64（AFT）

| Test Item Name: NAT 64（AFT） |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that the AFT functions correctly for access from an IPv6 network to the IPv4 Internet. |  |
| Test Topology: |  |
| Test Process: - Configure an AFT address group and add IPv4 addresses to the group. - Configure IPv6 ACL 2000 to o identify packet sourced from 30::/96. - Configure the firewall to use AFT address group 1 to translate source addresses for IPv6 packets permitted by ACL 2000. IPv6 addresses of packets sourced from 30::/96 are translated IPv4 addresses in from 60.1.1.1 to 60.1.1.255. - Configure a NAT64. - Enable AFT on the IPv6-side and IPv4-side interfaces of the firewall. - Access the server from the host. Result 1 is expected. |  |
| Expect Results: - Access to the IPv4 network is allowed. IPv6 packets are translated into IPv4 packets. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv6 address of interface GE1/4/1 to 30::1/96 and add the interface to the Trust zone. Set the IPv4 address of interface GE1/4/2 to 10.1.1.1 and add the interface to the Untrust zone. - Click Policies > Security Policies > Security Policies. Configure the ipv4 and ipv6 security policy. For the IPv6 security policy, configure a rule named 2 to allow the firewall to perform AFT on the IPv6 PC traffic destined for the IPv4 servers. For the IPv4 security policy, configure a rule named 1 to allow the firewall to forward the AFT-translated packets to the IPv4 servers. - Click Policies > AFT > AFT Address Groups Configure an AFT address group and add IPv4 addresses to the group. - Click Objects > ACL > IPv6 ACL Configure IPv6 ACL 2000 to identify packet sourced from 30::/96. - Click Policies > AFT > AFT Policies. Configure the firewall to use AFT address group 0 to translate source addresses for IPv6 packets permitted by ACL 2000. IPv6 addresses of packets sourced from 30::/96 are translated IPv4 addresses from 60.1.1.1 to 60.1.1.255. - Click Policies > AFT > AFT Prefixes. Configure a NAT64 prefix 40::/96. The destination IPv6 addresses matching the prefix will be translated into an IPv4 address. - Click Policies > AFT > Enable AFT. Enable AFT on the interfaces of the firewall. - Verify the connectivity between IPv6 hosts and IPv4 servers. Ping IPv4 server from IPv6 PC. Display detailed information about IPv6 AFT sessions on the device. |  |
| ☐ Passed | ☐ Failed |

  - NAT 66

| Test Item Name: NAT 66 |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that NAT66 prefix translation functions correctly. |  |
| Test Topology: |  |
| Test Process: - Add GE1/4/1, the interface that connects to PC on the private network, to the zone Trust, add GE1/4/2, the interface that connects to Internet on the public network, to the zone Untrust, and configure a security policy to permit packets from Trust to Untrust. - On GE1/4/2, configure a NAT66 prefix translation rule from 30::/96 to 50::/96. - Ping Internet from PC. Result 1 is expected. |  |
| Expect Results: - The ping succeeds. A NAT66 session has been generated. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv6 address of interface GE1/4/1 to 30::1/96 and add the interface to the Trust zone. Set the IPv6 address of interface GE1/4/2 to 40::1/96 and add the interface to the Untrust zone. - Click Policies > Security Policies > Security Policies. Configure the ipv6 security policy. - Click Policies > NAT66 > NAT Prefix Translation. Click Create. Configure a NAT66 prefix translation rule. The source IPv6 prefix after translation cannot be the same as the external prefix of the NAT66 device or the prefix of the external destination address. So here we set the IPv6 prefix/prefix length after NAT to 50:: /96. - Use the PC to access the Internet. Internal PC can access the Internet. The relevant NAT66 sessions are established. |  |
| ☐ Passed | ☐ Failed |

  - Global NAT (source address translation)

| Test Item Name: Global NAT (source address translation) |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that the global NAT can perform source IP address translation. |  |
| Test Topology: |  |
| Test Process: - Add GE1/4/1, the interface that connects to PC1 on the private network, to the zone Trust, add GE1/4/2, the interface that connects to PC2 on the public network, to the zone Untrust, and configure a security policy to permit packets from Trust to Untrust. - Configure a NAT address group on the device. - Configure a global NAT rule on the device. - Ping 10.1.1.2 from PC1. Result 1 is expected. |  |
| Expect Results: - The source IP address of PC1 is translated into an IP address in the NAT address group. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv4 address of interface GE1/4/1 to 1.1.1.1 and add the interface to the Trust zone. Set the IPv4 address of interface GE1/4/2 to 10.1.1.1 and add the interface to the Untrust zone. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Untrust zone. - Configure global NAT. Click Policies > NAT > NAT Advanced settings > NAT Address Groups. Click Create to create a new NAT address group. - Click the Policies > NAT > NAT Policy. Create a NAT policy. - Ping address 10.1.1.2 on the PC1. Result 1 is expected. The ping operation successes. The source IP address of PC1 is translated into an IP address in the NAT address group. |  |
| ☐ Passed | ☐ Failed |

  - IRF HA

| Test Item Name: IRF HA |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that the device supports DHCP |  |
| Test Topology: |  |
| Test Process: - Add the interface G1/0/2 of firewall A to the Trust zone. Configure the security policy to allow the access of traffic prior to the Trust and Local zones. - Add the interface G1/0/2 of firewall B to the Trust zone. Configure the security policy to allow the access of traffic prior to the Trust and Local zones. - Configuring IRF on firewall A. - Configuring IRF on firewall B. - Check the IRF status on firewall A. Result 1 is expected. |  |
| Expect Results: - Firewall A and firewall B are IRF member devices and the IRF ports are up. |  |
| Actual Result： - Add the G1/0/2 of firewall A and interface G1/0/2 of firewall B to the security zone. On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Add the interface G1/0/2 of firewall A to the Trust zone. Add the interface G1/0/2 of firewall A to the Trust zone. - Set Action of the security policy configured for interface G1/0/2 of firewalls A and G1/0/2 of firewalls B to Permit. On firewall A, perform the following: Click the Policies tab. Choose Security Policies > Security Policies. On firewall B, perform the following: - Configuring IRF on firewall A. Click the System tab. Choose Virtualization > IRF. Click the Settings icon for the target IRF port. In this example, IRF-port 2 is used Configure IRF parameters as shown in figure. - Configuring IRF on firewall B. Click the System tab. Choose Virtualization > IRF. Click the Settings icon for the target IRF port. In this example, IRF-port 1 is used Configure IRF parameters as shown in figure. - Use the management address of the master (firewall A) to log in to the Web interface of the IRF fabric. Click the System tab. Choose Virtualization > IRF. Verify that firewall A and firewall B are IRF member devices and their IRF ports are up. |  |
| ☐ Passed | ☐ Failed |

  - Virtual firewall - independent management and independent services

| Test Item Name: Virtual firewall - independent management and independent services |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that a physical firewall can be virtualized as multiple virtual firewalls that are independently managed, independently implement resource management, and independently provide services. |  |
| Test Topology: |  |
| Test Process: - Enter the system view of the physical firewall. Create virtual firewalls vf1 and vf2. - Allocate shared interface GE1/4/0 to vf1 as the management interface. Allocate interfaces GE1/4/1 and GE1/4/2 to vf1 in exclusive mode. Allocate shared interface GE1/4/0 to vf2 as the management interface. Allocate interfaces GE1/4/3 and GE1/4/4 to vf2 in exclusive mode. - Switch services from the primary firewall to vf1, add interface GE1/4/0 to the Management zone, set the management IP address to 1.1.1.10, and enable the HTTPS service. Switch services from the primary firewall to vf2, add interface GE1/4/0 to the Management zone, set the management IP address to 1.1.1.11, and enable the HTTPS service. - Log in to vf1 from https://1.1.1.10. Result 1 is expected. - Log in to vf2 from https://1.1.1.11. Result 2 is expected. - Log in to vf1 and restart it. Result 3 is expected. |  |
| Expect Results: - You can successfully log in to vf1. Only interfaces GE1/4/1 and GE1/4/2 are available. - You can successfully log in to vf2. Only interfaces GE1/4/3 and GE1/4/4 are available. - If vf1 is restarted, vf2 continues the services normally without any influence. |  |
| Actual Result： - Create two virtual firewalls. Click the System > Virtualization > Contexts > Contexts. Create two virtual firewalls. Allocate shared interface GE1/4/0 to vf1 as the management interface. Allocate interfaces GE1/4/1 and GE1/4/2 to vf1 in exclusive mode. Allocate shared interface GE1/4/0 to vf2 as the management interface. Allocate interfaces GE1/4/3 and GE1/4/4 to vf2 in exclusive mode. Select two virtual firewalls, and click Start. On firewall console, configure as follows: - Switch services from the primary firewall to vf1 and vf2. - Configure root firewall Firewall. - Switch services to vf1. [Firewall] switchto context context1 Add interface GE1/4/0 to the Management zone and set the management IP address to 1.1.1.10. Enable the HTTP and HTTPS service and configure the user name and password for web login. (3) Switch services to vf2. [Firewall] switchto context context2 Add interface GE1/4/0 to the Management zone and set the management IP address to 1.1.1.11. Enable the HTTPS service and configure the user name and password for web login. - Visit the web page https://1.1.1.10 of the virtual firewall in the browser. Enter the account name (admin) and password (h3c.com123) to log in to vf1. You can successfully log in to vf1. Only interfaces GE1/4/1 and GE1/4/2 are available. - Visit the web page https://1.1.1.11 of the virtual firewall in the browser. Enter the account name (admin) and password (h3c.com123) to log in to vFW2. You can successfully log in to vf2. Only interfaces GE1/4/3 and GE1/4/4 are available. - Reboot the vf1, check the status of vf2. The vf2 can be used normally when vf1 is rebooting. |  |
| ☐ Passed | ☐ Failed |

  - IPS

| Test Item Name: IPS |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that IPS functions correctly. |  |
| Test Topology: |  |
| Test Process: - Connect GE1/4/1 on the firewall to the PC. - Create an IPS profile with all rules enabled. - Add GE1/4/1 to security zone Trust, and create a security policy. In the security policy, specify both source security zone and destination zone as Trust, and apply the IPS profile. - Use the PC to send the prepared testing packets to perform packet replay, for example, the xss.pcap file. Make sure the testing packets can be processed by the firewall. - Log in to the web interface of the device. Result 1 is expected. |  |
| Expect Results: - The device generates a threat log. You can view the threat log on the Monitor > Security Logs > Threat Logs page. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the security zone for G1/4/1 and change the port to bridge mode. - Click the Network > Interface Configuration > Inline > Interface Pairs, click Create. In the dialog box that opens, select Blackhole for the Forwarding mode field. - On the Objects > APPSecurity > IPS > Profiles page, click Create. In the dialog box that opens, select All in the Protected target field, and then click OK. - On the Policies > Security Policies > Security Policies page, create a security policy and apply IPS policy 1 and enable the logging function. Then, click Activate and Submit. - Use the PC to send the prepared testing packets to perform packet replay. - Start the packet player. - Select network card: select the network card to play back the packet. - Select packet file: select the packet file to be played back; Some packets may be invalid, so it is recommended to add them all. - Select the number of times to send data packets, if it is 0, it means to send unlimited times - Send the data packets. - Click Monitor > Security Logs > Threat Logs to view the threat log on firewall. |  |
| ☐ Passed | ☐ Failed |

  - Signature library update

| Test Item Name: Signature library update |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that the IPS signature library, APR signature library, URL filtering signature library, and AV signature library can be updated correctly. |  |
| Test Topology: |  |
| Test Process: - Assign IP address 184.38.0.125/24 (a public network IP address) to GE1/4/2 on the firewall and add the GE1/4/2 to Untrust zone. - Assign IP address 1.1.1.1 to GE1/4/1 on the firewall and add the GE1/4/2 to Trust zone. - Log in to the Web interface of the firewall. - On the System > Update Center > Signature Library Update page, click Manually Update for the IPS signature library to manually update the IPS signature library. Result 1 is expected. - Click Auto Update for the IPS signature library and set the scheduled update time. Result 2 is expected. - Click Online Update for the IPS signature library to update the IPS signature library immediately. Result 2 is expected. |  |
| Expect Results: - The firewall loads the locally stored update IPS signature file and updates the IPS signature library successfully. - The firewall communicates with the H3C server to update the IPS signature library immediately or at the set scheduled time. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv4 address of interface GE1/4/2 to 184.38.0.125/24 and add the interface to the Untrust zone. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Untrust zone. - On the System > Update Center > Signature Update page, click Manually Update for the IPS signature library to manually update the IPS signature library. In this way, you just need to let the PC and the firewall can access each other. The firewall loads the locally stored update IPS signature file and updates the IPS signature library successfully. The link for downloading the signature library: https://www.h3c.com/cn/Products___Technology/Products/IP_Security/Characteristic_Service_Area/ It is recommended to download the latest released version. - Click Auto update for the IPS signature library and set the scheduled update time. - Click Online Update for the IPS signature library to update the IPS signature library immediately. In this way, the firewall must have the route to access the Internet. |  |
| ☐ Passed | ☐ Failed |

  - HTTP-based anti-virus

| Test Item Name: HTTP-based anti-virus |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that the HTTP-based anti-virus for HTTP functions correctly. |  |
| Test Topology: |  |
| Test Process: - Click the Network tab. Choose Interface Configuration > Interfaces. Configure IP addresses for the PCs that act as the server and client respectively. - Add two interfaces of the firewall to the Trust zone. - Configure antivirus. Create an antivirus profile named av. Adopt the default settings. - Apply the antivirus service to the security policy. Enter the IPv4 security policy rule view. Create security policy rule 1. Configure the rule with an action of Permit to apply anti-virus application profile to packets from security zone Trust to security zone Untrust. Set the action to Permit. - Activate the security policy configuration. - Install the HTTP server on the server. Access the HTTP server address on the client. Download the virus file to local. - If an HTTP configuration item is blocked by the antivirus profile, the virus file cannot be downloaded normally, result 1 is expected. |  |
| Expect Results: - After alarms are generated, you can view the following information: Click the Monitor tab. Choose Security Logs > Threat logs to view antivirus logs. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv4 address of interface GE1/4/1 to 1.1.1.1 and add the interface to the Trust zone. Set the IPv4 address of interface GE1/4/2 to 10.1.1.1 and add the interface to the Untrust zone. - Click Objects > APPSecurity > Anti-Virus > Profile. Click Create to customize an antivirus profile named av. - Click the Policies tab. Choose Security Policies > Security Policies. Configure the security policy to reference the custom antivirus profile. Configure the security policy between Trust zone and Untrust zone, apply the Anti-virus profile and enable the logging function. - Click the Objects > APPSecurity > Anti-Virus > Profile. Edit the antivirus profile and modify the HTTP protocol detection direction, including upload and download. Modify the action, including Alarm, Block, and Redirect. - Run HFS program, set the download address IP as 1.1.1.2. Right click the box on the left and choose Add files to upload the virus file on the http server. Use PC2 with the IP address 10.1.1.2 to visit the HTTP server 1.1.1.2 on the PC1. Click the file to download it. The firewall will block the virus files, so the download failed. - Click the Monitor > Security Logs > Threat Logs to view antivirus logs. Click to view details of the threat logs. |  |
| ☐ Passed | ☐ Failed |

  - URL filtering by category

| Test Item Name: URL filtering by category |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that the firewall supports URL filtering by category. |  |
| Test Topology: |  |
| Test Process: - Configure the interface IP address and add the interface to the security zone. - Click the Objects tab. Choose APPSecurity > URL Filtering > URL Categories. On the custom URL category page, configure the URL category named 1 with a priority of 1000. Select the text mode for the URL rules. Set the host to 1.1.1.2. Click OK. - Click the Objects tab. Choose APPSecurity > URL Filtering. On the URL policy configuration page, configure the profile. Set the name to uft1 and default action to Permit. Reference custom URL category 1 and set the action to Permit. - Click the Policies tab. Choose Security Policies > Security Policies. Reference URL filtering profile uft1 in content security, and activate the settings. - Visit 1.1.1.2. Expected result 1 is obtained. View the URL logs. Expected result 2 is obtained. - Change the action in URL filtering profile uft1 to Drop. Activate the settings. - Visit 1.1.1.2 News page again. Expected result 3 is obtained. View the URL logs. Expected result 4 is obtained. |  |
| Expect Results: - The PC can access the 1.1.1.2 website. - The firewall generates a URL filtering log. The URL filtering log is in the URL Filtering Log List of the Monitor > Security Logs > URL Filtering Logs page. - The PC fails to access the http://1.1.1.2. - The firewall generates a URL filtering log. The URL filtering log is in the URL Filtering Log List of the Monitor > Security Logs > URL Filtering Logs page. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv4 address of interface GE1/4/1 to 1.1.1.1 and add the interface to the Trust zone. Set the IPv4 address of interface GE1/4/2 to 10.1.1.1 and add the interface to the Untrust zone. - Click Objects > APPSecurity > URL Filtering > URL Categories. Click Create, configure the URL category and host name. - Click the Objects > APPSecurity > URL Filtering > Profiles, create a URL filtering profile named uflt and set the actions for URL category 1 to permit and logging. - Click the Policies tab. Choose Security Policies > Security Policies. Configure the security policy to apply the url category. Configure the security policy between Trust zone and Untrust zone, apply the URL category and enable the logging function. Click Activate and then Submit to activate the operation. - When the action in the profile is set to Permit, use PC2 to access the HTTP server page with URL http:/1.1.1.2 on PC1, PC2 cannot access the HTTP server of PC1, the following result is output: Click the Monitor > Security Logs > URL Filtering Logs. You can see that the firewall generates the URL filtering log. Click to view details of the logs. The action is permit. - Click the Objects > APPSecurity > URL Filtering > Profiles. Change the action in the URL profile to drop. - When the action in the profile is set to drop, use PC2 to access the HTTP server page with URL http:/1.1.1.2 on PC1, PC2 cannot access the HTTP server of PC1, the following result is output: Click the Monitor > Security Logs > URL Filtering Logs. You can see that the firewall generates the URL filtering log. Click to view details of the logs. The action is drop. |  |
| ☐ Passed | ☐ Failed |

  - Application auditing

| Test Item Name: Application auditing |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify the application auditing function. |  |
| Test Topology: |  |
| Test Process: - Configure IP addresses for GE1/4/1, and GE1/4/2. - Assign GE1/4/1 to the Trust security zone, and assign GE1/4/2 to the Untrust security zone and add the IP address. Configure the security policy between Trust zone and Untrust zone with the action as permit. - Create an audit-type audit policy, and configure an audit rule to permit ftp, audit all the actions and generate audit logs. - Deploy the audit policy. - Activate the audit policy. - Establish the ftp server on PC1 and use PC2 to visit the ftp server and download the file. Result 1 is expected. |  |
| Expect Results: - The FTP access and download are successful, and the log can be displayed from the Monitor > Application Audit Logs > File Transfer Logs page. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv4 address of interface GE1/4/1 to 1.1.1.1 and add the interface to the Trust zone. Set the IPv4 address of interface GE1/4/2 to 10.1.1.1 and add the interface to the Untrust zone. - Click the Policies tab. Choose Security Policies > Security Policies. Configure the security policy between Trust zone and Untrust zone. z - Click Policies > Application Audit > Audit Policies. Click Create. Configure an audit-type audit policy. - Establish the ftp server on PC1. Use the FileZilla Server to establish it. 3CD is no suitable for this, the traffic from 3CD will be recognized as general TCP instead of FTP. Start the ftp server. Add the user for ftp. Set the downloading path. - Use PC2 to visit the ftp server on PC1 Login to the server. Download a file on the server. The FTP access and download are successful. - Click the Monitor > Application Audit Logs > File Transfer Logs page to check the audit logs. |  |
| ☐ Passed | ☐ Failed |

  - Bandwidth limiting based on total bandwidth and shared mode

| Test Item Name: Bandwidth limiting based on total bandwidth and shared mode |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that bandwidth limiting can be performed based on total bandwidth and shared mode. |  |
| Test Topology: |  |
| Test Process: - Configure IP addresses for GE1/4/1, and GE1/4/2. - Assign GE1/4/1 to the Trust security zone, and assign GE1/4/2 to the Untrust security zone and add the IP address. Configure the security policy between Trust zone and Untrust zone with the action as permit. - Create a traffic profile named ftp in shared mode, and set the total bandwidth limit to 1 Mbps. Create a traffic policy named ftp, and specify applications FTP and FTP-data, and reference traffic profile ftp. - Establish the ftp server on PC1 and use PC2 to access the ftp server, then download the file on the ftp server. Result 1 is expected. |  |
| Expect Results: - The download speed is limited to 1 Mbps. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv4 address of interface GE1/4/1 to 1.1.1.1 and add the interface to the Trust zone. Set the IPv4 address of interface GE1/4/2 to 10.1.1.1 and add the interface to the Untrust zone. - Click the Policies tab. Choose Security Policies > Security Policies. Configure the security policy between Trust zone and Untrust zone. z - Click Policies > Bandwidth Management > Traffic Profiles. Create a traffic profile named ftp. - Click Policies > Bandwidth Management > Traffic Policies. Create a traffic policy named ftp. - Establish the ftp server on PC1. Use the FileZilla Server to establish it. 3CD is no suitable for this, the traffic from 3CD will be recognized as general TCP instead of FTP. Start the ftp server. Add the user for ftp. Set the downloading path. - Use PC2 to visit the ftp server on PC1. Use FileZilla to establish the ftp connection with the ftp server. Access the ftp server and download a file. The download speed is limited to 1 Mbps. |  |
| ☐ Passed | ☐ Failed |

  - IPsec transport encapsulation mode with ESP

| Test Item Name: IPsec transport encapsulation mode with ESP |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that the firewall devices support the IPsec transport encapsulation mode with the ESP security protocol. |  |
| Test Topology: |  |
| Test Process: - On Firewall, assign GE 1/4/1 to security zone Untrust and assign IP address 201.1.1.1/24 to GE 1/4/1. Assign GE 1/4/2 to security zone Trust and assign IP address 60.1.1.1/24 to GE 1/4/2. - On Router, assign IP address 201.1.1.3/24 to GE 0/0. Assign IP address 50.1.1.1/24 to GE 0/1. - On Firewall, configure security policies to allow traffic between zone Local and zone Untrust and between zone Trust and Untrust. On Router, configure security policies to allow traffic between zone Local and zone Untrust and zone Trust and zone Untrust. - On Firewall, configure a route destined for 50.1.1.0/24. On Router, configure a route destined for 60.1.1.0/24. Result 1 is expected. - On Firewall, configure an IPsec policy: ·Set the policy name to p2 and priority to 1. ·Specify interface GE 1/4/1, set the local address to 201.1.1.1 and the remote IP address to 201.1.1.3. ·Configure IKE profile settings. Set the negotiation mode to aggressive, the authentication method to preshared key, the preshared key to 123, and the peer ID to 201.1.1.3. ·Configure two data flow filtering rules. For one rule, configure the source address as 60.1.1.2 and destination address as 50.1.1.2. For the other rule, the source address as 201.1.1.1 and destination address as 201.1.1.3. ·Configure advanced settings. Set the encapsulation mode to transport, the security protocol to ESP, the authentication algorithm to SHA1, and the encryption algorithm to 3DES-CBC. - On Router, configure an IPsec policy: ·Set the policy name to p2 and priority to 1. ·Specify interface GE 0/0, set the local address to 201.1.1.3 and the remote IP address to 201.1.1.1. ·Configure IKE profile settings. Set the negotiation mode to aggressive, the authentication method to preshared key, the preshared key to 123, and the peer ID to 201.1.1.1. ·Configure two data flow filtering rules. For one rule, configure the source address as 50.1.1.2 and destination address as 60.1.1.2. For the other rule, the source address as 201.1.1.3 and destination address as 201.1.1.1. ·Configure advanced settings. Set the encapsulation mode to transport, the security protocol to ESP, the authentication algorithm to SHA1, and the encryption algorithm to 3DES-CBC. - Ping PC 2 from PC 1. Result 2 is expected. - Ping 201.1.1.3 from Firewall Result 3 is expected. |  |
| Expect Results: - PC2 can be successfully pinged on PC1. - PC2 cannot be successfully pinged on PC1. - Address 201.1.1.3 can be successfully pinged on FWA. Click the Network tab. On the navigation tree, choose VPN > IPsec > IPsec Tunnels. Click Details to view the detailed information of the tunnel. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv4 address of interface GE1/4/1 to 201.1.1.1 and add the interface to the Untrust zone. Set the IPv4 address of interface GE1/4/2 to 60.1.1.1 and add the interface to the Trust zone. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Untrust zone. - Configure a route destined for 50.1.1.0/24. Click the Network > Routing > Static Routing. Create a route with Destination address set to 50.1.1.0/24 and Next hop address to 201.1.1.3. - On Router, configure IP address for GE 0/0 and GE 0/1, then add a route destined for 60.1.1.0/24. - Ping PC2 from PC1, PC2 can be successfully pinged on PC1. - Create an IPSec policy on Firewall. Click the Network > VPN > IPsec > IPsec Policies. - Set Policy name to p2, Priority to 1, Interface to GE1/0/1, Local address to 201.1.1.1, and Remote IP/hostname to 201.1.1.3. - Configure the IKE policy. Set Negotiation mode to Aggressive, Authentication method to Preshared key, Preshared key to 123, and Peer ID to 201.1.1.3. - Configure protected data flow 1 with Src IP address set to 60.1.1.2 and Dest IP address to 50.1.1.2. Configure protected data flow 2 with Src IP address set to 201.1.1.1 and Dest IP address to 201.1.1.3. - Configure the advanced parameters with Encapsulation mode set to Transport, Security protocol to ESP, ESP authentication algorithm to SHA1, and ESP encryption algorithm to 3DES-CBC. - Create an IPSec policy on Router. - Create an IPsec transform set named p2. Set the encapsulation mode to transport, the security protocol to ESP, the authentication algorithm to SHA1, and the encryption algorithm to 3DES-CBC. - Configure an acl rule named ipsec and add two data flow filtering rules. For one rule, configure the source address as 50.1.1.2 and destination address as 60.1.1.2. For the other rule, the source address as 201.1.1.3 and destination address as 201.1.1.1. - Create an IKE keychain named p2. Specify 123 in plain text as the preshared key to be used with the remote peer at 201.1.1.1. - Create and configure the IKE profile named p2. - Create an IKE-based IPsec policy entry with name p2 and priority to 1. Apply ACL 3101. Apply IPsec transform set tran1. Specify the local and remote IP addresses of the IPsec tunnel as 201.1.1.3 and 201.1.1.1. - Apply IPsec policy map1 to GigabitEthernet 0/0. - Ping PC2 from PC1. PC2 cannot be successfully pinged on PC1. - Ping 201.1.1.3 from Firewall. Address 201.1.1.3 can be successfully pinged on Firewall. Click the Network tab. On the navigation tree, choose VPN > IPsec > IPsec Tunnels. Click Details to view the detailed information of the tunnel. |  |
| ☐ Passed | ☐ Failed |

  - IPsec transport encapsulation mode with AH

| Test Item Name: IPsec transport encapsulation mode with AH |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that the firewall devices support the IPsec transport encapsulation mode with the AH security protocol. |  |
| Test Topology: |  |
| Test Process: - On Firewall, assign GE 1/4/1 to security zone Untrust and assign IP address 201.1.1.1/24 to GE 1/4/1. Assign GE 1/4/2 to security zone Trust and assign IP address 60.1.1.1/24 to GE 1/4/2. - On Router, assign IP address 201.1.1.3/24 to GE 0/0. Assign IP address 50.1.1.1/24 to GE 0/1. - On Firewall, configure security policies to allow traffic between zone Local and zone Untrust and between zone Trust and Untrust. On Router, configure security policies to allow traffic between zone Local and zone Untrust and zone Trust and zone Untrust. - On Firewall, configure a route destined for 50.1.1.0/24. On Router, configure a route destined for 60.1.1.0/24. Result 1 is expected. - On Firewall, configure an IPsec policy: ·Set the policy name to p2 and priority to 1. ·Specify interface GE 1/4/1, set the local address to 201.1.1.1 and the remote IP address to 201.1.1.3. ·Configure IKE profile settings. Set the negotiation mode to aggressive, the authentication method to preshared key, the preshared key to 123, and the peer ID to 201.1.1.3. ·Configure two data flow filtering rules. For one rule, configure the source address as 60.1.1.2 and destination address as 50.1.1.2. For the other rule, the source address as 201.1.1.1 and destination address as 201.1.1.3. ·Configure advanced settings. Set the encapsulation mode to transport, the security protocol to AH, and the authentication algorithm to SHA1. - On Router, configure an IPsec policy: ·Set the policy name to p2 and priority to 1. ·Specify interface GE 0/0, set the local address to 201.1.1.3 and the remote IP address to 201.1.1.1. ·Configure IKE profile settings. Set the negotiation mode to aggressive, the authentication method to preshared key, the preshared key to 123, and the peer ID to 201.1.1.1. ·Configure two data flow filtering rules. For one rule, configure the source address as 50.1.1.2 and destination address as 60.1.1.2. For the other rule, the source address as 201.1.1.3 and destination address as 201.1.1.1. ·Configure advanced settings. Set the encapsulation mode to transport, the security protocol to AH, and the authentication algorithm to SHA1. - Ping PC 2 from PC 1. Result 2 is expected. - Ping 201.1.1.3 from Firewall. Result 3 is expected. |  |
| Expect Results: - PC 1 can ping PC 2 successfully. - PC 1 cannot ping PC 2. - Firewall can ping 201.1.1.3 successfully. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv4 address of interface GE1/4/1 to 201.1.1.1 and add the interface to the Untrust zone. Set the IPv4 address of interface GE1/4/2 to 60.1.1.1 and add the interface to the Trust zone. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Untrust zone. - Configure a route destined for 50.1.1.0/24. Click the Network > Routing > Static Routing. Create a route with Destination address set to 50.1.1.0/24 and Next hop address to 201.1.1.3. - On Router, configure IP address for GE 0/0 and GE 0/1, then add a route destined for 60.1.1.0/24. - Ping PC2 from PC1, PC2 can be successfully pinged on PC1. - Create an IPSec policy. Click the Network tab. Choose VPN > IPsec > IPsec Policies. - Set Policy name to p2, Priority to 1, Interface to GE1/0/1, Local address to 201.1.1.1, and Remote IP/hostname to 201.1.1.3. - Configure the IKE policy. Set Negotiation mode to Aggressive, Authentication method to Preshared key, Preshared key to 123, and Peer ID to 201.1.1.3. - Configure protected data flow 1 with Src IP address set to 60.1.1.2 and Dest IP address to 50.1.1.2. Configure protected data flow 2 with Src IP address set to 201.1.1.1 and Dest IP address to 201.1.1.3. - Configure the advanced parameters with Encapsulation mode set to Transport, Security protocol to AH, and AH authentication algorithm to SHA1. - Create an IPSec policy on Router. - Create an IPsec transform set named p2. Set the encapsulation mode to transport, the security protocol to ah, the authentication algorithm to SHA1. - Configure an acl rule named ipsec and add two data flow filtering rules. For one rule, configure the source address as 50.1.1.2 and destination address as 60.1.1.2. For the other rule, the source address as 201.1.1.3 and destination address as 201.1.1.1. - Create an IKE keychain named p2. Specify 123 in plain text as the preshared key to be used with the remote peer at 201.1.1.1. - Create and configure the IKE profile named p2. Set Negotiation mode to Aggressive. - Create an IKE-based IPsec policy entry with name p2 and priority to 1. Apply ACL name ipsec. Apply IPsec transform set tran1. Specify the local and remote IP addresses of the IPsec tunnel as 201.1.1.3 and 201.1.1.1. - Apply IPsec policy map1 to GigabitEthernet 0/0. - Ping PC2 from PC1. PC2 cannot be successfully pinged on PC1. - Click the Network tab. On the navigation tree, choose VPN > IPsec > IPsec Tunnels. Click Details to view the detailed information of the tunnel. |  |
| ☐ Passed | ☐ Failed |

  - IPsec SVTI tunnel mode

| Test Item Name: IPsec SVTI tunnel mode |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that the firewall devices support IPsec tunnels in SVTI mode. |  |
| Test Topology: |  |
| Test Process: - On Firewall, assign GE 1/4/1 to security zone Untrust and assign IP address 201.1.1.1/24 to GE 1/4/1. Assign GE 1/4/2 to security zone Trust and assign IP address 60.1.1.1/24 to GE 1/4/2. - On Router, assign IP address 201.1.1.3/24 to GE 1/0/1. Assign IP address 50.1.1.1/24 to GE 1/0/2. - On Firewall, configure security policies to allow traffic between zone Local , Untrust and Trust. - On Firewall, perform the following tasks: ·Create an IKE keychain, and specify 123 in plain text as the preshared key to be used with the remote peer at 201.1.1.3. ·Create an IKE profile, set the negotiation mode to aggressive, apply the IKE keychain to the IKE profile, and specify 201.1.1.3/32 as the peer IP address. ·Create an IPsec transform set, specify the ESP security protocol, set the encryption algorithm to 3DES-CBC, and set the authentication algorithm to HMAC-SHA1. ·Create an IPsec profile, and apply the IPsec transform set and IKE profile to the IPsec profile. ·Create an IPsec tunnel interface, configure the tunnel interface IP address as 77.0.0.1/24, the tunnel source address as 201.1.1.1, and the tunnel destination address as 201.1.1.3. Apply the IPsec profile to the IPsec tunnel interface and assign the tunnel interface to security zone Untrust. ·Configure a static route destined for PC 2. The output interface is the IPsec tunnel interface. - On Router, perform the following tasks: ·Create an IKE keychain, and specify 123 in plain text as the preshared key to be used with the remote peer at 201.1.1.1. ·Create an IKE profile, set the negotiation mode to aggressive, apply the IKE keychain to the IKE profile, and specify 201.1.1.1/32 as the peer IP address. ·Create an IPsec transform set, specify the ESP security protocol, set the encryption algorithm to 3DES-CBC, and set the authentication algorithm to HMAC-SHA1. ·Create an IPsec profile, and apply the IPsec transform set and IKE profile to the IPsec profile. ·Create an IPsec tunnel interface, configure the tunnel interface IP address as 77.0.0.2/24, the tunnel source address as 201.1.1.3, and the tunnel destination address as 201.1.1.1. Apply the IPsec profile to the IPsec tunnel interface and assign the tunnel interface to security zone Untrust. ·Configure a static route destined for PC 1. The output interface is the IPsec tunnel interface. - Ping PC 2 from PC 1. Result 1 is expected. |  |
| Expect Results: - PC 1 can ping PC 2 successfully. |  |
| Actual Result： - On Firewall, configure the following on the web UI and CLI: - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1 and G1/4/2. Set the IPv4 address of interface GE1/4/1 to 201.1.1.1 and add the interface to the Untrust zone. Set the IPv4 address of interface GE1/4/2 to 60.1.1.1 and add the interface to the Trust zone. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Untrust zone. - Configure the IKE keychain. - Configure the IKE profile. - Configure the IPSec security proposal. - Configure the IPSec security framework. - Configure the IPSec Tunnel interface. - Add IPSec Tunnel interface to the Untrust zone. - Configure a route destined for 50.1.1.0/24. On Router, configure the following on the CLI: - Configuration on Router. - Configure the IKE keychain. - Configure the IKE profile. - Configure the IPSec security proposal. - Configure the IPSec security framework. - Configure the IPSec Tunnel interface. - Configure a route destined for 60.1.1.0/24. - Ping PC 2 from PC 1. Run the display interface tunnel command to view the tunnel status of Firewall. The information is as follows: Run the display ipsec sa command to view the IPSec SA generated through negotiation by Firewall. The information is as follows: |  |
| ☐ Passed | ☐ Failed |

  - SYN flood attack defense

| Test Item Name: SYN flood attack defense |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that SYN flood attack defense functions correctly. |  |
| Test Topology: |  |
| Test Process: - Assign an IP address to GE1/4/1 on the firewall. - Add GE1/4/1 to security zone Trust. - Configure a security policy as follows: ·Select Any from the Source zone list. ·Select Any from the Destination zone list. ·Set the action to Permit. - Configure an attack defense policy. In the policy, configure flood attack defense global settings for SYN flood attacks as follows: ·Set the threshold to 10. ·Select Logging, Detect all IPs, and Packet drop. - Use xcap to send TCP SYN attack packets from the PC to the firewall at the rate of 100 fps. Make sure over 100 IP addresses on the subnet 1.1.1.0/24 are used as the source IP addresses of the attack packets. - Display flood attack defense logs. Result 1 is expected. |  |
| Expect Results: - The firewall generates a flood attack log and collects SYN flood attack statistics (including attack times and number of dropped packets). |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1. View the MAC address of interface G1/4/1. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Local zone. - Create an attack defense policy. Click the Policies > Attack Defense > Attack Defense Policies. Set Policy name to 1 and Apply to to Trust. Click Flood Attack Defense Global Settings, select Logging, Detect all IPs, and Packet drop check boxes for SYN, and set Threshold to 10. - Build TCP SYN packets using the XCAP tool. Inject the packets into the firewall with a rate of 100 packets per second. Open the XCAP tool. Right-click Interfaces and click Refresh interfaces to refresh the NIC list. Right-click the NIC used to send packets and enable it. Right-click Packet group and click Create packet group. Name the packet group. Click "+" to create a packet and name it. Double-click the created packet. Set Destination MAC to the MAC address of interface G1/4/1. Click Next. Set Destination address to 1.1.1.1, Source address to the default value, and Protocol to TCP. In Flags, select only SYN. Click Save and then the close button. Set Interval to 10 ms. Select Loop. Select the correct occupied interface. Click the button to start sending the packet group. On the firewall, expected result 1 is obtained. - Click Monitor > Security Logs > Flood Attack Logs. You can check the flood attack logs. - Click Monitor > Statistics > Attack Defense Statistics > Attacks. You can check the flood attack statistics. |  |
| ☐ Passed | ☐ Failed |

  - ARP inspection

| Test Item Name: ARP inspection |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify the ARP attack detection function targeted a fixed source MAC address. |  |
| Test Topology: |  |
| Test Process: - Set the IP address of firewall interface G1/4/1 to 1.1.1.1. Add the interface to the Trust zone. Configure the security policy to allow the access of traffic prior to the Trust and Local zones. - Enable the ARP attack detection function targeted a fixed source MAC address and select the filtering mode. - Set the threshold for ARP attack detection implemented on a fixed source MAC address to 30. - Set the aging time of ARP entry for attack detection targeted the fixed source MAC address to 60s. - Build ARP packets using the XCAP tool. Send the packets to the firewall with a rate of 100 packets per second. Result 1 is expected. |  |
| Expect Results: - When the ARP packets sent from the fixed source MAC address exceed the threshold configured on the firewall, the firewall filters the ARP packets and adds the MAC address to the arp source-mac entry with the aging time set to 60s. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Interface Configuration > Interfaces. Configure the IP address and security zone for G1/4/1. View the MAC address of interface G1/4/1. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Local zone. - Enable the ARP attack detection function targeted a fixed source MAC address and select the filtering mode (not supported by the web). - Set the threshold for ARP attack detection implemented on a fixed source MAC address to 30 (not supported by the web). - Set the aging time of ARP entry for attack detection targeted the fixed source MAC address to 60s (not supported by the web). Configure as follows: arp source-mac filter arp source-mac threshold 30 arp source-mac aging-time 60 - Build ARP packets using the XCAP tool. Inject the packets into the firewall with a rate of 100 packets per second. Open the XCAP tool. Right-click Interfaces and click Refresh interfaces to refresh the NIC list. Right-click the NIC used to send packets and enable it. Right-click Packet group and click Create packet group. Name the packet group. Double-click the created packet. Set Destination MAC to the MAC address of interface G1/4/1, and Protocol to ARP. Click Next. Set Destination IP/protocol address to 1.1.1.1. Click Next. Click Save and then the close button. Set Interval to 10 ms. Select Loop. Select the correct occupied interface. Click the button to start sending the packet group. Display arp source-mac on firewall, expected result 1 is obtained. Display arp source-mac on firewall, you can view the MAC address is added to the arp source-mac entry. Command: display arp source-mac interface GigabitEthernet 1/4/1 |  |
| ☐ Passed | ☐ Failed |

  - RADIUS Authentication

| Test Item Name: RADIUS Authentication |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that the firewall supports RADIUS authentication. |  |
| Test Topology: |  |
| Test Process: - Add interface G1/0/0 of the firewall to the Management zone. - Configure the firewall to implement RADIUS-based authentication and authorization for users login through SSH connections. - On the RADIUS server, configure the user hello@bbb, password 123456, and access policy. - On PC2, log in to the firewall through an SSH connection with user name hello@bbb and password 123. Result 1 is expected. - On PC2, log in to the firewall through an SSH connection with user name hello@bbb and password 123456. Result 2 is expected. |  |
| Expect Results: - The client fails to log in to the firewall. On the RADIUS server, choose User > User Access Log > Device User AuthN Log to view the authentication failure log. - The log shows that the client successfully logs in to the firewall through the SSH connection and has the super administrator permissions. On the RADIUS server, choose User > User Access Log > Device User AuthN Log to view the successful authentication log. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Links > VLANs. Configure a VLAN for the firewall. - Click the Network tab. Choose Interface Configuration > Interfaces. Create VLAN interface 2, add it to the Trust zone. Set the interface to the bridge mode. Configure the same VLAN for two interfaces. Configure the same VLAN for two interfaces G1/4/1 and G1/4/2. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Local zone. - Enable the SSH login function of the firewall. Click Network > Services > SSH. Enable the SSH function and service. - Create a local key pair. Click Objects > Public Key Management > Local Key Pairs, and click Create. Create the RSA key pair for SSH login. - Create a RADIUS scheme. Click the Objects > User > Authentication > RADIUS. Create a RADIUS scheme named rad. Create a primary authentication server. Set IP address to 192.168.207.57, Port to 1812, and Shared key to expert. - Configure the ISP domain. Click Objects > User > Authentication > ISP Domains. Create an ISP domain. Set Access types to Login. Select RADIUS for Authentication methods and Authorization methods with RADIUS scheme set to rad. Set Accounting methods to None. Click Advanced settings. Set Default ISP domain to bbb. - Configure the AAA authentication mode for VTY logins. Build the RADIUS server on the iMC. - Add an access device. Log in to the iMC. Click the User tab. On the navigation tree, choose User Access Policy > Access Device Management > Modify Access Device. The access device configuration page is displayed. Click Add. The page for adding an access device is displayed. - Set Authentication Port to 1812 and Accounting Port to 1813. - Set Service Type to Unlimited. - Set Access Device Type to H3C (General). - Set Shared Key used for authentication and accounting interaction with the firewall to expert. - Select or manually add the access device with the IP address 192.168.206.241. - Set other parameters to default values. - Click OK. - Add a device user. Click the User tab. On the navigation tree, choose Access User > All Access Users. The All Access Users page is displayed. Click Add. The Add Access User page is displayed. - Set Account Name to hello@bbb) and User Password (123456). - Add the IP address of the managed device, ranging from 192.168.206.241 to 192.168.206.241. - Set Login Type to SSH. - Set Role Name to network-admin. - Click OK. - Log in to the device over the client through an SSH connection. - On the client, establish an SSH connection to address 192.168.206.241 with the user name set to hello@bbb and password to 123. Login failed： - On the client, establish an SSH connection to address 192.168.206.241 with the user name set to hello@bbb and password to 123456. - On the RADIUS server, choose User > User Access Log > Device User AuthN Log to view the successful authentication log. |  |
| ☐ Passed | ☐ Failed |

  - TACACS Authentication

| Test Item Name: TACACS Authentication |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Purpose: Verify that the firewall supports TACACS authentication. |  |
| Test Topology: |  |
| Test Process: - Add interface G1/0/0 of the firewall to the Management zone. - Configure the firewall to implement HWTACACS-based authentication, authorization, and accounting for users who log in to the firewall through SSH connections. - On the HWTACACS server, configure the user hello@ccc, password 123456, and access policy. - On the client, log in to the firewall through an SSH connection with user name hello@ccc and password 123. Expected result 1 is obtained. - On the client, log in to the firewall through an SSH connection with user name hello@ccc and password 123456. Expected result 2 is obtained. |  |
| Expect Results: - The client fails to log in to the firewall. On the HWTACACS server, click the User tab, and choose Device User > Log Management > Authentication Logs. A log indicating authentication failure exists. - The log shows that the client successfully logs in to the firewall through the SSH connection and has the super administrator permissions. On the HWTACACS server, click the User tab, and choose Device User > Log Management > Authentication Logs. A log indicating successful authentication exists. |  |
| Actual Result： - On the top navigation bar, click Network. From the left navigation pane, select Links > VLANs. Configure a VLAN for the firewall. - Click the Network tab. Choose Interface Configuration > Interfaces. Create VLAN interface 2, add it to the Trust zone. Set the interface to the bridge mode. Configure the same VLAN for two interfaces. Configure the same VLAN for two interfaces G1/4/1 and G1/4/2. - Click Policies > Security Policies > Security Policies. Configure the security policy between Trust zone and Local zone. - Enable the SSH login function of the firewall. Click Network > Services > SSH. Enable the SSH function and service. - Create a local key pair. Click Objects > Public Key Management > Local Key Pairs, and click Create. Create the RSA key pair for SSH login. - Create the HWTACACS scheme. - Create HWTACACS scheme tacacs. - Configure the primary authentication server. Set IP address to 192.168.207.57, Port to 49, and Shared key to expert. - Configure the primary authorization server. Set IP address to 192.168.207.57, Port to 49, and Shared key to expert. - Configure the primary accounting server. Set IP address to 192.168.207.57, Port to 49, and Shared key to expert. Configure as follows: [Firewall]hwtacacs scheme tacacs Create a new HWTACACS scheme. [Firewall-hwtacacs-tacacs]primary authentication 192.168.206.79 key simple expert [Firewall -hwtacacs-tacacs]primary authorization 192.168.206.79 key simple expert [Firewall -hwtacacs-tacacs]primary accounting 192.168.206.79 key simple expert - Configure the ISP domain. - Create an ISP domain. Set Access types to Login. Select HWTACACS for Authentication methods, Authorization methods and Accounting methods with HWTACACS scheme set to tacacs. Configure as follows: [Firewall]domain ccc [Firewall -isp-ccc] authentication login hwtacacs-scheme tacacs [Firewall -isp-ccc] authorization login hwtacacs-scheme tacacs [Firewall -isp-ccc] accounting login hwtacacs-scheme tacacs Specify the ISP domain example as the default domain. [Firewall] domain default enable example - Configure the AAA authentication mode for VTY logins. [Firewall]line vty 0 63 [Firewall-line-vty0-63]authentication-mode scheme Build the HWTACACS server using the iMC. Add an access device. - Log in to the iMC. Click the User tab. On the navigation tree, choose Device User Policy > Device Management. The access device configuration page is displayed. Click Add. The page for adding an access device is displayed. - Set and confirm the shared key expert. - Set Authentication Port to 49. - Select or manually add the access device with the IP address 192.168.206.241 and device name firewall1. - Set other parameters to default values. - Click OK. - Click the User tab. On the navigation tree, choose Device User Policy > Authentication Command > Shell Profiles. On the Shell Profiles page, click Add. The Add Shell Profile page is displayed. - Set Shell Profile Name. - Set Privilege Level to 15. - Click OK. - Click the User tab. On the navigation tree, choose Device User Policy > Authorization Policies > Modify Authorization Policy. The Authorization Policy Info page is displayed. Click Add. The Access Authorization page is displayed. - Set Authorization Policy Name to 1. - Click Add. Set Shell Profile to 1 and Authorization Command Set to Unlimited. - Click OK. - Configure the device user. Click the User tab. On the navigation tree, choose Device User > All Device Users. The device user configuration page is displayed. Click Add. The Add Device User page is displayed. - Set Account Name to hello@ccc and Login Password to 123456. - Select 1 for User Authorization Policy. - Click OK. - Log in to the device over the client through an SSH connection. On the client, establish an SSH connection to address 192.168.207.193 with the user name set to hello@ccc and password to 123. On the client, establish an SSH connection to address 192.168.207.193 with the user name set to hello@ccc and password to 123456. Check the authentication logs on IMC: |  |
| ☐ Passed | ☐ Failed |

Appendix 1. Testing Protocol

| Number Protocol |  |
| --- | --- |
|  | YYYY.MM.DD / VENDOR-MODEL-sequence number |

This Protocol is based on the results of the sample testing equipment:

| The manufacturer's name, country | H3C,China |
| --- | --- |
| The equipment model |  |
| The serial number of equipment |  |

The composition of the test participants

By the Customer

Name, Position, Signature

Name, Position, Signature

Name, Position, Signature

Name, Position, Signature

Name, Position, Signature

On the part of the Participant / Supplier

Name, Position, Signature

Name, Position, Signature

Name, Position, Signature

Name, Position, Signature

Name, Position, Signature

The protocol is signed by the customer and participant / supplier, and sent in a scanned form by producer.

Equipment sample test results:

| All requirements are met requirements | Need to improveme |
| --- | --- |
| Signature: |  |
| Date: |  |