H3C WLAN

Test Procedures

Copyright © 2021 New H3C Technologies Co., Ltd. All rights reserved.

No part of this manual may be reproduced or transmitted in any form or by any

means without prior written consent of New H3C Technologies Co., Ltd.

The information in this document is subject to change without notice.

## Statement

This Test procedure is only used to describe possible security issues. Without the permission of H3C Security, the content of this test procedure may not be modified or added or deleted, and it may not be used for commercial purposes in any way. Any direct or indirect consequences and losses caused by the dissemination and use of the information provided in this security bulletin shall be the responsibility of the user himself. The New H3C Security and the author of the security bulletin shall not bear any responsibility for this. The New H3C Security has the right to modify and interpret this test procedure. If you want to reprint or disseminate this test procedure, you must ensure the integrity of this test procedure, including all content such as the copyright statement.

Revision Record

| Date | Revision Version | Sec No. | Change Description | Author |
| --- | --- | --- | --- | --- |

Contents

Statement	1

1	Performance test	4

1.1	Layer 2 forwarding performance	4

1.2	MAC address table capacity	15

1.3	Multicast Group Capacity	23

2	Layer 2 protocol	29

2.1	STP	29

2.2	RSTP	33

2.3	MSTP	37

2.4	VLAN specification	42

2.5	Isolate group	50

2.6	SPAN	55

2.7	RSPAN	59

2.8	LLDP	71

2.9	LACP (Link backup)	73

2.10	LACP (Load sharing)	76

2.11	Dynamic MAC limit	82

2.12	Jumbo frame	91

2.13	Ethernet auto negotiation	95

2.14	QinQ	97

3	Layer 3 protocol	102

3.1	DHCP	102

3.2	OSPF	107

3.3	OSPFv3 authentication	109

3.4	BFD for VRRP	111

3.5	Broadcast Suppression	117

3.6	Multicast Suppression	120

3.7	Unicast Suppression	126

3.8	IGMP V1/V2/V3	130

4	Network management	155

4.1	Authentication to CLI	155

4.2	RADIUS authentication	161

4.3	TACACS +	169

4.4	802.1X authentication	177

4.5	Syslog Server	183

4.6	FTP backup	184

4.7	TFTP backup	188

4.8	SNMPv2c	190

4.9	SNMP Traps	192

5	QACL	196

5.1	L2 ACL	196

5.2	L3 ACL	201

5.3	Traffic Classification	207

5.4	QoS	225

5.5	SP+WRR queuing	238

- Performance test

  - Client access performance with 80 MHz bandwidth

| Test Item Name: Client access performance with 80 MHz bandwidth |  |
| --- | --- |
| Work Hour: 180 min |  |
| Test Topology: |  |
| Test Process: - Set up an AC+fit AP network. - On the AC, configure the 5 GHz radio on the AP to operate in 802.11ac mode, set the bandwidth to 80 MHz, and set the SSID to Office-1. - Connect the client to SSID Office-1, and display the transmission speed of the client. Result 1 is expected. - Add 15 pairs of High_Performance_Throughput.scr from the PC to the client, and run the test for 5 minutes. Run this test three times. Optional: - Change the radio mode to 802.11ax, and repeat steps 3 and 4. - Compare the test results for the two radio modes. Result 2 is expected. |  |
| Expect Results: - The client can access SSID Office-1, with a negotiated rate of 866 Mbps. - 802.11ax mode provides higher performance than 802.11ac mode. |  |
| Actual Result： - The configuration on SW is the same as test 3.1. - Configure Smartrate-Ethernet 1/0/14 that connects the switch to the PC as a trunk port, configure the PVID as VLAN 200, and assign the port to VLAN 200. - Configure the wired network card on the PC to automatically obtain the IP address - The configuration of the former part on AC is the same as 3.1 from step 8 to step 13. - Configure wireless services: Create service template 1 and enter its view. Configure the SSID as office-1 and enable local forwarding for traffic in VLAN 200.Enable the service template. - Configure the AP: In radio 1 of WA6620X. Set the channel to 52, the power to 8. Enable the ldpc and set the fast-forwarding level to 3 (For WIFI5 the level is 3 and for WIFI6 the level is 6).Bind service template 1 and VLAN 200 to radio 1. Set the bandwidth to 80MHz. Enable radio 1. - Use telnet to login to AP. The default password is h3capadmin - Modify the interface connected to switch on AP to trunk and permit vlan 200. Option：Turn off channel detection： - If the test object is a mobile terminal, select 4. The process of throughput performance test is to control the Endpiont of wired and wireless terminals to send data through ixchariot server. The device throughput is counted by ixchariot server. - Placement of AP and terminal： - Use STA access to office-1.Result 1 is expected.You can see that the negotiation rate of the mobile phone in 802.11ac mode is 866Mbps. - If the receiving rate is not 866Mbps, wait a while and display wlan client verbose again. - Run endpoint.exe on the PC and STA.On PC: - You can check whether endpoint is running on task manager - On STA:Open the application IxChariot EP. - Run ixchariot on PC , and click "add pair" button after startup - Set the source and destination of traffic, where "endpoint1" is the source address, "endpoint2" is the destination address, the source is PC, and the destination is client device - Select the script as high_ Performance_ Throughput.scr - Modify the number of data streams to 15. Right click the stream, choose "Replicate". - The replication count is 14. - Right click in the blank space and click the "graph configuration" option to select "graph content" as "group": - Set running options: Set the test time to 5 minutes. Remove the "stop run on initialization failure" option. Change the value of stop test after 1 running pairs fail to 10 - Click the "run" button to start the performance test - After the traffic transmission is completed, click Throughput to view the average throughput - Run this test three times. - We can see that the average throughput is around 720Mbps. - Optional：Change the radio mode to 802.11ax, and repeat these steps. - The negotiation speed is about 1.2gbps - Click the "run" button to start the performance test. - Run this test three times. - We can see that the average throughput is around 900 Mbps. If there is no other signal interference, the throughput can reach 930 Mbps |  |
| ☐ Passed | ☐ Failed |

  - Client access performance with 20 MHz bandwidth at 2.4 GHz band

| Test Item Name: Client access performance with 20 MHz bandwidth at 2.4 GHz band |  |
| --- | --- |
| Work Hour: 180 min |  |
| Test Topology: |  |
| Test Process: - Set up an AC+fit AP network. - On the AC, configure the 2.4 GHz radio on the AP to operate in 802.11gn mode, set the bandwidth to 20 MHz, and set the SSID to Office-1. - Connect the client to SSID Office-1, and display the transmission speed of the client. Result 1 is expected. - Add 15 pairs of High_Performance_Throughput.scr from the PC to the client, and run the test for 5 minutes. Run this test three times. - Optional: - Change the radio mode to 802.11gax, and repeat steps 3 and 4. - Compare the test results for the two radio modes. Result 2 is expected. |  |
| Expect Results: - The client can access SSID Office-1, with a negotiated rate of 144Mbps. - 2802.11gax mode provides higher performance than 802.11gn mode. |  |
| Actual Result： Actual Result： - The configuration on SW is the same as test 3.2. - Configuration on AC: The configuration of the former part on AC is the same as 3.1 from step 8 to step 13. - Configure the AP: In radio 2 of WA6620X. Choose a channel with less occupancy, you can view the channel status on the AP. Set the power to 8. Enable the ldpc and set the fast-forwarding level to 3 (For WIFI5 the level is 3 and for WIFI6 the level is 6). Bind service template 1 and VLAN 200 to radio 1. Set the bandwidth to 80MHz respectively. Enable radio 1. - Configuration on SW: Telnet to AP on AC to check channel status： The default password is h3capadmin - The occupancy rate of channel 13 is a little high currently, so it is not recommended to use. Check the occupancy rate by changing the channel - Configuration on AP: Use telnet to login to AP. The default password is h3capadmin - Modify the interface connected to switch on AP to trunk and permit vlan 200. - If the test object is a mobile terminal, select 4. - The process of throughput performance test is to control the Endpiont of wired and wireless terminals to send data through ixchariot server. The device throughput is counted by ixchariot server. - Placement of AP and terminal： - The aerial is usually on the head of the phone. Therefore, put the head of the mobile phone at the same height as the two 5G interface of the AP. - Use STA access to office-1.Result 1 is expected. - You can see that the negotiation rate of the mobile phone in 802.11ac mode is 144Mbps. - Run endpoint.exe on the PC and STA.On PC: - You can check whether endpoint is running on task manager - On STA:Open the application IxChariot EP. - Run ixchariot on PC , and click "add pair" button after startup - Set the source and destination of traffic, where "endpoint1" is the source address, "endpoint2" is the destination address, the source is PC, and the destination is client device - Select the script as high_ Performance_ Throughput.scr - Modify the number of data streams to 15. Right click the stream, choose "Replicate". - The replication count is 14. - Right click in the blank space and click the "graph configuration" option to select "graph content" as "group": - Set running options: Set the test time to 5 minutes. Remove the "stop run on initialization failure" option. Change the value of stop test after 1 running pairs fail to 10 - Click the "run" button to start the performance test - After the traffic transmission is completed, click Throughput to view the average throughput - Run this test three times. We can see that the average throughput is around 110Mbps. - Optional：Change the radio mode to 802.11gax, and repeat these steps. The negotiation speed is 286 Mbps - Click the "run" button to start the performance test. - Run this test three times. - We can see that the average throughput is around 210Mbps. |  |
| ☐ Passed | ☐ Failed |

- Function testing

  - Configuring an AP to associate with the AC at Layer 2

| Test Item Name: Configuring an AP to associate with the AC at Layer 2 |  |
| --- | --- |
| Work Hour: min |  |
| Test Topology: |  |
| Test Process: - Connect the network. AC acts as a DHCP server to assign IP addresses to the AP, client, and host, and the Layer 2 switch supplies power to the AP. Configure centralized forwarding. - Add an AP template to the AC, enable the DHCP server, assign an address to the AP. - Connect the AP to the network and wait for the AP to go online. The result 1 is expected. |  |
| Expect Results: - After the AP comes online, the Discovery type field is displayed as DHCP in the output from the display wlan ap all command. |  |
| Actual Result： - Configuring the AC. Configure DHCP: # Enable DHCP. <AC> system-view [AC] dhcp enable # Create DHCP address pool 1 to assign an IP address to the AP, and specify subnet 192.168.201.0/24 in the DHCP address pool. [AC] dhcp server ip-pool 1 [AC-dhcp-pool-1] network 192.168.201.0 mask 255.255.255.0 # Specify the gateway address as 192.168.201.1 in the DHCP address pool. [AC-dhcp-pool-1] gateway-list 192.168.201.1 [AC-dhcp-pool-1] quit # Create DHCP address pool 2 to assign an IP address to the client, and specify subnet 192.168.202.0/24 in the DHCP address pool. [AC] dhcp server ip-pool 2 [AC-dhcp-pool-2] network 192.168.202.0 mask 255.255.255.0? # Specify the gateway address as 192.168.202.1 in the DHCP address pool. [AC-dhcp-pool-2] gateway-list 192.168.202.1 [AC-dhcp-pool-2] quit # Create DHCP address pool 3 to assign an IP address to the host, and specify subnet 192.168.203.0/24 in the DHCP address pool. [AC] dhcp server ip-pool 3 [AC-dhcp-pool-3] network 192.168.203.0 mask 255.255.255.0 # Specify the gateway address as 192.168.203.1 in the DHCP address pool. [AC-dhcp-pool-3] gateway-list 192.168.203.1 [AC-dhcp-pool-3] quit - Configure AC interfaces: # Create VLAN 100 and VLAN-interface 100, and assign an IP address to the VLAN interface. The AC will use this IP address to establish a CAPWAP tunnel with the AP. [AC] vlan 100 [AC-vlan100] quit [AC] interface vlan-interface 100 [AC-Vlan-interface100] ip address 192.168.201.1 255.255.255.0 [AC-Vlan-interface100] quit # Create VLAN 200 and VLAN-interface 200, and assign an IP address to the VLAN interface. VLAN 200 will be used for client access. [AC] vlan 200 [AC-vlan200] quit [AC] interface vlan-interface 200 [AC-Vlan-interface200] ip address 192.168.202.1 24 [AC-Vlan-interface200] quit # Create VLAN 300 and VLAN-interface 300, and assign an IP address to the VLAN interface. The AC will use this IP address to associate with the host. [AC] vlan 300 [AC-vlan300] quit [AC] interface vlan-interface 300 [AC-Vlan-interface300] ip address 192.168.203.1 24 [AC-Vlan-interface300] quit # Configure GigabitEthernet 1/0/1 that connects the AC to the switch as a trunk port, remove the port from VLAN 1, and assign the port to VLANs 100, 200, and 300. [AC] interface gigabitethernet 1/0/1 [AC-GigabitEthernet1/0/1] port link-type trunk [AC-GigabitEthernet1/0/1] undo port trunk permit vlan 1 [AC-GigabitEthernet1/0/1] port trunk permit vlan 100 200 300 # Configure the PVID as VLAN 100. [AC-GigabitEthernet1/0/1] port trunk pvid vlan 100 [AC-GigabitEthernet1/0/1] quit - Configure wireless services: # Create service template 1 and enter its view. [AC] wlan service-template 1 # Configure the SSID as service. [AC-wlan-st-1] ssid service # Enable the service template. [AC-wlan-st-1] service-template enable [AC-wlan-st-1] quit - Configure the AP: # Create manual AP officeap, and specify the AP model and serial ID. [AC] wlan ap officeap model WA560-WW [AC-wlan-ap-officeap] serial-id 219801A1NM8182032235 # Bind service template 1 and VLAN 200 to radio 1. [AC-wlan-ap-officeap] radio 1 [AC-wlan-ap-officeap-radio-1] service-template 1 vlan 200 # Enable radio 1. [AC-wlan-ap-officeap-radio-1] radio enable [AC-wlan-ap-officeap-radio-1] return - Configuring the Layer 2 switch # Create VLANs 100, 200, and 300. The switch will use VLAN 100 to forward packets between the AC and the AP. VLAN 200 will be used for client access. VLAN 300 will be used for host access. <L2 switch> system-view [L2 switch] vlan 100 [L2 switch-vlan100] quit [L2 switch] vlan 200 [L2 switch-vlan200] quit [L2 switch] vlan 300 [L2 switch-vlan300] quit # Configure GigabitEthernet 1/0/1 that connects the switch to the AC as a trunk port, configure the PVID as VLAN 100, and assign the port to VLANs 100, 200, and 300. [L2 switch] interface gigabitEthernet 1/0/1 [L2 switch-GigabitEthernet1/0/1] port link-type trunk [L2 switch-GigabitEthernet1/0/1] port trunk permit vlan 100 200 300 [L2 switch-GigabitEthernet1/0/1] port trunk pvid vlan 100 [L2 switch-GigabitEthernet1/0/1] quit # Configure GigabitEthernet 1/0/2 that connects the switch to the AP as a trunk port, remove the port from VLAN 1, configure the PVID as VLAN 100, and assign the port to VLANs 100 and 200. [L2 switch] interface gigabitEthernet 1/0/2 [L2 switch-GigabitEthernet1/0/2] port link-type trunk [L2 switch-GigabitEthernet1/0/2] undo port trunk permit vlan 1 [L2 switch-GigabitEthernet1/0/2] port trunk permit vlan 100 200 [L2 switch-GigabitEthernet1/0/2] port trunk pvid vlan 100 # Enable PoE on GigabitEthernet 1/0/2. [L2 switch-GigabitEthernet1/0/2] poe enable [L2 switch-GigabitEthernet1/0/2] quit # Configure GigabitEthernet 1/0/3 that connects the switch to the host as an access port, and assign the port to VLAN 300. [L2 switch] interface gigabitEthernet 1/0/3 [L2 switch-GigabitEthernet1/0/3] port access vlan 300 [L2 switch-GigabitEthernet1/0/3] quit - After the AP comes online, the Discovery type field is displayed as DHCP in the output from the display wlan ap all command. |  |
| ☐ Passed | ☐ Failed |

  - Configuring an AP to associate with the AC at Layer 3 (DHCP option 43)

| Test Item Name: Configuring an AP to associate with the AC at Layer 3 (DHCP option 43) |  |
| --- | --- |
| Work Hour: min |  |
| Test Topology: |  |
| Test Process: - Add an AP template to the AC, enable the DHCP server, assign an address to the AP, and deliver the option 43 field. - Connect the AP to the network and wait for the AP to go online. The result 1 is expected. |  |
| Expect Results: - After the AP comes online, the Discovery type field is displayed as DHCP in the output from the display wlan ap name ap1 verbose command. |  |
| Actual Results: - Configure the switch <Switch> system-view [Switch] dhcp enable [Switch] dhcp server ip-pool 1 [Switch-dhcp-pool-1] network 10.0.0.0 mask 255.255.255.0 [Switch-dhcp-pool-1] gateway-list 10.0.0.254 [Switch-dhcp-pool-1] option 43 hex 800700000174040002 # Configure the interface connected to the AC. [Switch]vlan 4 [Switch] interface vlan-interface 4 [Switch-Vlan-interface4] ip address 116.4.0.50 255.255.255.0 [Switch-Vlan-interface4] quit [Switch] interface gigabitethernet 1/0/1 [Switch-GigabitEthernet1/0/1] port access vlan 4 [Switch-GigabitEthernet1/0/1] quit # Configure the interface connected to the AP. [Switch]vlan 10 [Switch] interface vlan-interface 10 [Switch-Vlan-interface10] ip address 10.0.0.254 255.255.255.0 [Switch-Vlan-interface10] quit [Switch] interface gigabitethernet 1/0/2 [Switch-GigabitEthernet1/0/2] port access vlan 10 [Switch-GigabitEthernet1/0/2]poe enable [Switch-GigabitEthernet1/0/2] quit - Configure a service template and a manual AP on the AC [AC]vlan 4 [AC] interface vlan-interface 4 [AC-Vlan-interface4] ip address 116.4.0.2 255.255.255.0 [AC-Vlan-interface4] quit [AC]ip route-static 10.0.0.0 24 116.4.0.50 [AC] interface gigabitethernet 1/0/1 [AC-GigabitEthernet1/0/1] port access vlan 4 [AC-GigabitEthernet1/0/1] quit [AC] wlan service-template 1 [AC-wlan-st-1] ssid h3c-1 vlan 4 [AC-wlan-st-1] service-template enable [AC-wlan-st-1]quit [AC] wlan ap ap1 model WA510H-WW [AC-wlan-ap-ap1] serial-id 219801A1NS7D8172A007 [AC-wlan-ap-ap1] radio 1 [AC-wlan-ap-ap1-radio-1]radio enable [AC-wlan-ap-ap1-radio-1]service-template 1 vlan 4 [AC-wlan-ap-ap1] quit - After the AP comes online, the Discovery type field is displayed as DHCP in the output from the display wlan ap name ap1 verbose command. |  |
| ☐ Passed | ☐ Failed |

  - Local forwarding

| Test Item Name: Local forwarding |  |
| --- | --- |
| Work Hour: min |  |
| Test Topology: |  |
| Test Process: - Add an AP template to the AC, enable the DHCP server, assign an address to the AP, and deliver the option 43 field. - Connect the AP to the network and wait for the AP to go online. The result 1 is expected. |  |
| Expect Results: - AP comes online. - Verify that client traffic is forwarded by the AP. |  |
| Actual Result: - Create a .txt file named apcfg.txt and enter the following content, upload the file to the AC. # system-view vlan 200 quit interface GigabitEthernet 1/0/1 port link-type trunk port trunk permit vlan 200 # - Configure AC 1. Create VLAN 100 and VLAN-interface 100, and assign an IP address to the VLAN interface. The AC will use this IP address to establish a CAPWAP tunnel with the AP. <AC> system-view [AC] vlan 100 [AC-vlan100] quit [AC] interface vlan-interface 100 [AC-Vlan-interface100] ip address 192.1.1.1 16 [AC-Vlan-interface100] quit - Configure wireless services: Create service template 1 and enter its view. # [AC] wlan service-template 1 # Configure the SSID as service. [AC-wlan-st-1] ssid service # Enable local forwarding for traffic in VLAN 200. [AC-wlan-st-1] client forwarding-location ap vlan 200 # Enable the service template. [AC-wlan-st-1] service-template enable [AC-wlan-st-1] quit - Configure the AP: Create manual AP officeap, and specify the AP model and serial ID. # [AC] wlan ap officeap model WA560-WW [AC-wlan-ap-officeap] serial-id 219801A1NM8182032235 Bind service template 1 and VLAN 200 to radio 2. # [AC-wlan-ap-officeap] radio 2 [AC-wlan-ap-officeap-radio-2] service-template 1 vlan 200 [AC-wlan-ap-officeap-radio-2] radio enable [AC-wlan-ap-officeap-radio-2] quit - Deploy configuration file apcfg.txt to AP officeap. [AC-wlan-ap-officeap] map-configuration apcfg.txt [AC-wlan-ap-officeap] quit - Configuring the switch Create VLAN 100 and VLAN-interface 100, and assign an IP address to the VLAN interface. The switch will use VLAN 100 to forward packets between the AC and the AP. # <Switch> system-view [Switch] vlan 100 [Switch-vlan100] quit [Switch] interface vlan-interface 100 [Switch-Vlan-interface100] ip address 192.1.1.2 16 [Switch-Vlan-interface100] quit Create VLAN 200 and VLAN-interface 200, and assign an IP address to the VLAN interface. The switch will use VLAN 200 to forward client traffic. # [Switch] vlan 200 [Switch-vlan200] quit [Switch] interface vlan-interface 200 [Switch-Vlan-interface200] ip address 192.2.1.2 24 [Switch-Vlan-interface200] quit Configure GigabitEthernet 1/0/1 that connects the switch to the AC as a trunk port, remove the port from VLAN 1, and set the PVID to VLAN 100. # [Switch] interface GigabitEthernet 1/0/1 [Switch-GigabitEthernet1/0/1] port link-type trunk [Switch-GigabitEthernet1/0/1] undo port trunk permit vlan 1 [Switch-GigabitEthernet1/0/1] port trunk pvid vlan 100 [Switch-GigabitEthernet1/0/1] quit Configure GigabitEthernet 1/0/2 that connects the switch to the AP as a trunk port, remove the port from VLAN 1, set the PVID to VLAN 100, and assign the port to VLANs 100 and 200. # [Switch] interface GigabitEthernet 1/0/2 [Switch-GigabitEthernet1/0/2] port link-type trunk [Switch-GigabitEthernet1/0/2] undo port trunk permit vlan 1 [Switch-GigabitEthernet1/0/2] port trunk permit vlan 100 200 [Switch-GigabitEthernet1/0/2] port trunk pvid vlan 100 Enable PoE on GigabitEthernet 1/0/2. # [Switch-GigabitEthernet1/0/2] poe enable [Switch-GigabitEthernet1/0/2] quit - Configure DHCP: Enable DHCP. # [Switch] dhcp enable Create DHCP address pool vlan100 to assign an IP address to the AP, and specify subnet 192.1.0.0/16 in the DHCP address pool. # [Switch] dhcp server ip-pool vlan100 [Switch-dhcp-pool-vlan100] network 192.1.0.0 mask 255.255.0.0 Exclude IP address 192.1.1.1 from dynamic allocation in the address pool. # [Switch-dhcp-pool-vlan100] forbidden-ip 192.1.1.1 Specify the gateway address as 192.1.1.2 in the DHCP address pool. # [Switch-dhcp-pool-vlan100] gateway-list 192.1.1.2 [Switch-dhcp-pool-vlan100] quit Create DHCP address pool vlan200 to assign IP addresses to clients, and specify subnet 192.2.1.0/24 in the DHCP address pool. # [Switch] dhcp server ip-pool vlan200 [Switch-dhcp-pool-vlan200] network 192.2.1.0 mask 255.255.255.0 # Exclude IP address 192.2.1.1 from dynamic allocation in the address pool. [Switch-dhcp-pool-vlan200] forbidden-ip 192.2.1.1 Specify the gateway address as 192.2.1.2 in the DHCP address pool # [Switch-dhcp-pool-vlan200] gateway-list 192.2.1.2 [Switch-dhcp-pool-vlan200] quit - AP comes online. |  |
| ☐ Passed | ☐ Failed |

  - Channel and Power selection

| Test Item Name: Channel and Power selection |  |
| --- | --- |
| Work Hour: 60 min |  |
| Test Topology: |  |
| Test Process: - Configure 5g RF and 2.4G RF of AP to provide wireless services "WA1" and "WA2" respectively. - Check the working channel and transmitting power of AP, and result 1 is expected. - STA1 is associated with "WA1" and "WA2" respectively, result 2 is expected. - Manually modify the working channel and power of AP, for example, 5g is configured as 149 channel, 2.4G is configured as 1 channel, power is configured as 18dBm and 19dbm respectively, result 3 is expected. - STA1 is associated with "WA1" and "WA2" respectively, result 2 is expected. |  |
| Expect Results: - The AP works at maximum power by default and automatically selects a channel as the working channel. - STA1 and sta2 are online successfully, and can ping the gateway. - The AP works with the configured channel and power. |  |
| Actual Result： - Configure control vlan 600 and service vlan 700 on AC: - Configure DHCP server. - Configure port connected with POE SW as trunk port and configure ip address. - Configure on POE SW, and vlan 600 and vlan 700, configure port connected with AC as trunk port and configure port connected with AP as access port and enable poe - Configure 5g RF and 2.4G RF of AP to provide wireless services "WA1" and "WA2" on AC, respectively. - Check the working channel and transmitting power of AP, and result 1 is expected. - STA1 is associated with "WA1" and "WA2" respectively, result 2 is expected. - Manually modify the working channel and power of AP, for example, 5g is configured as 149 channel, 2.4G is configured as 1 channel, power is configured as 18dBm and 19dbm respectively, result 3 is expected. - STA1 is associated with "WA1" and "WA2" respectively, result 2 is expected. |  |
| ☐ Passed | ☐ Failed |

  - Configuring auto AP

| Test Item Name: Configuring auto AP |  |
| --- | --- |
| Work Hour： min |  |
| Test Purpose: |  |
| Test Topology: |  |
| Test Process: - Configure auto AP function on AC. - Configure Switch |  |
| Expect Results: - After the AP comes online, the AP has established a CAPWAP tunnel with the AC using the display wlan ap all command. |  |
| Actual Result： - Configure interfaces on the AC: Create VLAN 100 and VLAN-interface 100, and assign an IP address to the VLAN interface. The AC will use this IP address to establish a CAPWAP tunnel with the AP. # <AC> system-view [AC] vlan 100 [AC-vlan100] quit [AC] interface vlan-interface 100 [AC-Vlan-interface100] ip address 192.1.0.1 16 [AC-Vlan-interface100] quit Configure GigabitEthernet 1/0/1 that connects the AC and the switch as a trunk port, remove the port from VLAN 1, and assign it to VLAN 100. # [AC] interface gigabitethernet 1/0/1 [AC-GigabitEthernet1/0/1] port link-type trunk [AC-GigabitEthernet1/0/1] port trunk permit vlan 100 [AC-GigabitEthernet1/0/1] undo port trunk permit vlan 1 Set the PVID of GigabitEthernet 1/0/1 to VLAN 100. # [AC-GigabitEthernet1/0/1] port trunk pvid vlan 100 [AC-GigabitEthernet1/0/1] quit Configure DHCP: Enable DHCP. # [AC] dhcp enable Create a DHCP address pool named 1, and specify subnet 192.1.0.0/16 and gateway IP address 192.1.0.1 in the DHCP address pool. # [AC] dhcp server ip-pool 1 [AC-dhcp-pool-1] network 192.1.0.0 16 [AC-dhcp-pool-1] gateway-list 192.1.0.1 [AC-dhcp-pool-1] quit Enable auto AP. # [AC] wlan auto-ap enable Convert auto APs to manual APs automatically after auto APs come online. # [AC] wlan auto-persistent enable - Configuring the switch # Create VLAN 100. The switch will use this VLAN to forward the traffic on the CAPWAP tunnel between the AC and AP. <Switch> system-view [Switch] vlan 100 [Switch-vlan100] quit Configure GigabitEthernet 1/0/1 that connects the switch and the AC as a trunk port, remove the port from VLAN 1, and assign the trunk port to VLAN 100. # [Switch] interface gigabitethernet 1/0/1 [Switch-GigabitEthernet1/0/1] port link-type trunk [Switch-GigabitEthernet1/0/1] undo port trunk permit vlan 1 [Switch-GigabitEthernet1/0/1] port trunk permit vlan 100 Set the PVID of GigabitEthernet 1/0/1 to VLAN 100. # [Switch-GigabitEthernet1/0/1] port trunk pvid vlan 100 [Switch-GigabitEthernet1/0/1] quit # Configure GigabitEthernet 1/0/2 that connects the switch and the AP as an access port. Assign the access port to VLAN 100. [Switch] interface gigabitethernet 1/0/2 [Switch-GigabitEthernet1/0/2] port link-type access [Switch-GigabitEthernet1/0/2] port access vlan 100 # Enable PoE on GigabitEthernet 1/0/2. [Switch-GigabitEthernet1/0/2] poe enable [Switch-GigabitEthernet1/0/2] quit - Verify that the AP has established a CAPWAP tunnel with the AC. [AC] display wlan ap all Check that the AP comes online with mac-address as the AP name |  |
| ☐ Passed | ☐ Failed |

  - Channel bandwidth

| Test Item Name: Channel bandwidth |  |
| --- | --- |
| Work Hour: 60 min |  |
| Test Topology: |  |
| Test Process: - Configure 5g RF and 2.4G RF of AP to provide wireless services "WA1" and "WA2" respectively. - The bandwidth of 5g RF and 2.4G RF channel is 20MHz. - STA1 is associated with "WA1" and "WA2" respectively, result 1 is expected. - Modify the AP channel bandwidth to 40MHz, repeat step 3 and result 2 is expected. - Modify the 5g channel bandwidth of AP to 80MHz, and the STA1 of wireless terminal is associated with "WA1", result 3 is expected. |  |
| Expect Results: - STA1 is online successfully, the terminal information is in 20MHz mode, the rate negotiation is correct, and the gateway can be pinged. - STA1 and sta2 are online successfully, the terminal information is in 40MHz mode, the rate negotiation is correct, and the gateway can be pinged. - STA1 is online successfully, the terminal information is viewed in 80MHz mode, the rate negotiation is correct, and the gateway can be pinged. |  |
| Actual Result： - Configure control vlan 600 and service vlan 700 on AC: - Configure DHCP server. - Configure port connected with POE SW as trunk port and configure ip address. - Configure on POE SW, and vlan 600 and vlan 700, configure port connected with AC as trunk port and configure port connected with AP as access port and enable poe - Configure 5g RF and 2.4G RF of AP to provide wireless services "WA1" and "WA2" respectively. The bandwidth of 5g RF and 2.4G RF channel is 20MHz. - STA1 is associated with "WA1" and "WA2" respectively, result 1 is expected. - Modify the AP channel bandwidth to 40MHz, repeat step 3 and result 2 is expected. - Modify the 5g channel bandwidth of AP to 80MHz, and the STA1 of wireless terminal is associated with "WA1", result 3 is expected. |  |
| ☐ Passed | ☐ Failed |

  - SNMP

| Test Item Name: SNMP |  |
| --- | --- |
| Work Hour: 20 min |  |
| Test Topology: |  |
| Test Process: - Enable SNMP function. - Using a network management system or software (such as MIB Browser), select SNMPv2 and SNMPv3 respectively, and log in to the AC. The result 1 is expected. |  |
| Expect Results: - Can successfully log in to the AC and be able to read and write the corresponding MIB node. |  |
| Actual Result： - Configure snmp, the version is v2c and v3. Configure v3 authentication mode. # snmp-agent snmp-agent local-engineid 800063A280782C2956534500000001 snmp-agent community write private snmp-agent community read public snmp-agent sys-info version v2c v3 snmp-agent usm-user v3 123 user-role network-admin simple authentication-mode md5 123456 - Configure IP address, make sure ac can ping PC vlan 199 # Interface vlan 199 Ip add 192.168.207.199 23 # Interface g1/0/5 Port access vlan 199 # - Login by MIB, result 1 is expected. SNMPv2 SNMPv3 |  |
| ☐ Passed | ☐ Failed |

  - SSH

| Test Item Name: SSH |  |
| --- | --- |
| Work Hour: 20 min |  |
| Test Topology: |  |
| Test Process: - Open the SSH service on the device. - The administrator logs in to the device through SSH, and the result 1 is expected. |  |
| Expect Results: - Can successfully log in to the AC and manage the device. |  |
| Actual Result： - Configure IP address, make sure ac can ping PC Vlan 199 # Interface vlan 199 Ip add 192.168.207.199 23 # Interface g1/0/5 Port access vlan 199 # - Enable ssh server and create local-user and password # local-user admin class manage password simple h3c.com123 service-type ssh authorization-attribute user-role network-admin # ssh server enable - Configure login line and set authentication mode as scheme and add protocol ssh. # Line vty 0 31 authentication-mode scheme protocol inbound SSH user-role network-admin # - Login by ssh, and result 1 is expected. |  |
| ☐ Passed | ☐ Failed |

  - FTP and TFTP

| Test Item Name: FTP and TFTP |  |
| --- | --- |
| Work Hour: 30 min |  |
| Test Topology: |  |
| Test Process: - Start the FTP Server service on the device. - The administrator logs in to the device via FTP, the result 1 is expected. - Start the FTP Server and TFTP Server services on the PC. - The administrator logs in to the server through FTP and TFTP on the AC, and the result 2 is expected. |  |
| Expect Results: - Can log in to AC successfully. - Can successfully log in via FTP or TFTP, and can upload and download files. |  |
| Actual Result： - Configure FTP local-user and enable ftp server # ftp server enable # local-user admin class manage password hash $h$6$BRydS4hO6rPFaZqU$PzVsw00xEGj9+hDMHvXyCMe9252gT4SXS5CaLjPz+yEEmAiZz58laACuJjfLOaumv7I4K2rLmLQvNjbKBchiJA== service-type ftp authorization-attribute user-role network-admin # - Configure IP address, make sure ac can ping PC Vlan 199 # Interface vlan 199 Ip add 192.168.207.199 23 # Interface g1/0/5 Port access vlan 199 # - Login in by ftp, result 1 is expected - Start 3CD as tftp server and login from AC, result 2 is expected |  |
| ☐ Passed | ☐ Failed |

  - CLI & GUI Management

| Test Item Name: CLI & GUI Management |  |
| --- | --- |
| Work Hour: 30 min |  |
| Test Topology: |  |
| Test Process: - Log in to the device through the console port and manage the device through the CLI. The result 1 is expected. - Log in to the device via WEB and download files to the device via WEB, the result 2 is expected. |  |
| Expect Results: - The administrator can manage the device through CLI, such as configuring the device or managing files, etc. - Can login by web and can download files. |  |
| Actual Result： - Login by CLI and result1 is expected. - Configure local user and enable WEB login function. Configure IP address, make sure your PC IP address can ping AC. - Login by WEB, result 2 is expected. |  |
| ☐ Passed | ☐ Failed |

  - Logging Systems

| Test Item Name: 1.1 Logging Systems |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Topology: |  |
| Test Process: - Add a log server on the AC. - Start the log server on the PC and receive the logs. - Shut down and turn on an interface or port on the device, execute command lines, user logs in or out of the device, construct power supply in place or remove, etc. (You can use tftpd or 3CDaemon as the syslog server) |  |
| Expect Results: - Corresponding logs can be received on the log host. |  |
| Actual Result： - Configure info-center loghost on AC # info-center enable info-center loghost 192.168.207.18 # - Configure IP address, make sure ac can ping PC Vlan 199 # Interface vlan 199 Ip add 192.168.207.199 23 # Interface g1/0/5 Port access vlan 199 # - Start 3CD as log server, then execute commands on AC, result1 is expected |  |
| ☐ Passed | ☐ Failed |

  - 802.1x

| Test Item Name: 802.1x |  |
| --- | --- |
| Work Hour: 120 min |  |
| Test Topology: |  |
| Test Process: - Enable jumbo frame forwarding on Port A, Port B, and Port C of Tester. - Port A of Tester sends broadcast packets with the length as 9000(9216) bytes continuously. Result 1 is expected. - Disable jumbo frame forwarding on Port 1 of DUT1. Result 2 is expected. - Enable jumbo frame forwarding on Port 1 of DUT1. Result 1 is expected. |  |
| Expect Results: - Packets sent out Port A of Tester are received on Port B and Port C of Tester. - Packets sent out Port A of Tester are not received on Port B and Port C of Tester. |  |
| Actual Result： - Enable jumbo frame forwarding on Port A, Port B, and Port C of Tester - Port A of Tester sends broadcast packets with the length as 9000(9216) bytes continuously. Result 1 is expected - Disable jumbo frame forwarding on Port 1 of DUT1. Result 2 is expected - Enable jumbo frame forwarding on Port 1 of DUT1. Result 1 is expected |  |
| ☐ Passed | ☐ Failed |

  - Private-PSK local certification

| Test Item Name: Private-PSK local certification |  |
| --- | --- |
| Work Hour: 90 min |  |
| Test Topology: |  |
| Test Process: - Configure the wireless service on the AC, SSID is "WA1". - Add the authentication username and password locally (the username and password are both the terminal's MAC address), etc. - The wireless terminal STA1 associates with "WA1" and the result 1 is expected. |  |
| Expect Results: - STA1 authentication is successfully |  |
| Actual Result： - Configure local authentication domain # domain local-mac authentication lan-access local authorization-attribute idle-cut 15 1024 authentication lan-access local # - Check terminal Mac address. Configure the local user on AC , the user account and password are the terminal MAC address, and there is no hyphen in the middle # local-user f887f16d3dba class network password simple f887f16d3dba service-type lan-access authorization-attribute user-role network-operator # - Authorization-attribute user-role network-operator Configure user name format for local MAC address authentication # mac-authentication user-name-format mac-address without-hyphen lowercase # - Configure the wireless service template, enable MAC authentication and call the corresponding MAC domain, and configure psk authentication # wlan service-template wa1 ssid WA1 vlan 700 akm mode private-psk cipher-suite ccmp security-ie rsn client-security authentication-mode mac mac-authentication domain local-mac service-template enable # - Bind the wireless service template on AC # wlan ap ap3 model WA4320i-X serial-id 210235A1GSC166000297 vlan 1 radio 1 radio enable service-template wa1 vlan 700 radio 2 gigabitethernet 1 gigabitethernet 2 # - Configure control vlan 600 and service vlan 700 on AC # vlan 600 # vlan 700 - Configure DHCP server. # dhcp enable # dhcp server ip-pool vlan600 gateway-list 192.168.60.1 network 192.168.60.0 mask 255.255.255.0 dns-list 192.168.60.1 forbidden-ip 192.168.60.1 # dhcp server ip-pool vlan700 gateway-list 3.3.3.1 network 3.3.3.0 mask 255.255.255.0 forbidden-ip 3.3.3.1 - Configure port connected with POE SW as trunk port and configure ip address. # interface GigabitEthernet1/0/3 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # interface Vlan-interface600 ip address 192.168.60.1 255.255.255.0 # interface Vlan-interface700 ip address 3.3.3.1 255.255.255.0 - Configure on POE SW, and vlan 600 and vlan 700, configure port connected with AC as trunk port and configure port connected with AP as access port and enable poe. # vlan 600 # vlan 700 # interface Smartrate-Ethernet1/0/1 port link-mode bridge port access vlan 600 poe enable # interface Smartrate-Ethernet1/0/11 port link-mode bridge port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # - The user login password is the MAC address of the terminal, result 1 is expected. |  |
| ☐ Passed | ☐ Failed |

  - Private-PSK Remote Authentication

| Test Item Name: Private-PSK Remote Authentication |  |
| --- | --- |
| Work Hour: 120 min |  |
| Test Topology: |  |
| Test Process: - Configure the wireless service on the AC, and the SSID is "WA1". - Configure the Radius server, add the AC as the NAS access device, and authenticate the user name and password (both the user name and password are the terminal's MAC address), etc. - The wireless terminal STA1 associates with "WA1" and has the expected result 1. |  |
| Expect Results: - STA1 authentication is successful, and can ping the gateway. |  |
| Actual Result： - Configure control vlan 600 and service vlan 700 on AC. # vlan 600 # vlan 700 - Configure DHCP server. # dhcp enable # dhcp server ip-pool vlan600 gateway-list 192.168.60.1 network 192.168.60.0 mask 255.255.255.0 dns-list 192.168.60.1 forbidden-ip 192.168.60.1 # dhcp server ip-pool vlan700 gateway-list 3.3.3.1 network 3.3.3.0 mask 255.255.255.0 forbidden-ip 3.3.3.1 - Configure port connected with POE SW as trunk port and configure ip address. # interface GigabitEthernet1/0/3 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # interface Vlan-interface600 ip address 192.168.60.1 255.255.255.0 # interface Vlan-interface700 ip address 3.3.3.1 255.255.255.0 - Configure on POE SW, and vlan 600 and vlan 700, configure port connected with AC as trunk port and configure port connected with AP as access port and enable poe. # vlan 600 # vlan 700 # interface Smartrate-Ethernet1/0/1 port link-mode bridge port access vlan 600 poe enable # interface Smartrate-Ethernet1/0/11 port link-mode bridge port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # - Configure service template, MAC authentication mode, MAC authentication domain, guest VLAN, PSK authentication. # wlan service-template wa1 ssid WA1 vlan 700 akm mode private-psk cipher-suite ccmp security-ie rsn client-security authentication-mode mac mac-authentication domain office service-template enable - Configure RF interface and bind service template. # wlan ap ap3 model WA4320i-X serial-id 210235A1GSC166000297 vlan 1 radio 1 radio enable service-template wa1 vlan 700 radio 2 gigabitethernet 1 gigabitethernet 2 # - Configure radius authentication, configure the IP address, secret key, no domain name and source address of radius message, 192.168.207.57 is IMC server address, 192.168.207.199 is AC address. # radius scheme office primary authentication 192.168.207.57 primary accounting 192.168.207.57 key authentication simple 123456 key accounting simple 123456 user-name-format without-domain nas-ip 192.168.207.199 # - Create an ISP domain named Office1, configure authentication, billing and authorization schemes. # domain office authentication lan-access radius-scheme office authorization lan-access radius-scheme office accounting lan-access radius-scheme office # - Configure the user name and password of MAC address authentication as the user's MAC address without hyphen (the configuration is the default configuration). # mac-authentication user-name-format mac-address without-hyphen lowercase # - Configure IP address, make sure ac can ping IMC. Vlan 199 # Interface vlan 199 Ip add 192.168.207.199 23 # Interface g1/0/5 Port access vlan 199 # - Login in IMC and add route to DHCP server, make sure sta can ping IMC. - Configure radius server, Access device configuration. Log in to the IMC management platform, select the "user" tab, and click [access policy management / access device management / access device configuration] menu item in the navigation tree to enter the access configuration management page. Click the Add button in this page to enter the page of adding access devices. Set the authentication and billing shared key to 123456, and keep the default configuration for others. Select or manually add access devices and add access devices with IP address of 192.168.207.199 - Access policy configuration Select the "user" tab, click [access policy management / access policy management] menu item in the navigation tree, and click the < add > button to create an access policy. · configure the access policy named "office". Other parameters are set to default values, and click the < OK > button to complete the operation. - Add service configuration. Select the "user" tab, click the [access policy management / access service management] menu item in the navigation tree, and click the < add > button to create a service. The configuration service name is "office_ Mac "(the service name here can be arbitrarily named). The default access policy is "office". The others are configured by default. - Increase access users Select the "user" tab, click the [access user management / access user] menu item in the navigation tree, and click the < add > button to add an access user. · click the add user button and enter the user name "111 "and ID" 111 ", click the" OK > button to finish. Configure the account name and password as the client's MAC address " f887f16d3dba ". · check the binding service name "office_ mac”. - Use the terminal to access SSID, and you can see the terminal online on the AC and IMC online users, result 1 is expected. |  |
| ☐ Passed | ☐ Failed |

  - Wireless Access Blacklist

| Test Item Name: Wireless Access Blacklist |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Topology: |  |
| Test Process: - Configure wireless service, SSID is "WA1". - Add STA1 as a blacklist, STA1 attempts to associate with "WA1”. |  |
| Expect Results: - Port 4 can get IP address from DHCP server. - The captured DHCP discover packet contains the Circuit ID sub-option and the Remote ID sub-option of Option 82 |  |
| Actual Result： - Configure the wireless service template. # wlan service-template wa1 ssid WA1 vlan 700 service-template enable # return - Bind the wireless service template on AC. # wlan ap ap3 model WA4320i-X serial-id 210235A1GSC166000297 vlan 1 radio 1 radio enable service-template wa1 vlan 700 radio 2 gigabitethernet 1 gigabitethernet 2 # - Configure control vlan 600 and service vlan 700 on AC. # vlan 600 # vlan 700 - Configure DHCP server. # dhcp enable # dhcp server ip-pool vlan600 gateway-list 192.168.60.1 network 192.168.60.0 mask 255.255.255.0 dns-list 192.168.60.1 forbidden-ip 192.168.60.1 # dhcp server ip-pool vlan700 gateway-list 3.3.3.1 network 3.3.3.0 mask 255.255.255.0 forbidden-ip 3.3.3.1 - Configure port connected with POE SW as trunk port and configure ip address. # interface GigabitEthernet1/0/3 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # interface Vlan-interface600 ip address 192.168.60.1 255.255.255.0 # interface Vlan-interface700 ip address 3.3.3.1 255.255.255.0 - Configure on POE SW, and vlan 600 and vlan 700, configure port connected with AC as trunk port and configure port connected with AP as access port and enable poe. # vlan 600 # vlan 700 # interface Smartrate-Ethernet1/0/1 port link-mode bridge port access vlan 600 poe enable # interface Smartrate-Ethernet1/0/11 port link-mode bridge port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # - STA login , result 1 is expected - Set blacklist, forbidden the mac-address of terminal, can not go to online. # wlan static-blacklist mac-address f887-f16d-3dba # - Login again, result 2 is expected |  |
| ☐ Passed | ☐ Failed |

  - ACL (IP-Address)

| Test Item Name: ACL (IP-Address) |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Topology: |  |
| Test Process: - Configure wireless service, SSID is "WA1". - Add ACL based on IP address, if you can't access STA2 in the network. - STA1 is associated with "WA1" and there is expected result 1. - STA1 ping STA2, there is expected result 2. |  |
| Expect Results: - STA1 goes online successfully and can ping the gateway. - STA1 cannot ping STA2. |  |
| Actual Result： - Configure the wireless service template. # wlan service-template wa1 ssid WA1 vlan 700 service-template enable # return - Bind the wireless service template on AC. # wlan ap ap3 model WA4320i-X serial-id 210235A1GSC166000297 vlan 1 radio 1 radio enable service-template wa1 vlan 700 radio 2 gigabitethernet 1 gigabitethernet 2 # - Configure control vlan 600 and service vlan 700 on AC. # vlan 600 # vlan 700 - Configure DHCP server. # dhcp enable # dhcp server ip-pool vlan600 gateway-list 192.168.60.1 network 192.168.60.0 mask 255.255.255.0 dns-list 192.168.60.1 forbidden-ip 192.168.60.1 # dhcp server ip-pool vlan700 gateway-list 3.3.3.1 network 3.3.3.0 mask 255.255.255.0 forbidden-ip 3.3.3.1 - Configure port connected with POE SW as trunk port and configure ip address. # interface GigabitEthernet1/0/3 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # interface Vlan-interface600 ip address 192.168.60.1 255.255.255.0 # interface Vlan-interface700 ip address 3.3.3.1 255.255.255.0 - Configure on POE SW, and vlan 600 and vlan 700, configure port connected with AC as trunk port and configure port connected with AP as access port and enable poe. # vlan 600 # vlan 700 # interface Smartrate-Ethernet1/0/1 port link-mode bridge port access vlan 600 poe enable # interface Smartrate-Ethernet1/0/11 port link-mode bridge port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # - STA1 associates with "WA1", result 1 is expected. - Create acl , and apply rule, result 2 is expected. # acl advanced 3000 rule 0 deny ip source 3.3.3.0 0.0.0.255 destination 3.3.3.2 0 # interface Vlan-interface700 ip address 3.3.3.1 255.255.255.0 packet-filter 3000 outbound # return |  |
| ☐ Passed | ☐ Failed |

  - ACL (TCP/UDP-Port)

| Test Item Name: ACL (TCP/UDP-Port) |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Topology: |  |
| Test Process: - Configure wireless service, SSID is "WA1". - Add ACL based on TCP/UDP Port filtering, such as unable to access Ftp service on AC in the network. - STA1 is associated with "WA1" and there is expected result 1. - STA1 accesses FTP with expected result 2. |  |
| Expect Results: - STA1 goes online successfully and can ping the gateway. - STA1 cannot access the FTP services on the AC. |  |
| Actual Result： - Configure the wireless service template. # wlan service-template wa1 ssid WA1 vlan 700 service-template enable # return - Bind the wireless service template on AC # wlan ap ap3 model WA4320i-X serial-id 210235A1GSC166000297 vlan 1 radio 1 radio enable service-template wa1 vlan 700 radio 2 gigabitethernet 1 gigabitethernet 2 # - Configure control vlan 600 and service vlan 700 on AC. # vlan 600 # vlan 700 - Configure DHCP server. # dhcp enable # dhcp server ip-pool vlan600 gateway-list 192.168.60.1 network 192.168.60.0 mask 255.255.255.0 dns-list 192.168.60.1 forbidden-ip 192.168.60.1 # dhcp server ip-pool vlan700 gateway-list 3.3.3.1 network 3.3.3.0 mask 255.255.255.0 forbidden-ip 3.3.3.1 - Configure port connected with POE SW as trunk port and configure ip address. # interface GigabitEthernet1/0/3 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # interface Vlan-interface600 ip address 192.168.60.1 255.255.255.0 # interface Vlan-interface700 ip address 3.3.3.1 255.255.255.0 - Configure on POE SW, and vlan 600 and vlan 700, configure port connected with AC as trunk port and configure port connected with AP as access port and enable poe. # vlan 600 # vlan 700 # interface Smartrate-Ethernet1/0/1 port link-mode bridge port access vlan 600 poe enable # interface Smartrate-Ethernet1/0/11 port link-mode bridge port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # - Configure FTP server and local user on AC. # ftp server enable # local-user 111 class manage password simple h3c.com123 service-type ftp authorization-attribute user-role network-operator # Line vty 0 31 authentication-mode scheme user-role network-admin # - STA goes online and can ftp, result 1 is expected. - Add ACL based on TCP Port filtering, such as unable to access Ftp service on PC in the network. result 2 is expected. # acl advanced 3000 rule 0 deny tcp source 3.3.3.4 0 destination 3.3.3.1 0.0.0.255 # interface Vlan-interface 700 ip address 3.3.3.1 255.255.255.0 packet-filter 3000 inbound # |  |
| ☐ Passed | ☐ Failed |

  - DHCP Relay Option82

| Test Item Name: DHCP Relay Option82 |  |
| --- | --- |
| Work Hour：60 min |  |
| Test Topology: |  |
| Test Process: - Enable the automatic AP function on the AC, configure the wireless service "SSID1", and enable the DHCP relay function, add Option82 option content. - Associate STA with "SSID1", adopt automatic address acquisition, and has expected result 1. |  |
| Expect Results: - STA goes online successfully, and can carry Option82 information to the server. |  |
| - Configure DHCP for wlan client, configure route static to AP. # Dhcp enable # vlan 800 # dhcp server ip-pool vlan800 gateway-list 3.3.3.1 network 3.3.3.0 mask 255.255.255.0 dns-list 3.3.3.1 forbidden-ip 3.3.3.1 # interface Vlan-interface800 ip address 4.1.1.1 255.255.255.0 dhcp server apply ip-pool vlan800 # interface GigabitEthernet1/0/17 port access vlan 800 # ip route-static 3.3.3.0 24 4.1.1.2 # - Configure DHCP relayfor wlan client, and add option 82 information. # Vlan 600 vlan 700 vlan 800 # Dhcp enable # interface Vlan-interface700 ip address 3.3.3.1 255.255.255.0 dhcp select relay dhcp relay information circuit-id normal sub-interface-vlan format hex dhcp relay information enable dhcp relay information remote-id string h3c dhcp relay server-address 4.1.1.1 # interface GigabitEthernet1/0/1 port access vlan 800 # interface Vlan-interface800 ip address 4.1.1.2 255.255.255.0 # interface GigabitEthernet1/0/3 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # - Configure DHCP server for ap on AC. # vlan 600 # vlan 700 # dhcp enable # # dhcp server ip-pool vlan600 gateway-list 192.168.60.1 network 192.168.60.0 mask 255.255.255.0 dns-list 192.168.60.1 forbidden-ip 192.168.60.1 # interface GigabitEthernet1/0/3 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # - Configure poe sw. # vlan 600 # vlan 700 # interface Smartrate-Ethernet1/0/1 port link-mode bridge port access vlan 600 poe enable # interface Smartrate-Ethernet1/0/11 port link-mode bridge port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # - Configure AP to provide wireless services "WA1" on AC and bind to ap. # wlan service-template wa1 ssid WA1 vlan 700 service-template enable # wlan ap ap3 model WA4320i-X serial-id 210235A1GSC166000297 vlan 1 radio 1 radio enable service-template wa1 vlan 700 radio 2 gigabitethernet 1 gigabitethernet 2 # - Configure mirroring group to capture option82 packet on AC. # mirroring-group 1 local # interface GigabitEthernet1/0/6 mirroring-group 1 monitor-port # interface GigabitEthernet1/0/17 port access vlan 800 mirroring-group 1 mirroring-port both # - STA go online. - Check capture, result 1 expected. |  |
| ☐ Passed | ☐ Failed |

  - Remote Portal Authentication

| Test Item Name: Remote Portal Authentication |  |
| --- | --- |
| Work Hour：10 min |  |
| Test Topology: |  |
| Test Process: - Configure the wireless service " SSID1 ", then configure Portal function and NAS-ID and other parameters. - Configure the Portal server,add the AC as the NAS access device, and add the authentication user name, password, - STA1 connects to " SSID1 " and uses a browser to access the page http://1.1.1.1 , with the expected result 1 . - Enter the user name and password for Portal authentication and have the expected result 2. |  |
| Expect Results: - WEB page is redirected to Portal authentication interface. - The STA1 Portal authentication is successful, ping to gateway can is ok. |  |
| Actual Result： - Config AC - Configure radius scheme [AC]radius scheme portal [AC-radius-portal]primary authentication 192.168.206.79 key simple 123456 [AC-radius-portal]primary accounting 192.168.206.79 key simple 123456 [AC-radius-portal]user-name-format without-domain [AC-radius-portal]nas-ip 192.168.207.199 - Configure authentication domain [AC-isp-portal]dis this # domain portal authentication portal radius-scheme portal authorization portal radius-scheme portal accounting portal radius-scheme portal # - Configure portal authentication server [H3C]portal server portal [H3C-portal-server-portal]ip 192.168.206.79 key simple 12345678 [H3C-portal-server-portal]quit - Configure portal web server [AC-portal-websvr-portal]dis this # portal web-server portal url http://192.168.206.79:8080/portal # - Configure portal authentication binding wireless service template [AC-wlan-st-re-portal]dis this # wlan service-template re-portal ssid re-portal vlan 2 portal enable method direct portal domain portal portal apply web-server portal service-template enable # - Bind the portal authentication service template to radio [H3C-wlan-ap-ap1-radio-1] service-template portal - Configure IP address group: configure start and end addresses in access policy management portal service management IP address group configuration. The IP address of the client should be included in the starting and ending addresses. The maximum and minimum addresses of the same network segment can be configured. - Add portal device: in access policy management - portal service management - device configuration, select Add to enter the add device information configuration page, fill in the required items, IP address is the IP address of AC. The key here is consistent with the key on the portal authentication server of AC - Access policy management: configure the portal authentication policy, set the access policy name, and default the rest - Configure access service management: set the service name, check the previously configured portal authentication policy, and default the rest - Access user configuration: add access user under user access user management access user, set access user name and password, and check the previously set access service name. - LOG IN |  |
| ☐ Passed | ☐ Failed |

  - Local Portal Authentication

| Test Item Name: Local Portal Authentication |  |
| --- | --- |
| Work Hour: 20 min |  |
| Test Topology: |  |
| Test Process: - Configure the wireless service " SSID1 ". - Open the local Portal function, add authentication user name, password - STA1 connects to " SSID1 " and uses a browser to access the page http://1.1.1.1 , with the expected result 1 . - Enter the user name and password for Portal authentication and have the expected result 2. |  |
| Expect Results: - WEB page is redirected to Portal authentication interface. - The STA1 Portal authentication is successful, ping to gateway is ok, |  |
| Actual Result： - AC configuration Configure local user settings [AC-luser-network-portal1]dis this # local-user portal1 class network password simple 12345678 service-type portal authorization-attribute user-role network-operator # Portal web authentication page configuration [AC-portal-websvr-portal1]dis this # portal web-server portal1 url http://173.13.1.2:2331/portal # Create a local portal web server, enter the local portal web server view, and specify the use of HTTP protocol and client interaction authentication information [AC-portal-local-websvr-http]dis this # portal local-web-server http default-logon-page defaultfile.zip tcp-port 2331 Configure authentication domain # domain local authentication portal local authorization portal local accounting portal local # Configure portal authentication binding wireless service template [AC-wlan-st-local-portal]dis this # wlan service-template local-portal ssid local-portal vlan 2 portal enable method direct portal domain local portal apply web-server portal1 service-template enable # - Log in Customized view |  |
| ☐ Passed | ☐ Failed |

  - Wireless local packet capture

| Test Item Name: Wireless local packet capture |  |
| --- | --- |
| Work Hour: 20 min |  |
| Test Topology: |  |
| Test Process: - Configure the wireless service, SSID is "WA1", turn on the local packet capture function, set the packet capture conditions and the FTP server (such as AC or local PC) for packet storage, etc.. - Configure AC or local PC as FTP server. - STA1 associates with "SSID1", STA1 pings PC1, there is expected result 1. - Start local packet capture on the wireless air interface, and stop the capture after a period of time. - Check the captured message on the FTP server, there is expected result 2. |  |
| Expect Results: - STA1 goes online successfully, you can ping PC1. - The captured message is uploaded to the FTP server, and the captured message can be parsed by software such as wireshark. |  |
| Actual Result： - Configure the wireless service template. # wlan service-template wa1 ssid WA1 vlan 700 service-template enable # return - Bind the wireless service template on AC. # wlan ap ap3 model WA4320i-X serial-id 210235A1GSC166000297 vlan 1 radio 1 radio enable service-template wa1 vlan 700 radio 2 gigabitethernet 1 gigabitethernet 2 # - Configure control vlan 600 and service vlan 700 on AC. # vlan 600 # vlan 700 - Configure DHCP server. # dhcp enable # dhcp server ip-pool vlan600 gateway-list 192.168.60.1 network 192.168.60.0 mask 255.255.255.0 dns-list 192.168.60.1 forbidden-ip 192.168.60.1 # dhcp server ip-pool vlan700 gateway-list 3.3.3.1 network 3.3.3.0 mask 255.255.255.0 forbidden-ip 3.3.3.1 - Configure port connected with POE SW as trunk port and configure ip address. # interface GigabitEthernet1/0/3 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # interface Vlan-interface600 ip address 192.168.60.1 255.255.255.0 # interface Vlan-interface700 ip address 3.3.3.1 255.255.255.0 - Configure on POE SW, and vlan 600 and vlan 700, configure port connected with AC as trunk port and configure port connected with AP as access port and enable poe. # vlan 600 # vlan 700 # interface Smartrate-Ethernet1/0/1 port link-mode bridge port access vlan 600 poe enable # interface Smartrate-Ethernet1/0/11 port link-mode bridge port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # - Add an FTP user abc on the switch, and set its authentication password to 123456. The user role is network admin, and the service type is FTP. # local-user 111 class manage password simple 123456 service-type ftp authorization-attribute user-role network-admin authorization-attribute user-role network-operator # ftp server enable - Open the local packet capture function on Radio 1 of AP WA6620X on AC, specify the filter string as "icmp", send the captured packets to the FTP server with IP address of 192.168.207.199, and save the file name as abcd.pcap , FTP user name is 111, password is 123456, exit the capture process. <AC>packet-capture local ap ap3 radio 1 write url ftp://192.168.207.199/abcd.pcap username 111 password simple 123456 - Use STA to access wireless WiFi and send 1000 Ping packets. Result 1 is expected. <AC>packet-capture stop - Check the files on the switch, you can find abc.pacp. - Send the abc.pacp file on the switch to the computer with wireshark installed via FTP function. - Open abcd.pacp by wireshark on the computer. Result 2 is expected. |  |
| ☐ Passed | ☐ Failed |

  - Wireless access escape

| Test Item Name: Wireless access escape |  |
| --- | --- |
| Work Hour: 45 min |  |
| Test Topology: |  |
| Test Process: - Build a basic wireless environment, and the routes between the devices can be reached. - Configure authentication service (clear, PSK, MAC or 802.1x) on AC, assuming WA1 as wireless service. - Configure WA1 with the same name. - The terminal accesses the wireless service ssid1 and result 1 is expected. - Make the device and authentication server unreachable, trigger escape, result 2 is expected. - Disconnect the connection between AC and AP, trigger escape, result 2 is expected. |  |
| Expect Results: - The terminal can access the network normally, and the communication is normal. - The user can access the released escape service, and the communication is normal. |  |
| Actual Result： - Configure control vlan 600 and service vlan 700 on AC. # vlan 600 # vlan 700 - Configure DHCP server. # dhcp enable # dhcp server ip-pool vlan600 gateway-list 192.168.60.1 network 192.168.60.0 mask 255.255.255.0 dns-list 192.168.60.1 forbidden-ip 192.168.60.1 # dhcp server ip-pool vlan700 gateway-list 3.3.3.1 network 3.3.3.0 mask 255.255.255.0 forbidden-ip 3.3.3.1 - Configure port connected with POE SW as trunk port and configure ip address. # interface GigabitEthernet1/0/3 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # interface Vlan-interface600 ip address 192.168.60.1 255.255.255.0 # interface Vlan-interface700 ip address 3.3.3.1 255.255.255.0 - Configure on POE SW, and vlan 600 and vlan 700, configure port connected with AC as trunk port and configure port connected with AP as access port and enable poe. # vlan 600 # vlan 700 # interface Smartrate-Ethernet1/0/1 port link-mode bridge port access vlan 600 poe enable # interface Smartrate-Ethernet1/0/11 port link-mode bridge port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # - Configure service template, MAC authentication mode, MAC authentication domain, PSK authentication. And configure escape template. # wlan service-template wa1 ssid WA1 vlan 700 fail-permit enable akm mode private-psk cipher-suite ccmp security-ie rsn client-security authentication-mode mac mac-authentication domain office service-template enable # wlan service-template wa2 ssid WA2 vlan 700 fail-permit template service-template enable - Configure RF interface and bind service template. # wlan ap ap3 model WA4320i-X serial-id 210235A1GSC166000297 vlan 1 radio 1 radio enable service-template wa1 vlan 700 radio 2 radio enable service-template wa1 vlan 700 gigabitethernet 1 gigabitethernet 2 # - Configure radius authentication, configure the IP address, secret key, no domain name and source address of radius message, 192.168.207.57 is IMC server address, 192.168.207.199 is AC address. # radius scheme office primary authentication 192.168.207.57 primary accounting 192.168.207.57 key authentication simple 123456 key accounting simple 123456 user-name-format without-domain nas-ip 192.168.207.199 # - Create an ISP domain named Office1, configure authentication, billing and authorization schemes. # domain office authentication lan-access radius-scheme office authorization lan-access radius-scheme office accounting lan-access radius-scheme office # - Configure the user name and password of MAC address authentication as the user's MAC address without hyphen (the configuration is the default configuration). # mac-authentication user-name-format mac-address without-hyphen lowercase # - Configure IP address, make sure ac can ping IMC. Vlan 199 # Interface vlan 199 Ip add 192.168.207.199 23 # Interface g1/0/5 Port access vlan 199 # - Login in IMC and add route to DHCP server, make sure sta can ping IMC. - Configure radius server, Access device configuration. Log in to the IMC management platform, select the "user" tab, and click [access policy management / access device management / access device configuration] menu item in the navigation tree to enter the access configuration management page. Click the Add button in this page to enter the page of adding access devices. Set the authentication and billing shared key to 123456, and keep the default configuration for others. Select or manually add access devices and add access devices with IP address of 192.168.207.199 - Access policy configuration Select the "user" tab, click [access policy management / access policy management] menu item in the navigation tree, and click the < add > button to create an access policy. · configure the access policy named "office". Other parameters are set to default values, and click the < OK > button to complete the operation. - Add service configuration. Select the "user" tab, click the [access policy management / access service management] menu item in the navigation tree, and click the < add > button to create a service. The configuration service name is "office_ Mac "(the service name here can be arbitrarily named). The default access policy is "office". The others are configured by default. - Increase access users Select the "user" tab, click the [access user management / access user] menu item in the navigation tree, and click the < add > button to add an access user. · click the add user button and enter the user name "111 "and ID" 111 ", click the" OK > button to finish. Configure the account name and password as the client's MAC address " f887f16d3dba ". · check the binding service name "office_ mac”. - Use the terminal to access SSID, and you can see the terminal online on the AC and IMC online users, result 1 is expected. - Shutdown the link between IMC and AC, can still go online without authentication. |  |
| ☐ Passed | ☐ Failed |

  - Portal Authentication Escape

| Test Item Name: Portal Authentication Escape |  |
| --- | --- |
| Work Hour: 25 min |  |
| Test Topology: |  |
| Test Process: - Configure the portal authentication wireless service "ssid1" to enable the escape function. - STA1 is online in the way of portal authentication, and result1 is expected. - Disconnect from the authentication server, observe the status and communication of online portal users, and result 2 is expected. - The new user sta2 is online and result3 is expected. |  |
| Expect Results: - The STA1 authentication is successful and the communication is normal. - Online users are not affected, STA1 communication is normal, and the network can be used normally. - Sta2 can go online successfully without portal authentication, and the communication is normal. |  |
| Actual Result： - Configure control vlan 600 and service vlan 700 on AC. # vlan 600 # vlan 700 - Configure DHCP server. # dhcp enable # dhcp server ip-pool vlan600 gateway-list 192.168.60.1 network 192.168.60.0 mask 255.255.255.0 dns-list 192.168.60.1 forbidden-ip 192.168.60.1 # dhcp server ip-pool vlan700 gateway-list 3.3.3.1 network 3.3.3.0 mask 255.255.255.0 forbidden-ip 3.3.3.1 - Configure port connected with POE SW as trunk port and configure ip address. # interface GigabitEthernet1/0/3 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # interface Vlan-interface600 ip address 192.168.60.1 255.255.255.0 # interface Vlan-interface700 ip address 3.3.3.1 255.255.255.0 - Configure on POE SW, and vlan 600 and vlan 700, configure port connected with AC as trunk port and configure port connected with AP as access port and enable poe. # vlan 600 # vlan 700 # interface Smartrate-Ethernet1/0/1 port link-mode bridge port access vlan 600 poe enable # interface Smartrate-Ethernet1/0/11 port link-mode bridge port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # interface Vlan-interface700 ip address 3.3.3.2 255.255.255.0 # interface M-GigabitEthernet0/0/0 ip address 192.168.207.200 255.255.254.0 # - Configure static route on AC, make sure they can ping each other. # ip route-static 192.168.207.57 23 3.3.3.2 # - Login in IMC and add route to DHCP server, make sure sta can ping IMC. - Configure radius, domain and portal web-server, portal server on AC. # radius session-control enable # radius scheme rs1 primary authentication 192.168.207.57 primary accounting 192.167.207.57 key authentication simple 123456 key accounting simple 123456 user-name-format without-domain nas-ip 3.3.3.1 # domain dm1 authentication portal radius-scheme rs1 authorization portal radius-scheme rs1 accounting portal none # portal free-rule 0 destination ip 192.168.206.0 255.255.254.0 # portal web-server newpt url http://192.168.207.57:8080/portal server-detect interval 30 trap server-type cmcc url-parameter ssid ssid url-parameter wlanacname value AC url-parameter wlanuserip source-address # portal server newpt ip 192.168.207.57 key simple 123456 server-type cmcc - Configure wlan template with portal escape, configure wlan ap. # wlan service-template wa1 ssid WA1 vlan 700 portal enable method direct portal domain dm1 portal bas-ip 3.3.3.1 portal apply web-server newpt portal fail-permit web-server service-template enable # wlan ap ap3 model WA4320i-X serial-id 210235A1GSC166000297 vlan 1 radio 1 radio 2 radio enable service-template wa1 vlan 700 gigabitethernet 1 gigabitethernet 2 # - Configure IMC access equipment, access policy and service. - Policy is default configuration and just add name. - Binding access policy. - Add access user with account clinet1 and password 123456, bind the service. - Configure portal server, use default configuration. - Configure IP group. - Configure portal device. - Use terminal to connect wlan , can login in, AC show portal log and wlan client. Result 1 is expected. - Shut the link between SW and radius, trigger escape, at this time online client can still work without. Result 2 is expected. - After 3 min, the server state become down, and login out the wlan, then login again, result 3 is expected. |  |
| ☐ Passed | ☐ Failed |

  - Automatic power adjustment

| Test Item Name: Automatic power adjustment |  |
| --- | --- |
| Work Hour: 30 min |  |
| Test Topology: |  |
| Test Process: - Select two adjacent APs and set the power to auto-adjust. - Configure the neighbor threshold, the minimum power automatically adjusted by the AP, and the threshold triggered by the power adjustment, and finally enable the automatic detection and adjustment function. |  |
| Expect Results: - Check the configuration, the AP adjusts the power in a period of 3 minutes until the minimum power is 12dBm. |  |
| Actual Result： - Configure the wireless service template. # wlan service-template wa1 ssid WA1 vlan 700 service-template enable # return - Configure control vlan 600 and service vlan 700 on AC. # vlan 600 # vlan 700 - Configure DHCP server. # dhcp enable # dhcp server ip-pool vlan600 gateway-list 192.168.60.1 network 192.168.60.0 mask 255.255.255.0 dns-list 192.168.60.1 forbidden-ip 192.168.60.1 # dhcp server ip-pool vlan700 gateway-list 3.3.3.1 network 3.3.3.0 mask 255.255.255.0 forbidden-ip 3.3.3.1 - Configure port connected with POE SW as trunk port and configure ip address. # interface GigabitEthernet1/0/3 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # interface Vlan-interface600 ip address 192.168.60.1 255.255.255.0 # interface Vlan-interface700 ip address 3.3.3.1 255.255.255.0 - Configure on POE SW, and vlan 600 and vlan 700, configure port connected with AC as trunk port and configure port connected with AP as access port and enable poe. # vlan 600 # vlan 700 # interface Smartrate-Ethernet1/0/1 port link-mode bridge port access vlan 600 poe enable # interface Smartrate-Ethernet1/0/2 port link-mode bridge port access vlan 600 poe enable # interface Smartrate-Ethernet1/0/11 port link-mode bridge port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # - Configure the neighbor threshold, the minimum power automatically adjusted by the AP, and the threshold triggered by the power adjustment, and finally enable the automatic detection and adjustment function. # wlan ap ap3 model WA4320i-X serial-id 210235A1GSC166000297 vlan 1 radio 1 channel 40 radio enable channel band-width 20 service-template wa1 vlan 700 client-rate-limit enable client-rate-limit inbound mode static cir 8192 client-rate-limit outbound mode static cir 8192 rrm calibrate-power self-decisive enable calibrate-power min 12 calibrate-power threshold 90 adjacency-factor 1 radio 2 gigabitethernet 1 gigabitethernet 2 # wlan ap ap4 model WA6330 serial-id 219801A23V819BE0012V vlan 1 radio 1 channel 40 radio enable channel band-width 20 service-template wa1 vlan 700 client-rate-limit enable client-rate-limit inbound mode static cir 8192 client-rate-limit outbound mode static cir 8192 rrm calibrate-power self-decisive enable calibrate-power min 12 calibrate-power threshold 90 adjacency-factor 1 radio 2 radio 3 gigabitethernet 1 smartrate-ethernet 1 # - Check the configuration in a period of 3 minutes, result 1 is expected. |  |
| ☐ Passed | ☐ Failed |

  - Remote AP

| Test Item Name: Remote AP |  |
| --- | --- |
| Work Hour：30 min |  |
| Test Topology: |  |
| Test Process: - Use the serial ID labeled on the AP's rear panel to specify an AP. - Remote AP takes effect only on an AP that operates in local forwarding mode. - For port on the AP to join VLAN 200, edit the AP configuration file, and upload it to the storage media of the AC. |  |
| Expect Results: - The PC successfully logs in to the DUT. |  |
| Actual Result： - Create VLAN 600 and VLAN-interface 600, and assign an IP address to the VLAN interface. The AC will use this IP address to establish CAPWAP tunnels with APs. # vlan 600 interface vlan-interface 600 ip address 192.60.1.1 24 # - Create VLAN 700 and VLAN-interface 700, and assign an IP address to the VLAN interface. The AC will use VLAN 700 for client access. # vlan 700 interface vlan-interface 700 ip address 3.3.3.1 24 # - Configure GigabitEthernet 1/0/1 that connects the AC and the switch as a trunk port, and assign it to VLANs 600 and 700. # interface gigabitethernet 1/0/1 port link-type trunk port trunk permit vlan 600 700 # - Create a service template named 1 and enter its view. # wlan service-template WA1 ssid service client forwarding-location ap vlan 700 service-template enable # - Create a manual AP named officeap, and specify the AP model and serial ID. # wlan ap officeap model WA560-WW serial-id 219801A1NM8182032235 radio 2 service-template wa1 vlan 700 radio enable hybrid-remote-ap enable map-configuration apcfg.txt # - Configure interfaces on the switch: Create VLAN 600 and VLAN-interface 600, and assign an IP address to the VLAN interface. The switch will use VLAN 600 to forward the traffic on the CAPWAP tunnel between the AC and AP. # vlan 600 interface vlan-interface 600 ip address 192.60.1.2 24 # - Create VLAN 700 and VLAN-interface 700, and assign an IP address to the VLAN interface. The switch will use VLAN 700 to forward client traffic. # vlan 700 interface vlan-interface 700 ip address 3.3.3.2 24 # - Configure Smartrate-Ethernet1/0/1 that connects the switch and the AC as a trunk port, and assign the trunk port to VLANs 600 and 700. # interface Smartrate-Ethernet1/0/1 port link-type trunk port trunk permit vlan 600 700 # - Configure Smartrate-Ethernet1/0/1 that connects the switch and the AP as a trunk port, remove the port from VLAN 1, and assign the trunk port to VLANs 600 and 700. # interface gigabitethernet 1/0/2 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 port trunk pvid vlan 600 # - Configure DHCP services. # dhcp enable dhcp server ip-pool vlan600 network 192.60.1.0 mask 255.255.255.0 forbidden-ip 192.60.1.1 gateway-list 192.60.2.2 # dhcp server ip-pool vlan700 network 3.3.3.0 mask 255.255.255.0 forbidden-ip 3.3.3.1 gateway-list 3.3.3.2 # - Configuring AP configuration file apcfg.txt. Copy the following text to a text file and upload the file to the AC by tftp. Script: system-view vlan 700 interface GigabitEthernet 1/0/1 port link-type trunk port trunk permit vlan 700 - Client login, result 1 is expected. |  |
| ☐ Passed | ☐ Failed |

  - Software and Device Configuration Updates

| Test Item Name: Software and Device Configuration Updates |  |
| --- | --- |
| Work Hour：60 min |  |
| Test Topology: |  |
| Test Process: - The route between Devices and NMS (IMC WSM) is ok, Ping, SNMP packets are ok, Configure SNMP in AC device. - Add and manage AC in NMS (IMC WSM), enter the AC detail information page. - Upload the new AC software version in NMS (IMC WSM), configure the upgrade. |  |
| Expect Results: - Add and manage AC in NMS (IMC WSM), enter the AC detail information page, expect result 1 appear. - Upload the new AC software version in NMS (IMC WSM), configure the upgrade, expect result 2 appear. |  |
| Actual Result： - Configure control vlan 600 and service vlan 700 on AC. # vlan 600 # vlan 700 - Configure DHCP server. # dhcp enable # dhcp server ip-pool vlan600 gateway-list 192.168.60.1 network 192.168.60.0 mask 255.255.255.0 dns-list 192.168.60.1 forbidden-ip 192.168.60.1 # dhcp server ip-pool vlan700 gateway-list 3.3.3.1 network 3.3.3.0 mask 255.255.255.0 forbidden-ip 3.3.3.1 - Configure port connected with POE SW as trunk port and configure ip address. # interface GigabitEthernet1/0/3 port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # interface Vlan-interface600 ip address 192.168.60.1 255.255.255.0 # interface Vlan-interface700 ip address 3.3.3.1 255.255.255.0 - Configure on POE SW, and vlan 600 and vlan 700, configure port connected with AC as trunk port and configure port connected with AP as access port and enable poe. # vlan 600 # vlan 700 # interface Smartrate-Ethernet1/0/1 port link-mode bridge port access vlan 600 poe enable # interface Smartrate-Ethernet1/0/11 port link-mode bridge port link-type trunk undo port trunk permit vlan 1 port trunk permit vlan 600 700 # - Configure service template. # wlan service-template wa1 ssid WA1 vlan 700 service-template enable - Configure RF interface and bind service template. # wlan ap ap3 model WA4320i-X serial-id 210235A1GSC166000297 vlan 1 radio 1 radio enable service-template wa1 vlan 700 radio 2 gigabitethernet 1 gigabitethernet 2 # - Configure IP address, make sure ac can ping IMC. Vlan 199 # Interface vlan 199 Ip add 192.168.207.199 23 # Interface g1/0/5 Port access vlan 199 # - Login in IMC and add route to DHCP server, make sure sta can ping IMC. - Configure SNMP in AC device # snmp-agent snmp-agent local-engineid 800063A280441AFACF01A800000001 snmp-agent community write private snmp-agent community read public snmp-agent sys-info version all # - Add AC on imc, result 1 is expected - Upload the new AC software version in NMS (IMC WSM), configure the upgrade task - Execute the task, result 2 is expected |  |
| ☐ Passed | ☐ Failed |