H3C ADWAN Solution Test Procedure

H3C Technologies Co., Ltd.

July 2021

Content

1.	Test Description	4

1.1	Project information	4

2.	Checklist of Testing	4

3.	Methods of Testing Item	5

3.1	Device management	5

3.2	Link management	7

3.3	Topology display	8

3.4	Device and link state monitoring	10

3.5	Link selection based on WAN selection policies	11

3.6	(DPI)APR application based traffic engineering	15

3.7	CBQ and congestion	20

3.8	Link –reselection—based on packet loss	30

3.9	Link –reselection—based on jitter and delay	36

3.10	Load balancing (multi line multiplexing for the same application)	44

3.11	WAAS	49

3.12	Visible device information	56

3.13	Visible link information	56

3.14	Visible alarm management	58

3.15	ZTP by email(URL encryption)	64

3.16	Identify unknown traffic	72

Appendix 1. Testing Protocol	76

Name, Position, Signature	77

- Test Description

  - Project information

| Project Name | Belline |
| --- | --- |
| Customer | Beeline |
| Testing Date | July 2021 |
| Testing Site | Beeline Existing Site |
| Participants |  |

- Checklist of Testing

| No | Test Item | Result Yes/No |
| --- | --- | --- |
| 1 | Device management |  |
| 2 | Link management |  |
| 3 | Topology display |  |
| 4 | Device and link state monitoring |  |
| 5 | Link selection based on WAN selection policies |  |
| 6 | APR application based traffic engineering |  |
| 7 | CBQ and congestion |  |
| 8 | Link –reselection—based on packet loss |  |
| 9 | Link –reselection—based on jitter and delay |  |
| 10 | Load balancing (multi line multiplexing for the same application) |  |
| 11 | WAAS |  |
| 12 | Visible device information |  |
| 13 | Visible link information |  |
| 14 | Visible alarm management |  |
| 15 | ZTP by email(URL encryption) |  |
| 16 | Identify unknown traffic |  |

- Methods of Testing Item

  - Device management

- Purpose

Verify that you can query network devices on the controller.

Description : 1. Check Device infotmation.

- Standard compliance

N/A

- Test conditions

The test network has been set up as shown

- Procedure

### 1. In the device list, the device state is normal, and the device information, such as the device name, tier, type, and serial number is displayed correctly ### 2. The interface information is displayed correctly, including the interface state, name, IP address, interface speed, and attached network. ### 3. The interface module information is displayed correctly, including the module index, chassis and slot, role, and states. ### 4. The basic information of the device is displayed correctly, including the device name, tier, model, vendor, serial number, and software version.

  - Link management

- Purpose

Description: This test is to verify that the controller can view the information of the links between managed devices.

- Standard compliance

N/A

- Test conditions

The test network has been set up as shown

- Procedure

### 1. In the link list, you can view link information, including link state, name, interface, and available bandwidth.

  - Topology display

- Purpose

Description 1. Check monitor topology

- Standard compliance

N/A

- Test conditions

The test network has been set up as shown in Figure 1.

Zero-touch deployment is complete, and the devices come online correctly.

- Procedure

1. The topology page displays the topology of the entire network. The connections between devices and networks are correct. 2. The overlay topology displays the tunnels established in the network. The connections between devices are correct.

  - Device and link state monitoring

- Purpose

This test verifies the monitoring function of the controller to the device and link. When the device or link fails, the controller will monitor the response status and generate alarm information. Here we will simulate the situation of device and link failure respectively

- Standard compliance

N/A

- Test conditions

The test network has been set up as shown

- Procedure

- 1. Shut down an interface on the device in branch, On the topology page, the link becomes down (displayed in gray). On the system alarms page, you can see a link down alarm for the link. - 4. Bring up the interface on the device in branch, On the topology page, the link becomes up (displayed in green). The link state alarm recovers.

  - Link selection based on WAN selection policies

- Purpose

Description - G0/1 and G0/2 of MSR 810 , the bandwidth is 10oM - Establish 1 application simulation on controller - Create policy to ensure this application can choose on this 2 paths, and mpls1 have higher priority - Send traffic and MPLS1 have 10M , while MPLS2 is zero

- Standard compliance

N/A

- Test conditions

The test network has been set up as shown in Figure 1.

- Procedure

- 1. Set MPLS1 priority higher than MPLS2 link so that traffic is forwarded through MPLS1 link 2. Create policy scope, make branch and HQ both source device. 4. Create user-defined applications for traffic. Use IP protocol 5. Bind user-defined application , sla profile and polcy scope to application group and deploy 6. create iperf server on HQ PC Commands iperf – s 172.16.10.4 –i 1 7. Send traffic from Branch PC and view on topology. Traffic goes on MSTP link Iperf –c 172.16.10.4 –i 1 –t 100 –b 10M

  - (DPI)APR application based traffic engineering

- Purpose

Verify that the controller supports traffic engineering based on predefined and user-defined applications (the DPI feature).

This test is to verify the controller can be configured to identify the traffic of an application group. In this test, we will define an FTP application group to match FTP data, and set the link selection rules . We will access the FTP server from branch PC to HQ and download the file, and make the traffic transmit on the MPLS1 link

- Standard compliance

N/A

- Test conditions

The test network has been set up as shown in Figure 1.

- Procedure

- 1. Add a signature group. Select application signatures for the signature group. - 2. Add an application. Select a signature group as the match rule, and select a predefined or user-defined signature group. - 3. Add SLA profile : setting a higher priority for link MPLS1 than others - 4. set policy scope - 5. Use the added application to create a traffic engineering application group - 6. Establish FTP SERVER on HO PC, - 7. Branch PC downloads files from HQ Account 111 Password 111111 8. ftp DATA choose MPLS1 link

  - CBQ and congestion

- Purpose

Topology:

Description : Inside the same link: - G0/1 of MSR 810 limit the bandwidth to 10M, and bind CBQ policy. G0/2 of MSR 810 bandwidth is 100M. - Establish three application simulation on HQ-PC and BranchPC, with UDP destination ports 60001, 60002, 60003 - Create policy to ensure red and purple traffic can only run on MPLS2(10M) and send traffic - Congestion is identified and CBQ QOS policy is triggered - The Red traffic becomes to 7 M and the Purple is 3M. Different links: - Send the black traffic, - The Black traffic have higher priority of MPLS2 than MPLS1 - The traffic will try pass MPLS2 at first, but MPLS 2 bandwidth is occupied by the red and purple traffics. - The result the black one can only choose MPLS1.

- Standard compliance

N/A

- Test conditions

The test network has been set up as shown in Figure 1.

All links have a good quality.

- Procedure

1. Make sure there are no other teste configuration on controller. 2. Create TE policy for red traffic , level is critical and only choose MPLS2 path make sure red traffic can only run on MPLS2 3. Create TE policy for purple traffic , level is flashoverride and only choose MPLS2 path make sure purple traffic can only run on MPLS2 4. Create TE policy for black traffic , level is critical and can choose 2 pathes, make sure black traffic have more priority on MPLS2 than MPLS1. 5. Create policy scope, make branch and HQ both source device. 6. Create user-defined applications for red, purple and black traffic. Use UDP protocol and destination port are 60001, 60002 ,60003. 7. Bind user-defined application , sla profile and polcy scope to application group and deploy them

8. Create qos template for red and purple traffic and bind this profile to corresponding port to make sure when congestion occurs this port will trigger QOS policy. 9. limit the g0/1 of MSR 810 bandwidth to 10M. 10. Establish the iperf server on HQ-PC with protocol udp and ports are 60001, 60002,60003. Commands: iperf –u –s 172.16.10.4 –i 1 –p 60001 iperf –u –s 172.16.10.4 –i 1 –p 60002 iperf –u –s 172.16.10.4 –i 1 –p 60003 11. First send red and purple traffic on branch pc to make G0/1 of MSR 810 congestion and tragger qos policy. Commands: Iperf –u –c 172.16.10.4 –t 1000 –i 1 –b 10M –p 60001 –l 1000 Iperf –u –c 172.16.10.4 –t 1000 –i 1 –b 10M –p 60002 –l 1000 12. Check on HQ-PC, red is 7M and purple is 3M 13.Check on control the link color is yellow. This is because our bandwidth is 10M and Now the whole running traffic is 10m, so the system will feel that there is no excess traffic for other applications, so an alarm will be generated. Generally speaking, if the running traffic exceeds 50% of the bandwidth for a long time, the system will alarm. However, this also confirms that our strategy has taken effect, and only red and purple traffic will go through mpls2 14. Then send black traffic on branch PC Iperf –u –c 172.16.10.4 –t 1000 –i 1 –b 20M –p 60002 –l 1000 15 Check on server PC, receive 20M 16 Check on controller, black one choose MPLS1 because congestion od MPLS2 path. NOTE : Iperf software has unstable performance, and long-term use will lead to blank screen of CMD view

  - Link –reselection—based on packet loss

- Purpose

Topology:

Description : - WAN port bandwidth of MSR810 is 100M, sending traffic is 20 M, - Ensure this traffic can choose 2 paths, MPLS2 have higher priority - When link is in good quality the whole traffic can run on MPLS2. - Simulate packet instability on the MPLS side, - Packet loss, latency or jitter will cause the traffic to change to MPLS1 link.

- Standard compliance

N/A

- Test conditions

The test network has been set up as shown in Figure 1.

All links have a good quality.

- Procedure

Before test, check the controller if there are rest configuration for other test.

1. Make sure there are no other teste configuration on controller. 2. Create TE policy for traffic , level is critical and choose MPLS2 path and MPLS1 path, MPLS 2 path has higher priority 3. Create policy scope, make branch and HQ both source device. 4. Create user-defined applications for traffic. Use IP protocol 5. Bind user-defined application , sla profile and polcy scope to application group and deploy

6. Establish the iperf server on HQ-PCCommands: iperf –s 172.16.10.4 –i 1 7. First send traffic on branch pc Commands: Iperf –c 172.16.10.4 –t 1000 –i 1 –b 20M 8. Check on HQ-PC , receive 20 M 9. Check controller, 20M traffic run on MPLS2. 10. shutdown and undo shutdown the g0/2 of MPLS 2 equipment for many times to trigger port oscillation 11. Check on HQ-PC, STILL 20M, check on controller now run on MPLS1 path 12. WHEN recover the link and wait for a moment ,it will choose mpls2 again NOTE : Iperf software has unstable performance, and long-term use will lead to blank screen of CMD view

  - Link –reselection—based on jitter and delay

- Purpose

Topology:

Description : - WAN port bandwidth of MSR810 is 100M - Sending traffic is 20 M - Ensure this traffic can choose 2 paths, but MPLS2 have higher priority. - When link is in good quality the whole traffic can run on MPLS2. - Change the link to high jitter and latency, then link will change to MPLS1

- Procedure

Before test, check the controller if there are rest configuration for other test.

1. Make sure there are no other teste configuration on controller. Link – switching time may a little longer . 2. Create TE policy for traffic , level is critical and choose MPLS2 path and MPLS1 path, MPLS 2 path has higher priority 3. Create policy scope, make branch and HQ both source device. 4. Create user-defined applications for traffic. Use IP protocol 5. Bind user-defined application , sla profile and polcy scope to application group and deploy

6. Establish the iperf server on HQ-PCCommands: iperf –s 172.16.10.4 –i 1 7. First send traffic on branch pc Commands: Iperf –c 172.16.10.4 –t 1000 –i 1 –b 20M 8. Check on HQ-PC , receive 20 M 9. Check controller, 20M traffic run on MPLS2. 10. Check the sla profile, because we choose critical level, there is a standard to defines link quality. If delay, packet loss rate and jitter are three conditions, there will be path selection if one condition is not satisfied. 11. login in IXIA Network Emulator(192.168.207.179), account: Admin password: admin And choose to make latency is higher than 50MS 12. choose Port 1( port 1 is connected with branch) and default profile, then choose Impairments. 13. Choose delay and close packet-loss 14.Click make windows become green, and imput number 120 , A round-trip link is 120, and a one-way link is 60, then submit 15. Check on HQ-PC, STILL 20M, check on controller now run on MPLS1 path 16. WHEN recover the link and wait for a moment ,it will choose mpls2 again How to turn off delay 17. Choose delay of port 2 and close delay 18. Check on HQ-PC, STILL 20M, check on controller now run on MPLS1 path 19. WHEN recover the link and wait for a moment ,it will choose mpls2 again Make delay windows gray then submit NOTE : Iperf software has unstable performance, and long-term use will lead to blank screen of CMD view

  - Load balancing (multi line multiplexing for the same application)

- Purpose

Support multi line multiplexing for the same application

- Procedure

1. Make sure there are no other teste configuration on controller. 2. Create TE policy for traffic , level is critical and choose MPLS2 path and MPLS1 path, 2 paths have the same priority. 3. Create policy scope, make branch and HQ both source device. 4. Create user-defined applications for traffic. Use IP protocol 5. Bind user-defined application , sla profile and polcy scope to application group and deploy(choose per packet, )

6. Establish the iperf server on HQ-PC Commands: iperf –u –s 172.16.10.4 –i 1 –p 60002 7. First send traffic on branch pc Commands: Iperf –u –c 172.16.10.4 –t 1000 –i 1 –b 20M -p 60002 8. Check on HQ-PC , receive 20 M 9. Check controller, 20M traffic run on MPLS2 and MPLS1

- Clear the configuration (except for the default configuration) in this test before you perform the subsequent test.

  - WAAS

- Purpose

Verify that WAAS can increase the download speed on a low-speed WAN link.

In this test, we apply the WAAS policy on the WAN interface of the device. The packet traffic sent or received from the WAN side will match the policies referenced by the WAN interface, such as FTP traffic. If the matching is successful, the FTP transmission time can be significantly reduced.

When use was , make sure there is no application group. Or waas can not work, Make sure there are no other teste configuration on controller. 1. Select wan acceleration and set WAAS policy, then add FTP and FTP-data types to the new WAAS policy 2. Add two WAAS applications, which are distributed on HQ and BRANCH respectively. And apply the WAAS policy about FTP 3. You can see the WAAS status on the device 4. In order to see the optimization of FTP download speed by WAAS more clearly, limit the speed of the port. 5. ON hq pc , enable 3cd (just enable once , no twice) 6. Without deploying WAAS, use the PC in the branch to access the FTP server in the Hub to download the file ( I rename the file, now name is waas) account 111 password 111111 6. After deploying WAAS, use the PC in the branch to access the FTP server in the Hq to download the file. The download speed is significantly faster than that without WAAS deployment

  - Visible device information

- Purpose

Verify that the controller can display the device information.

This test is to verify that you can see some information of the device on the controller

- Standard compliance

N/A

- Test conditions

The test network has been set up as shown in Figure 1.

The controller runs for a while.

- Procedure

- Actual results

- 1. On the device information page, you can view the CPU usage, memory usage, and temperature information about the device.

  - Visible link information

- Purpose

Verify that the controller can display the link history information.

This test is to verify that you can see some historical information of the links on the controller

- Procedure

- 1.On the link history running information page, you can view the trend information about the jitter, latency, and bandwidth usage in the past hour. Put the cursor on a position of the trend line, the value at that time point will be displayed. - 2.You can view the jitter, latency, and bandwidth usage trend in the selected time span，You can view the trend information about the jitter, latency, and bandwidth usage by time spans.

  - Visible alarm management

- Purpose

Verify that the controller can display the alarm information.

This test is to verify that you can see the alarm information on the controller, including the current alarm and the recovered alarm, and you can acknowledge the alarm information.

- Standard compliance

N/A

- Test conditions

The test network has been set up as shown in Figure 1.

Zero-touch deployment is complete, and the devices come online correctly.

- Procedure

- Shut down the outbound interface of the headquarters on the link (headquarters-MSTP-branch 1). Result 1 is expected.

- Bring up the interface that is shut down previously. Result 2 is expected.

- Acknowledge an alarm. Result 3 is expected.

- Expected results

- The link offline alarm is generated.

- The alarm recovers and the recovery time is displayed.

- The status of the alarm changed to acknowledged.

- Remarks

- Clear the configuration (except for the default configuration) in this test before you perform the subsequent test.

- Actual results

- 1.The link offline alarm is generated. - 2.The alarm recovers and the recovery time is displayed 3. Acknowledge an alarm

  - ZTP by email(URL encryption)

- Purpose

- Procedure 1. Before you open this link, make sure your PC is connected with ZTP device by Console and cable. And ZTP is in empty configuration . 192.168.0.1 is default ip address 2. Use cable to connect branch PC and ZTP device port g0/5. Change PC ip address. 3. Configure mail server parameters: the admin user tries to configure the mail server. The user name and password need to be filled 4. you need import ZTP device , and because ZTP is in empty configuration, can not access to Controller, so the status is down 5. Add site for ZTP device 6. Add mstp line and Import wan network detail. Here we use OSPF to bulid underlay for HQ and ZTP device. And you can see wan detail, there is a fail session. Here we will use email to finish this session. We can also check topology, you can see the link is down. 7. check ZTP parameter, we use secret key for encrypting the url If choose no secret key,URL will show information about your ZTP device configuration 8. Check your branch PC winmail server. Account ztp@123.com Password: 123456h3c 9. Click this link and jump a web ,first need input secret key adwan, next input the account and password are both admin (default account and password ) Now configuring Check on GUI by console Before ZTP by email :

  - Identify unknown traffic

- Purpose

- Procedure

1. First send untraffic , then use analyser to identify unknown traffic - 2. Check the unknown traffic parameters - 3. click Action it will jump to application group 4. then input your member appliication 4. add sla profile 5. edit sla profile

Appendix 1. Testing Protocol

| Number Protocol |  |
| --- | --- |
|  | YYYY.MM.DD / VENDOR-MODEL-sequence number |

This Protocol is based on the results of the sample testing equipment:

| The manufacturer's name, country | H3C,China |
| --- | --- |
| The equipment model |  |
| The serial number of equipment |  |

The composition of the test participants

By the Customer

### Name, Position, Signature