The firewall uses default cipher suites during the SSL interaction process for compatibility purposes when utilizing HTTPS and SSL VPN. These cipher suites inevitably include insecure algorithms, such as RC4. Therefore, it is necessary to create a secure SSL server policy to exclude these algorithms. The HTTPS and SSL VPN modules will then call the newly created SSL server policy, preventing the corresponding SSL-related vulnerabilities from being scanned.

#### Process Analysis

The firewall uses default cipher suites during the SSL interaction process for compatibility purposes when utilizing HTTPS and SSL VPN. These cipher suites inevitably include insecure algorithms, such as RC4. Therefore, it is necessary to create a secure SSL server policy to exclude these algorithms. The HTTPS and SSL VPN modules will then call the newly created SSL server policy, preventing the corresponding SSL-related vulnerabilities from being scanned.

#### Solution

1. To configure the SSL server policy, you need to reference the PKI domain. First, create a new PKI domain. As shown in the figure below, click the [New PKI Domain] button in the firewall web interface under the [Object/PKI/Certificate] menu bar, and create a PKI domain named "test." Other settings can remain as defaults.

2. Then, import the CA certificate and local certificate for the newly created PKI domain (for information on how to obtain the CA certificate and local certificate in this step, please refer to the description in Step 3).

2. Then, import the CA certificate and local certificate for the newly created PKI domain (for information on how to obtain the CA certificate and local certificate in this step, please refer to the description in Step 3).

3. Certificates can be obtained through Microsoft Active Directory Certificate Services or Linux OpenSSL. If you need an authoritative CA certificate, you must pay the authoritative CA organization to obtain it. The firewall flash contains a self-signed certificate, which is the one used by default for HTTPS and SSL VPN. You can export this certificate and separate the public key and private key to import them as the CA certificate and local certificate into the created PKI domain. The operation is as follows (the logic of this step is that if an authoritative CA is not needed, you can use the local certificate provided by the firewall to separate the CA, thus forming a set of certificates consisting of the CA and local certificate):

3.1. Use FTP or SFTP to obtain the corresponding /pki/folder from the firewall and place the https-server.p12certificate file on the desktop.

3.2. Double-click the certificate file to import the exported certificate back into the local PC, to separate the CA certificate that does not contain the private key.

3.3. Click "Next."

3.4. Since the self-signed certificate provided by the firewall does not protect the private key with a password, you do not need to enter a password here; just click "Next."

3.5. Store the certificate in the [Personal] directory, then continue to click "Next" and "Finish."

3.6. Open the operating system's [Internet Options/Content/Certificates/Personal] menu to see the imported H3C self-signed certificate.

3.7. Export the certificate imported into the device to generate a CA certificate that does not contain the private key (the certificate name is subject to the actual export; different devices and versions may have different certificate filenames).

4. Create a new SSL server policy "test" and reference the created PKI domain "test,"

5. Enable the SSL server policy for HTTPS.

Disable the current HTTPS/HTTP services provided externally.

[H3C] undo ip https enable

[H3C] undo ip http enable

Configure SSL services such as HTTPS services to reference the previously customized SSL server policy:

[H3C] ip https ssl-server-policy test

Re-enable the HTTPS/HTTP services.

[H3C] ip https enable

[H3C] ip http enable